| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0097_r1 |
| --- | --- |

| Title: | CE5: Informational tests on reducing both maximum palette size and maximum palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, Shawmin Lei 2860 Junction Ave. San Jose CA 95134, USA | Email: | {polin.lai, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents informational test results of reducing both the maximum palette size and maximum palette predictor size. Under the current common test conditions (CTC) with SCM2.0 software settings, the maximum palette size per CU is 31, and the maximum palette predictor size is 64. Two tests were reported in this contribution: Test 1 with maximum palette size 15, and maximum palette predictor size 32. Test 2 with maximum palette size 15, and maximum palette predictor size 48.

# Introduction

In CE5, the effects of changing maximum palette size and maximum palette predictor size are investigated [1]. Currently under the common test conditions (CTC) with SCM2.0 software settings, the maximum palette size per CU is 31, and the maximum palette predictor size is 64. Note that, currently, these two maximum values are not transmitted. This contribution presents results of reducing both the maximum palette size and maximum palette predictor size. Two tests were conducted:

- Test 1: Maximum palette size 15, and maximum palette predictor size 32.

- Test 2: Maximum palette size 15, and maximum palette predictor size 48.

# Sequence level reduction of both the maximum palette size and maximum palette predictor size

The reduction of maximum palette size and maximum palette predictor size is achieved simply by changing the macro defines in the SCM2.0 software. Thus these maximum values are set for the entire video sequence. Below are the simple changes made to the SCM2.0 software to conduct the tests reported in this contribution.

- Test 1: Maximum palette size 15, maximum palette predictor size 32.

#define MAX_PLT_SIZE		15

#define MAX_PLT_PRED_SIZE	32

- Test 2: Maximum palette size 15, maximum palette predictor size 48.

#define MAX_PLT_SIZE		15

#define MAX_PLT_PRED_SIZE	48

# Simulation results

## Summary of lossy results are presented in this section. For details and for the lossless results, please refer to the accompanying Excel sheets. Note that the runtime measurements are not accurate (tests uses a slower platform than anchor).

## Test 1: Maximum palette size 15, maximum palette predictor size 32.

- Test 2: Maximum palette size 15, maximum palette predictor size 48.

# Summary

This contribution presents results of reducing both the maximum palette size and maximum palette predictor size, as compared to the current settings.

Two tests were conducted:

- Test 1: Maximum palette size 15, and maximum palette predictor size 32.

- Test 2: Maximum palette size 15, and maximum palette predictor size 48.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, and X. Xiu, “CE5: Investigation of maximum palette size and maximum palette predictor size,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1105, 30 June – 9 July 2014.