| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0099 |
| --- | --- |

| Title: | Non-CE6: Syntax redundancy removal for palette mode with one index value |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Jungsun Kim, Shan Liu, and Shawmin Lei 2860 Junction Ave. San Jose CA 95134, USA | Email: | {polin.lai, jungsun.kim, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

For palette mode, it is possible that for all samples, their palette indices are all the same (e.g. all zeros), such as there is only one colour in the current CU’s palette. This contribution proposes that, for palette mode with one index value (e.g. zero), omit signalling the syntax elements related to the palette index map, and derive the unique palette index value (e.g., zero).

Full-length results will be provided once available.

# Introduction

In the 18th JCTVC Meeting, palette mode was adopted into SCM2.0 [1]. It represents samples in a CU using representative colours (entries in palette table), and each sample is coded with an index pointing to an entry in the palette table. For samples that are not represented by the palette colours, they are signalled as Escape samples with their index values equal to “the current palette size”, and their actually sample values are encoded directly. For a CU with palette mode, its palette colours can be predicted or directly coded, together with the palette index map which is the palette indices of all the samples in the CU.

It is possible that for all samples, their palette indices are all the same. For example, when the current CU has only one colour entry in its palette table and has no Escape samples, all the palette indices are zero. In such scenario, syntax elements related to the index map are redundant. This contribution proposes to omit signalling palette index map and derive the indices in such cases.

# Omit index map syntax elements for one index palette

Under the syntax in SCM-2.0, one index palette (all zeros) occurs under the following scenarios:

- Current palette has one colour, and no Escape sample

  - # of Predicted palette colour = 1, # of signaled palette colour = 0, CU level Escape flag = 0

  - # of Predicted palette colour = 0, # of signaled palette colour = 1, CU level Escape flag = 0

- Current palette is empty, ALL samples are Escape

  - # of Predicted palette colour = 0, # of signaled palette colour = 0, CU level Escape flag = 1

It is proposed in the contribution that, for any of these scenarios, palette index map is not signalled, and all the palette indices within the current CU are derived.

# Simulation results

## Summary of lossy results are presented in this section. For details and for the lossless results, please refer to the accompanying Excel sheets.

## Current only SHORT length, AI results.

## Full results will be updated once available.

# Summary

This contribution proposes that, when palette indices in the current CU are all the same (e.g. all zeros), omit signalling the syntax elements related to palette index map, and derive the unique palette index value (e.g., zero).

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- P. Onno (Canon), X. Xiu (InterDigital), Y.-W. Huang (MediaTek), and R. Joshi (Qualcomm), “Suggested combined software and text for run-based palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0348, 30 June – 9 July 2014.