| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0101 |
| --- | --- |

| Title: | AHG14: Intra Block Copy reference area for Wavefront Parallel Procsssing (WPP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai1, Xiaozhong Xu1, Shan Liu1, Tzu-Der (Peter) Chuang 2, Shawmin Lei1,2 1 2860 Junction Ave. San Jose CA 95134, USA 2 No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | polin.lai, xiaozhong.xu shan.liu, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents modifications to the IBC reference area, such that when WPP is utilized, IBC will only reference to blocks within the already reconstructed CTUs by the WPP threads. With WPP turned on (WaveFrontSynchro > 0 in configuration files), two reference area patterns are tested: 1. Near-rectangular reference area, and 2. Ladder-shaped reference area.

# Introduction

Wavefront parallel processing (WPP) in HEVC divides a slice into rows of CTUs to simultaneously encode/decode multiple threads. The upper-right side CTUs which have not yet been processed are not available when coding the current CTU, even though they are located above the current CTU row.

Currently in SCM2.0, full-frame IBC can be enabled to achieve improved coding efficiency. For a given block, full-frame IBC allows it to reference any CTU that is above the current CTU. Thus, if WPP is utilized, such full-frame IBC scheme could reference blocks that are outside of WPP processing threads.

This contribution presents IBC reference area patterns, such that when WPP is utilized, IBC will only reference blocks that are already reconstructed by the WPP threads.

# IBC reference area according to WPP setting

## Software fixes for WPP:

## For SCM2.0, decoder would crash when WPP is turned on. To resolve this, the parts of fix defined using RExt__R0128_HIGH_THROUGHPUT_PROFILE_FIX are added to reference software. The fix does not affect coding efficiency.

## To tests the proposed IBC reference area patterns, WPP are enabled for both anchor and test data.

## Summary of lossy results are presented in this section. For details and for the lossless results, please refer to the accompanying Excel sheets. Currently only SHORT length results are provided below. Full results will be updated once available.

## Reference area pattern 1: Near-rectangular area

In method 1, the reference area is set to be to the left of the WPP thread head of the immediate above CTU row. Figure 1 blow illustrates such reference area under two different WPP settings.

- Anchor: SCM2.0 + HIGH_THROUGHPUT _FIX, with WPP on (IBC reference area full-frame)

- Test:	SCM2.0 + HIGH_THROUGHPUT _FIX, with WPP on, IBC near-rectangular reference area

Figure 1. Reference area pattern 1: Near-rectangular area SHORT length results

## Reference area pattern 2: Ladder-shaped area

In method 2, the reference area covers all CTUs that are already reconstructed with WPP. Figure 2 blow illustrates such reference area under two different WPP settings.

- Anchor: SCM2.0 + HIGH_THROUGHPUT _FIX, with WPP on (IBC reference area full-frame)

- Test:	SCM2.0 + HIGH_THROUGHPUT _FIX, with WPP on, IBC ladder-shaped reference area

Figure 2. Reference area pattern 2: Ladder-shaped area SHORT length results

# Draft Text change

Reference area pattern 2: IBC ladder-shaped reference area for WPP

### 8.4.4	Derivation process for block vector components in intra block copying prediction mode

……

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output is set equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output is set equal to TRUE.

–	One or both of the following conditions shall be true: [Ed. (GJS): Clarify that this is a bitstream constraint (if that is the correct interpretation).]

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw is less than or equal to 0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh is less than or equal to 0

–	Additionally, when entropy_coding_sync_enabled_flag is equal to 1, it is a requirement of bitstream conformance that

(xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1)/CtbSizeY – xCurr/CtbSizeY <=

yCurr/CtbSizeY – (yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 )/CtbSizeY

# Summary

This contribution presents two reference area patterns to ensure IBC only referencing to blocks within the already reconstructed CTUs by the WPP threads. With the proposed IBC reference area, WPP can be utilized for the SCC project.

# Recommendations

Inclusion of the proposed IBC reference area patterns, such that WPP can be utilized for SCC.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References