| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0102 |
| --- | --- |

| Title: | CE9 Test A.1: Optionally disabling the usage of the intra boundary filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to conditionally disable the boundary filters for Intra horizontal, vertical, and DC modes altogether, for both lossless and lossy coding, with flag controls in SPS and slice header. Experimental results reportedly show that when the all three Intra boundary filters are selectively disabled in SPS, the lossy coding BD-rates in AI, RA and LD are as follows:

-0.5%, -0.2% and -0.1% for RGB, text & graphics with motion, 1080p
-0.6%, -0.4% and -0.5% for RGB, text & graphics with motion,720p
-1.2%, -0.1% and -0.9% for RGB, mixed content, 1440p
-0.9%, -0.6% and -0.5% for RGB, mixed content, 1080p
-0.2%, 0.0% and 0.0% 	for YUV, text & graphics with motion, 1080p
-0.1%, 0.0% and -0.1%	for YUV, text & graphics with motion,720p
0.0%, 0.0% and -0.3% 	for YUV, mixed content, 1440p
0.0%, 0.0% and 0.0% 	for YUV, mixed content, 1080p.

Besides, one additional test of combining Test A.1 and A.2 is presented. Experimental results reportedly show that this combined method outperforms the individual tests of CE9 test A.1 and A.2.

# Introduction

In HEVC, for intra DC, horizontal and vertical modes, boundary filters are applied only to the first component. The boundary prediction samples of DC mode will be smoothed with an [1, 3] or [1, 2, 1] filter to reduce the blocking artifact, as shown in Fig. 1. For vertical and horizontal intra prediction directions, a gradient based boundary filter is applied. Fig. 2 shows an example for the gradient based boundary filter for vertical intra prediction direction. In the current HEVC-SCC platform SCM2.0, the gradient filters in Intra horizontal and vertical modes are disabled when the derived value of disableIntraBoundaryFilter is 1. This occurs only when implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag (lossless mode) are both equal to 1.

.

Fig. 1.  DC filter

Fig. 2.  Gradient filter for vertical prediction

In CE9 test A.1, it is proposed to selectively disable both types of intra boundary filtering (Intra DC and Intra gradient filters) for the first component, whereas in CE9 test A.2, it is suggested to enable these intra boundary filtering for the other two components. Both methods are reasonable, because filtering 3 components differently, especially for blocks with sharp edges, will probably produce prediction efficiency loss.

# Proposed method

The performance loss produced by the boundary filtering is due to the fact that the filters are only applied to the first component, and never to the second or third component. When all three components have sharp edges, only filtering the first component will produce both subjective and objective performance loss. However, if enabling the filters for all three components, the complexity is increased. In this contribution, we call this non-alignment filtering problem.

The Test A.1 methods:

Therefore, it is firstly proposed to add flags in SPS and slice header, which control the disabling of two boundary filters (Intra DC and Intra gradient filters) in both vertical and horizontal directions. In the lossless and lossy test results provided in this contribution, the boundary filters are always disabled using these control flags. Note that it is an encoder choice to turn on or off the Intra boundary filters. Therefore, there are two results which correspond to the following two tests. These tests are related to CE9-Test 1.

- Test 1: Disabling all the intra boundary filters, as the encoder implementation in R0114

- Test 2: Utilizing thresholds to selectively disable the intra boundary filters only. The detailed algorithm is as follows: If the number of 16x16 2nd-and-3rd–component blocks with sharp edges reaches up to 1/9 of the total 2nd-and-3rd–component blocks, it means no less than 1/9 of the 16x16 units(3 components) may have the non-alignment filtering problem. Only in such case, the intra boundary filters are disabled. The sharp edges are discovered by checking whether there are neighboring pixels with larger than 1<<(bit_depth-2) differences.

Supplementary Test of Combining Test A.1 and A.2 (Test 3):

Besides, it is also proposed to add flags in SPS and slice header, which control the enabling and disabling of two boundary filters (Intra DC and Intra gradient filters) in both vertical and horizontal directions for all the three components. As for the selectivity mechanism, if the following condition is satisfied, the IBF is disabled for all components; Otherwise, the IBF is enabled for all components.

The number of 16x16 sharp-edge blocks exceeds the number of 16x16 smooth-edge blocks.

- Definition of sharp-edge block: Its boundary pixels are not equal on all the three components. Also, its horizontal or vertical edges have no less than 3 positions where neighbouring pixel difference exceed (1<<(bit_depth-2))

- Definition of smooth-edge block: Its boundary pixels are not equal on all the three components. One block whose horizontal or vertical edges have less than 3 positions where neighbouring pixel difference exceed (1<<(bit_depth-2))

- Experimental results

The proposed methods are integrated on HM-14.0+RExt-7.0+SCM-2.0 [1].

Results of Test A.1 Methods, test 1 and test 2:

With the introduced flags, the results for test 1, where the intra boundary filters are always disabled for lossless and lossy CTC conditions [2], are shown in Table 1 and 2. Whereas the results for test 2, where the intra boundary filters are selectively disabled, are shown in Table 3 and 4.

From the results we can see, test2 outperforms test 1 in YUV video coding.

Table 1.  Results of test 1 for lossy coding under CTC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.2% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.1% | -0.2% | 0.1% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2.  Results of test 1 for lossless coding under CTC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3.  Results of test 2 for lossy coding under CTC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -1.2% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.9% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -1.1% | -0.7% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.9% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | 0.5% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4.  Results of test 2 for lossless coding under CTC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Results of Test 3, the supplemental test for combining CE9 test A.1 and A.2:

Compared with CE9 Test A.1 and A.2, the BDRate gains of test 3 are shown in Table 5 and Table 6 for lossy coding; whereas Table 7 and Table 8 are for lossless coding.

Table 5 Comparing test 3 with Test A.1 on lossy Coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| RGB, Animation, 720p | -0.4% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | 0.0% |
| YUV, Animation, 720p | -0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.2% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p | -0.3% | 0.3% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | 0.5% | 0.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.1% | 0.5% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

Table 6. Compared with Test A.2 on lossy Coding (time is not accurate)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 70% |  |  |
| Dec Time[%] | 58% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 72% |  |  |
| Dec Time[%] | 64% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 73% |  |  |
| Dec Time[%] | 65% |  |  |

Table 7. Comparing test3 with Test A.1 on lossless Coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 88% |  |  |  |

Table 8. Compared with Test A.2 on lossless Coding(time is not accurate)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 74% |  |  |  |
| Dec Time[%] | 52% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 76% |  |  |  |
| Dec Time[%] | 62% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 76% |  |  |  |
| Dec Time[%] | 61% |  |  |  |

# Text specification changes

For Test 1 and Test 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| if(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_pic_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-1)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-2)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-3)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-4)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-5)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

For Test 3:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_onlyLuma_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| if(!intra_boundary_filter_pic_onlyLuma_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |

intra_boundary_filter_pic_ onlyLuma_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 0. intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 3 - intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 2. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 3. When not present, IntraBoundaryFilterSliceDisable equal to 0.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 2, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to IntraBoundaryFilterSliceDisable.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is not equal to 1, cIdx + disableIntraBoundaryFilter is larger than 2 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-6)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-7)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-8)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-9)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-10)

8.4.5.2.6 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 2, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to IntraBoundaryFilterSliceDisable.

  - When predModeIntra is equal to 26 (vertical), cIdx + disableIntraBoundaryFilter is larger than 2, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-11)

- When predModeIntra is equal to 10 (horizontal), cIdx + disableIntraBoundaryFilter is larger than 2, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-12)

# Conclusion

In this paper, it is suggested adopt the method combining CE9 Test A.1 and A.2, if the complexity increase of adding IBF for 2nd and 3rd components is not a problem. Otherwise, it is suggested to adopt CE9 Test A.1.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- HM-14.0+RExt-7.0+SCM-2.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-2.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April 2014