| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0105 |
| --- | --- |

| Title: | CE6-related: Syntax fixes for zero palette in palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Tzu-Der Chuang2, Shan Liu3, Jicheng An1, Xianguo Zhang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 3 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Kai Zhang +86-10-5690-0888 ext. 83890 {kai.zhang, peter.chuang, shan.liu, jicheng.an, xianguo.zhang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

There are two problems in the syntax design for palette coding. First, the palette of the current CU may be coded in a sharing mode even when the palette size of the previous CU is zero. Second, there is no valid palette index for coding pixels in a CU when the palette size of the current CU is zero and the CU-level escape flag is 0. For solving the first problem of syntax redundancy, it is proposed to infer the palette_sharing_flag as 0 if the palette size of last coded palette is 0. For solving the second problem that can lead to decoder crash, it is proposed to infer palette_escape_val_present_flag as 1 if the palette size of current palette CU is 0. The results reportedly show negligible coding performance change.

# Problem statement

In the current palette coding, a palette table can be coded as a sharing mode that the palette table of the current CU can be copied from the palette table of the last palette-coded CU. After palette table coding, a palette_escape_val_present_flag is signalled to indicate whether the current CU contains escape pixels. If the palette_escape_val_present_flag is 1, one palette index value is used to represent escape index.

However, there are two problems in the current palette syntax design. First, the palette of the current CU may be coded in a sharing mode even when the palette size of the previous CU is zero. Such syntax redundancy should be removed.

Second, before coding the palette index map, the maximum possible palette index (indexMax) is derived as shown in Table 1. If the palette_escape_val_present_flag is 0, the indexMax is equal to the palette size of the current CU minus 1. However, if the palette size of the current CU is zero and the palette_escape_val_present_flag is 0, the indexMax is equal to -1, which means there is no valid palette index for decoding. In this case the decoder behaviour is undefined, and the decoder can crash. This serious issue on decoder stability must be solved.

Table 1.  Derivation of indexMax

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + num_signalled_palette_entries |  |
| } |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) |  |
| indexMax = palette_size |  |
| else |  |
| indexMax = palette_size − 1 |  |
| … |  |
| } |  |

# Proposed methods

For these problems, two syntax fixes for palette coding are proposed.

First, when the palette size of last coded palette is 0, the palette_sharing_flag is proposed to be inferred as 0, as demonstrated in the syntax table below.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_transpose_flag | ae(v) |
| if ( previousPaletteSize > 0 ) |  |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_share_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } |  |

The decoder needs to decode the previous_palette_entry_flag and the palette_entries to reconstruct the palette table of current CU.

Second, when the palette size of current palette CU is 0, the palette_escape_val_present_flag is proposed to be inferred as 1, as demonstrated in the syntax table below.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + num_signalled_palette_entries |  |
| } |  |
| if ( palette_size > 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) |  |
| indexMax = palette_size |  |
| else |  |
| indexMax = palette_size − 1 |  |
| … |  |
| } |  |

In this way, when the palette size of the current CU is zero, the palette_escape_val_present_flag is inferred as 1, the indexMax is equal to 0, which is a valid index.

# Experimental results

The proposed method is integrated on SCM-2.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC with the full picture IBC search range. Table 2 and Table 3 show the results for lossy and lossless coding respectively. The results reportedly show negligible coding performance change by applying the proposed fixes.

# Conclusions

This contribution proposes two syntax fixes for palette coding. By constraining palette_share_flag, the syntax redundancy can be eliminated. By constraining palette_escape_val_present_flag to a valid value when the palette size of the previous CU or the current CU is zero, the undefined decoder behaviour can never happen. Negligible coding performance change is reported by applying the proposed fixes.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 1,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1005, July 2014.

- SCM-2.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-2.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, July 2014.

Table 2.  Lossy results of the proposed syntax fixes (to be updated)

Table 3.  Lossless results of the proposed syntax fixes (to be updated)