| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0107 |
| --- | --- |

| Title: | CE3 Test 1.2: Segmental prediction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Kai Zhang +86-010-56900888 ext. 83890 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the CE3 results of the segmental prediction method for intra block copy (segIBC). When segIBC is applied, two steps are applied to modify the IBC prediction block before it is used to predict the current block. First, samples in an IBC prediction block are implicitly classified into two segments. Second, a single value is signalled to represent all the prediction samples of the segment. As a simplification, residues are inferred to be all zero when segIBC is applied. Experimental results reportedly show that the segmental prediction method itself (test 1.2 in CE3) can achieve BD-rate savings in AI configurations as follows:
0.9%, RGB, text & graphics with motion, 1080p
0.3%, RGB, text & graphics with motion,720p
0.1%, RGB, mixed content, 1440p
0.2%, RGB, mixed content, 1080p
1.1%, YUV, text & graphics with motion, 1080p
0.3%, YUV, text & graphics with motion,720p
0.2%, YUV, mixed content, 1440p
0.3%, YUV, mixed content, 1080p.

# Introduction

Intra block copy (IBC) considers reduplicated patterns in a picture. As depicted in Fig.1, a reference block in the current picture is copied to the current block as the prediction if IBC is applied. The samples in the reference block must have been reconstructed already before the current block is coded or decoded.

IBC shows a prominent performance for screen content sequences generally because there are many reduplicated patterns in these sequences, especially the text characters. It is much more efficient to just copy a character than to code it with the traditional intra prediction mode.

On the other hand, character-rich content introduces a problem. The foreground and background usually hold very different brightness with sharp edges between them. Fig. 2 and Fig. 3 demonstrate two examples. Because the coding process loses high-frequency information generally, the quality of reconstruction is deteriorated more when there are acute pixel value changes. The coding efficiency of IBC will be reduced when such reconstructed pixels are copied.

Fig. 1.  Intra block copy.

Fig. 2.  A block with two segments with sharp sample value changes.

Fig. 3.  A block with three segments with sharp sample value changes.

# Proposed method

A segmental prediction method for intra block copy (segIBC) was proposed [1]. When segIBC is applied, two steps are applied to modify the IBC prediction block before it is used to predict the current block. Fig. 4 illustrates an example of segIBC with 2 segments for an 8x8 block.

In the first step, samples in an IBC prediction block are classified into two or three segments. When the prediction block is divided into two segments, a sample is classified to segment 0 if its value is smaller than the average value of all the samples in the IBC prediction block. Otherwise, it is classified into segment 1. When the prediction block is divided into three segments, two thresholds T1 and T2 are calculated as T1= (T+Vmin)/2 and T2= (Vmax+T)/2, where T is the average value of all the samples in the IBC prediction block. Vmax and Vmin are the minimum sample value and the maximum sample value in the IBC prediction block respectively. A sample is classified into segment 0 if its value is smaller than T1; it is classified into segment 2 if its value is larger than T2; otherwise it is classified into segment 1.

In the second step, a single value is assigned to all samples in each segment of the IBC prediction block. The single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the IBC prediction block and O is an offset signaled from the encoder to the decoder. E = (Vsegmax+Vsegmin)/2, where Vsegmax and Vsegmin are the minimum sample value and the maximum sample value in the segment of the IBC prediction block respectively.

At encoder, the offset for the segment of the IBC prediction block is calculated by subtracting the average value of all the original sample values in the corresponding segment of the current block and E.

For an IBC CU, a flag is signaled to indicate whether segIBC is applied. If segIBC is on, another flag is signaled to indicate the number of segments, 2 or 3, and then followed by the coded offsets of the segments. segIBC is only applied for lossy coding.

Fig. 4.  An example of segIBC with 2 segments for an 8x8 block.

# Experimental results

The proposed method is integrated on SCM-2.0 [2]. All tests are conducted under the common test condition (CTC) [3] for SCC with the full picture IBC search range. Tests are conducted to investigate the results for different segmental configuration. Furthermore, it is tested to set all residues to be 0 (cbf=0) when segIBC is applied as a simplification. Table 1 shows the configuration for different tests.

Table 1.  Configurations for tests

|  | Segments | Always set cbf=0 |
| --- | --- | --- |
| Test1 | 2+3 | No |
| Test2 | 2 | No |
| Test3 | 2+3 | Yes |
| Test4 | 2 | Yes |

Tables 2-5 demonstrates the coding results in test 1 – test 4. It can be seen that the coding performance is similar in these tests. Therefore test 4 which is the simplest one is considered as the best tradeoff between coding gain and complexity increase.

Table 2.  Results for lossy coding in test 1 under CTC

Table 3.  Results for lossy coding in test 2 under CTC

Table 4.  Results for lossy coding in test 3 under CTC

Table 5.  Results for lossy coding in test 4 under CTC

# Conclusions

This contribution proposes a segmental prediction method for intra block copy (segIBC). The segIBC divides an IBC prediction block into segments and modify them separately. Experimental results reportedly show that the combination of horizontal SIBC and vertical SIBC can achieve 0.9% / 0.3% / 0.1% / 0.2% / 1.1% / 0.3% / 0.2% / 0.3% BD-rate savings for RGB, text & graphics with motion, 1080p / RGB, text & graphics with motion, 720p / RGB, mixed content, 1440p / RGB, mixed content, 1080p / YUV, text & graphics with motion, 1080p / YUV, text & graphics with motion,720p / YUV, mixed content, 1440p / YUV, mixed content, 1080p classes in AI configurations under the common test condition for screen content coding.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Zhang, J. An, X. Zhang, H. Huang, S. Lei, “Non-SCCE1: Segmental prediction for intra block copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0117, July 2014.

- SCM-2.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0/.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, July 2014.