| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0108r1 |
| --- | --- |

| Title: | Non-CE6: Improvement On Palette Sharing Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Marta Karczewicz, Rajan Joshi Feng Zou, Vadim Seregin, Joel Sole 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 Wei.Pu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The palette mode adopted at the Sapporo meeting contains a CU level syntax element palette_share_flag which specifies the palette entry signaling methods. If this flag equals to 1, the current CU’s palette is derived from previousPaletteSize and previousPaletteEntries without additional signaling. In this contribution, it is proposed to extend the palette_share_flag to exploit the additional coding gain when all reconstructed pixels within the current CU share the same color. It is reported that the proposed method achieves (0.3%, 0.3%, 0.3%) and (0.3%, 0.2%, 0.2%) BD-rate reduction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, under SCC common test condition.

# Technical Description

It is proposed to binarize palette_share_flag according to Table I.

Table I. Binarization and Semantics of palette_share_flag

| value | binarization | semantics |
| --- | --- | --- |
| 0 | ‘1’ | All of the pixels have the same color as previousPaletteEntries[ cIdx ][ 0 ] |
| 1 | ‘01’ | palette sharing mode semantics in spec. [2]. |
| 2 | ‘00’ | Non-sharing mode semantics in spec. [2]. |

If palette_share_flag equals to 0, the following palette syntax elements in the CU are bypassed. Specifically, the syntax table can be updated as:

palette_coding( x0, y0, nCbS ) {

palette_transpose_flag

palette_share_flag[ x0 ][ y0 ]

If (palette_share_flag[ x0 ][ y0 ] > 0) {

palette_transpose_flag

…

}

}

# Simulation Results

The proposed method was implemented on top of SCM2.0. Simulations configurations are specified in SCC common test condition [1].  Simulations performed on x86_64 Linux cluster with source code compiled using GCC4.1.2. Only AI results are presented in this section. Complete simulation results can be found in the attached spreadsheets.

Table 1. Proposed method v.s. SCM2.0 Anchor (Lossy)

Table 2. Proposed method v.s. SCM2.0 Anchor (Lossless)

# Conclusion

In this contribution, the syntax element of palette_share_flag is extended to exploit additional coding gain. It is reported that (0.3%, 0.3%, 0.3%) and (0.3%, 0.2%, 0.2%) BD-rate reduction are achieved for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, under SCC common test condition.

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1015).

- R. Joshi and J. Xu, “High efficiency video coding (HEVC) screen content coding: Draft 1,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1005-v2).

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).