| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0110r1 |
| --- | --- |

| Title: | Non-CE6: Syntax Redundancy Fixes for Palette Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Marta Karczewicz, Vadim Seregin, Feng Zou, Rajan Joshi, Joel Sole 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 Wei.Pu@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports three syntax redundancy fixes related palette mode.

First, the palette mode adopted at the Sapporo meeting contains a CU level syntax element palette_escape_val_present_flag which specifies whether escape pixel may exist in the current CU or not. If escape pixel does not exist and the palette size equals to 1, it can be implicitly inferred that all of the pixels in the current CU share the same color as the one in the palette while in the current design, redundant syntax elements, such as palette_mode, palette_run, are still required to be coded into the bitstream. This document proposes to simplify the codec by removing the two syntax elements when such case happens.

Second, in the scenario when palette size equals to zero, i.e. all of the pixels in the current CU are escape pixels, the pallete syntax elements but palette_escape_val can be bypsssed. Alternatively, a normative restriction can be imposed to ensure that when palette mode is used, the palette size is always greater than zero.

Third, the palette mode contains an implicit restrictions to the bitstream to improve coding efficiency. Specifically, an INDEX run mode will always select the longest possible run. In another word, an INDEX run mode cannot be follow by a pixel who has the same index. This document reports a redundancy associated with this restriction and proposes a method to fix it. It is reported that (0.3%, 0.3%, 0.3%) and (0.4%, 0.3%, 0.3%) BD-rate reduction can be achieved for AI RGB text & graphics with motion1080p and AI YUV text & graphics with motion 1080p.

# Technical Description

## Syntax Redundancy Fix Related to Palette Size Equals to 1

Using the notations in SCC Draft 1 [1]. for palette mode, if the following conditions are satisfied:

- palette_size == 1

- palette_escape_val_present_flag == 0

The following three syntax elements can be implicitly derived as:

palette_mode = INDEX

palette_index = 0

palette_run = nCbS * nCbS – 1

palette_transpose_flag = 0

However, according to the current draft specification, palette_transpose_flag, palette_mode, and palette_run are still signaled into the bitstream. At the decoder side, to parse and decode these syntax elements, it requires to decode:

One context coded bin of palette_transpose_flag (context is calculated by accessing the left neighbor)

One context coded bin of palette_mode (context is calculated by accessing the above neighbor)

Three context coded bins plus multiple bypass bins of palette_run

Table I specifies the number of bins the decoder needs to decode palette_run for different CU sizes.

Table I. CU size v.s. number of bins to code palette_run

| CU size | Number of context coded bin | Number of bypass bins | Total CABAC bins |
| --- | --- | --- | --- |
| 8×8 | 3 | 11 | 14 |
| 16×16 | 3 | 15 | 18 |
| 32×32 | 3 | 19 | 22 |
| 64×64 | 3 | 23 | 26 |

As the palette_run is coded using truncated rice code, at least 3 + (Number of bypass bins >> 1) bins cannot be decoded in parallel. This contribution proposes to simplify the decoder by removing such redundancy. Specifically, it is suggested to make the following modification to the working draft.

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_transpose_flag | ae(v) |
| … |  |
| scanPos = 0 |  |
| if (palette_size != 1 ¦¦ palette_escape_val_present_flag) { |  |
| palette_transpose_flag | ae(v) |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| … |  |
| } |  |
| } |  |
| … |  |

## Syntax Redundancy Fix Related to Palette Size Equals to 0

Using the notations in SCC Draft 1 [1]. if palette_size == 0, the only possibility is that all of the pixels in  the CU are escape pixels. Therefore, palette_transpose_flag and the following syntax elements except for palette_escape_val are not useful. It is proposed to bypass the signaling of these syntax elements in this case.

Allowing all escape pixels in a palette mode block does not enable any functionality that is not already presented in the current HEVC SCC design. For example, it is possible to use IPCM mode with quantization or a DC mode with transform skip to achieve similar results. Hence, it is proposed to disallowing palette_size equal to zero normatively by changing the semantics for palette_num_signalled_entries as follows:

palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit.

The value of palette_num_signalled_entries is restricted as follows:

- If numPredPreviousPalette is greater than 0, palette_num_signalled_entries shall be in the range of 0 to (max_palette_size – numPredPreviousPalette), inclusive.

- Otherwise (numPredPreviousPalette is equal to 0), palette_num_signalled_entries shall be in the range of 1 to max_palette_size, inclusive.

## Syntax Redundancy Fix Related to Copy Above Run Mode

As illustrated in Fig. 1, assuming that B is the end of INDEX run mode, then according to SCC draft specification [2].

Fig. 1. Example of the identified redundancy.

If pixel A’s index equals to pixel B’s index and they are not escape pixel, then palette_mode can be inferred to be INDEX or ESCAPE. In another word, it cannot be COPY_ABOVE. The reason is, if X is the starting point of a COPY_ABOVE run mode, then X’s index equals to A’s index, and therefore equals to B’s index. Due to coding efficiency, the adopted palette mode does not allow an index following an INDEX run mode with the same index value.

According to the above explanation, there is redundancy of signaling palette_mode if pixel A’s index equals to pixel B’s index and they are not escape pixel. To fix this problem without introducing additional parsing dependency, and without changing the syntax specification, this contribution proposes that if this condition occurs, and X selects COPY_ABOVE run mode, X’s index value is not copied from A but from D, as illustrated in Fig. 2.

Fig. 2. Example of the proposed fix.

# Simulation Results

The proposed method was implemented on top of SCM2.0. Simulations configurations are specified in SCC common test condition [1].  Simulations performed on x86_64 Linux cluster with source code compiled using GCC4.1.2. Complete simulation results can be found in the attached spreadsheets.

## Results on Fixing Palette Size Equals to 1

Table 1. Proposed method v.s. SCM2.0 Anchor (Lossy)

Table 2. Proposed method v.s. SCM2.0 Anchor (Lossless)

## Results on Fixing Palette Size Equals to 0

### Syntax Redundancy Removal

Table 3. Proposed method v.s. SCM2.0 Anchor (Lossy)

Table 4. Proposed method v.s. SCM2.0 Anchor (Lossless)

### Results on Fixing Copy Above Run Mode

Table 5. Proposed method v.s. SCM2.0 Anchor (Lossy)

Table 6. Proposed method v.s. SCM2.0 Anchor (Lossless)

# Conclusion

First, a syntax redundancy in the current palette mode was identified which affects the decoder’s throughput. It is proposed to remove this redundancy and simplify the codec. Simulation results show that in terms of coding efficiency, such modification shows small gain compared with SCM2.0 anchor.

Second, to deal with the corner case of palette_size = 0, two solutions were proposed. The first solution proposes to remove related syntax redundancy when palette_size = 0 while the other one propose to put a normative restriction to the specification without change any syntax.

Third, a syntax redundancy in the current palette mode was identified which affects the coding efficiency. A method is proposed to fix this redundancy without changing the syntax specification and without introducing additional parsing dependency. Simulation results show that in terms of coding efficiency, such modification shows (0.3%, 0.3%, 0.3%) and (0.4%, 0.3%, 0.3%) BD-rate reduction can be achieved for AI RGB text & graphics with motion1080p and AI YUV text & graphics with motion 1080p compared with SCM2.0 anchor.

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1015).

- R. Joshi and J. Xu, “High efficiency video coding (HEVC) screen content coding: Draft 1,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1005-v2).

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).