| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0111r3 |
| --- | --- |

| Title: | Non-CE6: Improvement On Palette Run Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Rajan Joshi, Marta Karczewicz Feng Zou, Vadim Seregin, Joel Sole 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 Wei.Pu@qti.qualcomm.com rajanj@qti.qualcomm.com martak@qti.qualcomm.com fzou@qti.qualcomm.com vseregin@qti.qualcomm.com joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The palette mode adopted at the Sapporo meeting uses coefficient coding method to code the palette_run syntax element, as in SCC draft 1. CE6 Test A.4, A.5, and A.6 were established to study alternative CABAC context design and binarization methods. In this contribution, an improvement on palette_run binarization is proposed, which achieves 1.7% and 2.1% BD-rate redunction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, compared with SCM2.0 anchor.

# Technical Description

## Binarization Method

In CE6 Test A.6 [3]. TEG0 is composed of a prefix truncated unary code and a suffix truncated binary code. To code a run_length value x using TEG0, it is assumed that its maximum achievable value is known, denoted as X. As an example, the TEG0 codeword for X = 6 is listed in Table I.

In the example, when x = 0, 1, 2, 3, the prefix code is not truncated. When x = 4, 5, 6, the prefix code is truncated to ‘111’.

Table I. Example of TEG0 (X = 6)

| Value x | Codeword (prefix-suffix) | Codeword length |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 10 | 2 |
| 2 | 110-0 | 4 |
| 3 | 110-1 | 4 |
| 4 | 111-0 | 4 |
| 5 | 111-10 | 5 |
| 6 | 111-11 | 5 |

In the proposed improved TEG0, when the prefix code is truncated, the corresponding suffix code is changed to:

if (x == X) suffix = ‘1’

else suffix = ‘0’ + ‘TEG0 suffix (max = X−1)’

As an example, the improved TEG0 codeword for X = 6 is listed in Table II.

Table II. Example of Improved TEG0 (X = 6)

| Value x | Code word (prefix-suffix) | Code word length |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 10 | 2 |
| 2 | 110-0 | 4 |
| 3 | 110-1 | 4 |
| 4 | 111-00 | 5 |
| 5 | 111-01 | 5 |
| 6 | 111-1 | 4 |

## Discussion

The proposed run-to-the-end signaling principal does not have to be packed with CE6 Test A.6, although on top of CE6 Test A.6 achieves higher BD-rate reduction. It can be built on top of other binarization by reserving a predefined codeword for the longest run.

# Simulation Results

The proposed method was implemented on top of CE6 Test A.6 software [3]. Simulations configurations are specified in SCC common test condition [1].  Simulations were performed on x86_64 Linux cluster with source code compiled using GCC4.1.2. Only AI lossy results are presented in this section. Complete simulation results can be found in the attached spreadsheets.

Table IV. Prthod in Section 1.1 using CE6 Test A.6 Config. 1 (i.e. new binarization only) v.s. SCM2.0 Anchor

Table V. Method in Section 1.1 Psing CE6 Test A.6 Config. 2 (i.e. new context and new binarization) v.s. SCM2.0 Anchor

The same as CE6 Test A.6 and SCM2.0, in the worst case, 1 context coded bin per pixel is used to code the palette_run syntax. The maximum number of context coded bin is 3 regardless of the length of palette_run.

Table VI presents an example to support the arguments in Subsection 1.2 by reserving palette_run == 3 as run-to-the-end. This simulation uses SCM2.0-like binarization by signaling greater than 0, greater than 1, greater than 2 flags first, followed by EG2, as presented in [4].

Table VI. Method in Section 1.2 (EG2) v.s. SCM2.0 Anchor

Table VII presents another example implementation to support the arguments in Subsection 1.2 by also reserving palette_run == 3 as run-to-the-end. No syntax change is required.

Table VI. Method in Section 1.2 (SCM2.0) v.s. SCM2.0 Anchor

# Conclusion

An improvement on palette run length binarization is proposed. Simulation results show that the proposed improvement achieves 1.7% and 2.1% BD-rate redunction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, compared with SCM2.0 anchor.

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1015).

- R. Joshi and J. Xu, “High efficiency video coding (HEVC) screen content coding: Draft 1,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R1005-v2).

- R. Joshi, W. Pu, M. Karczewicz, F. Zou, and V. Seregin, “CE6 subtest A.6: Binarization sfor run coding in palette mode,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 (JCTVC-S0039).

- R. Joshi, M. Karczewicz, W. Pu, V. Seregin, and J. Sole, “Non-SCCE3: Contexts for coding index runs,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 (JCTVC-R0174).

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).