# Method Proposed in Section 1.2

# On Top of SCM2.0

#### 9.3.3.13		Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run. A boolean variable isRunToEnd indicating whether the palette_run reaches the end of the CU is defined as:

isRunToEnd = (nCbS * nCbS – 1 == scanPos)

Output of this process is the binarization of palette_run as specified in Table 9-1. [Ed. (GJS): Check/fix use of quote marks in table. Also, the last row of the table seems to have some problems, e.g., w.r.t. clarity of "TR code".]

A variable paletteRunAdjusted is defined as:

- if palette_run < 3, paletteRunAdjusted = palette_run;

- else if isRunToEnd, paletteRunAdjusted = 3;

- else paletteRunAdjusted = palette_run + 1;

Table 9-1 – Binarization for palette_run

| palette_run paletteRunAdjusted | Codeword |
| --- | --- |
| 0 | '0' |
| 1 | '10' |
| 2 | '110' |
| >2 | Prefix = '111', Suffix = TR code with symbolVal = palette_run paletteRunAdjusted − 3 , cRiceParam = 3 |

# On Top of EG2 Based Binarization [JCTVC-R0174]

#### 9.3.3.13		Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run. A boolean variable isRunToEnd indicating whether the palette_run reaches the end of the CU is defined as:

isRunToEnd = (nCbS * nCbS – 1 == scanPos)

Output of this process is the binarization of palette_run as specified in Table 9-1. [Ed. (GJS): Check/fix use of quote marks in table. Also, the last row of the table seems to have some problems, e.g., w.r.t. clarity of "TR code".]

A variable paletteRunAdjusted is defined as:

- if palette_run < 3, paletteRunAdjusted = palette_run;

- else if isRunToEnd, paletteRunAdjusted = 3;

- else paletteRunAdjusted = palette_run + 1;

Table 9-2 – Binarization for palette_run

| palette_run paletteRunAdjusted | Codeword |
| --- | --- |
| 0 | '0' |
| 1 | '10' |
| 2 | '110' |
| >2 | symbolVal = paletteRunAdjusted – 3, Prefix = '111', Suffix = TR EG2 code with symbolVal = palette_run paletteRunAdjusted − 3, cRiceParam = 3 |

# On Top of CE6 Test A.4, A.5, and A.6

#### 9.3.3.13		Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run. A boolean variable isRunToEnd indicating whether the palette_run reaches the end of the CU is defined as:

isRunToEnd = (nCbS * nCbS – 1 == scanPos)

Output of this process is the binarization of palette_run as specified in Table 9-1. [Ed. (GJS): Check/fix use of quote marks in table. Also, the last row of the table seems to have some problems, e.g., w.r.t. clarity of "TR code".]

A variable paletteRunAdjusted is defined as:

- if palette_run < 3, paletteRunAdjusted = palette_run;

- else if isRunToEnd, paletteRunAdjusted = 3;

- else paletteRunAdjusted = palette_run + 1;

Table 9-3 – Binarization for palette_run

| palette_run paletteRunAdjusted | Codeword |
| --- | --- |
| 0 | '0' |
| >0 | Prefix = '1', Suffix = 0-th order TEGk code symbolVal = palette_run paletteRunAdjusted – 1, cTEGParam = nCbS * nCbS – 2 – scanPos |

# On Top of JCTVC-S0269

#### 7.3.8.x Palette run syntax

| palette_run _coding( MaxPaletteRun) { | Descriptor |
| --- | --- |
| if (MaxPaletteRun > 0) { |  |
| palette_run msb_id_plus1 | ae(v) |
| palette_run refinement_bits | ae(v) |
| paletteRunIdc = (palette_run _msb_id_plus1 > 1 )? (1 << (palette_run _msb_id_plus1–1)) + palette_run refinement_bits |  |
| paletteRun = (paletteRunIdc == 3) ? MaxPaletteRun : (paletteRunIdc < 3) ? paletteRunIdc :paletteRunIdc – 1 |  |
| } else |  |
| paletteRun = 0 |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).