| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0112-r1 |
| --- | --- |

| Title: | Non-CE2: On Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, several changes are proposed for Intra block copy, including enabling line buffer for block vector predictor derivation, using the deblocking process like Inter, and using DCT for 4x4 luma block. With these changes, the performance is reported to be an average BD-rate of -0.8% and -0.9% respectively for RGB and YUV 1080p text & graphics with motions sequences under AI configuration.

# Introduction

In the current Intra block copy (Intra BC) mode, the above spatial neighbor Intra BC blocks are considered as unavailable in the block vector (BV) predictor derivation process if these blocks are out of current CTU. Additionally, Intra BC mode is treated as Intra in the deblocking process, and DST is used for Intra BC 4x4 luma block.

# Proposed

In this contribution, the following changes are proposed for Intra BC mode.

## Enable line buffer for BV predictor derivation

In HEVC AMVP, the above spatial neighbors can be out of current CTU, and line buffer is needed to store the motion vector (MVs) of the above row. Thus, this line buffer can be shared with Intra BC mode such that the above spatial neighbors can be used for BV predictor as well with no additional buffer requirement.

## Use Inter-like deblocking process

Intra BC often selects the region which contains sharp edges (e.g., text), it may be not appropriate to use strong deblocking filters as Intra to filter these edges, especially for the boundary of two Intra BC blocks, which may result in worse subjective and objective video quality. Similar situation happens along the boundary of an Inter coded block and an Intra BC coded block.

In this contribution, the Intra BC is treated as Inter during the deblocking process instead. Moreover, in order to keep the deblocking module unchanged, after an Intra BC block is decoded and before the deblocking process, it is converted to be a uni-directionally predictive Inter block with the BV being its MV and the current picture being the reference picture.

## Use DCT for Intra BC 4×4 luma block

As the prediction signal generation for Intra BC is similar to the motion compensation process for Inter, it is proposed to use DCT for Intra BC 4x4 luma block.

# Encoder improvements

In this section, two encoder improvements are proposed to better align with the software in CE2.

## Method 1

In Intra BC fast encoder search of the current SCM, the RD costs for Intra BC mode are checked only if the CU size is 16x16 and 8x8. In method 1, the RD costs of 2Nx2N partition modes for Intra BC are checked for all the CUs. Also, for the 2Nx2N partition of every CU size, the RD costs are checked with BV being one of the spatial merge candidates in HEVC (merge mode is not used).

## Method 2

In Intra BC fast encoder search of the current SCM, only the RD cost of 2Nx2N partition mode is checked for 16x16 CU. In method 2, the RD costs of 2NxN and Nx2N partition modes are checked as well for 16x16 CU.

# Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM2.0, and SCC common test conditions [1] are used. The BD-rates are provided for the lossy case under different encoder configurations.

- Test 1: Lossy results for enabling line buffer for Intra BC BV prediction

- Test 2: Lossy results for using the deblocking process as Inter for Intra BC

- Test 3: Lossy results for using DCT for Intra BC 4x4 luma block

- Test 4: Combination of Test1 and the encoder improvement method 1

- Test 5: Combination of Test1, Test2 and Test 3.

- Test 6: Combination of Test1, Test2, Test 3 and the encoder improvement method1 and method 2

Table 1 Lossy results for Test 1

Table 2 Lossy results for Test2

Table 3 Lossy results for Test3

Table 4 Lossy results for Test 4

Table 5 Lossy results for Test 5

Table 6 Lossy results for Test 6

# Draft Text Changes

The proposed text changes are based on the document JCTVC-R1005-v3. The changes are marked in yellow.

#### 8.4.4.2	Derivation process for intra block copy block vector prediction candidates

If the output is equal to TRUE, and (yPb/CtbSizeY) is equal to ( ( yPb − 1 ) / CtbSizeY ), availableFlagB is set equal to TRUE, and bvIntraB is set equal to bvIntra[ xPb + nPbW − 1 ][ yPb − 1 ]

…

#### 8.4.6.1 General

Depending on the values of CuPredMode[ xTbY ][ yTbY ], nTbS, and cIdx, the variable trType is derived as follows:

–	If CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA and intra_bc_flag[ xTbY ][ yTbY ] is equal to 0, nTbS is equal to 4, and cIdx is equal to 0, trType is set equal to 1.

–	Otherwise, trType is set equal to 0.

...

#### 8.7.2.4 Derivation process of boundary filtering strength

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode and intra_bc_flag is equal to 0, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with top-left luma sample covering ( xPb, yPb ), is equal to PredFlagL0[ xPb ][ yPb ] + PredFlagL1[ xPb ][ yPb ].

NOTE 3 – For the coding unit with intra_bc_flag is equal to 1, the motion vector is set equal to the block vector, the number of motion vectors is set equal to 1, and the reference picture is the current picture.

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-R1015, Sapporo, JP, July 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).