| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0114 |
| --- | --- |

| Title: | CE6-related : Enabling copy above mode prediction at the boundary of CU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Yu-Chen Sun, Shan Liu, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei 2860 Junction Ave. San Jose, CA, U.S.A. | Email: | Jungsun.kim@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes enabling ‘Copy Above’ prediction mode for the first line of the CU when it is coded as Palette mode. The index of neighbouring pixels of the CU is used as index predictor and the indices are derived with the palette of the current CU. It shows maximum 2.5%, 2.5%, 2.5% gain and average 0.6%, 0.7%, 0.7% gain for All Intra configuration. Maximum1.3%, 1.5%, 1.4% and average 0.4%, 0.5%, 0.5% gain for Random Access, Maximum 0.9%, 1.2%, 1.2% and average 0.3%, 0.4%, 0.3% gain for Low Delay cases respectively. This contribution presents 4 variations of the method to decide the indices of neighbouring pixels.

# Introduction

In Palette mode, there are two prediction modes indicate from which direction the current index is copied. ‘Copy Above’ prediction mode indicates that current index is copied from above index across the scanning order.

| - Horizontal scan | - Vertical scan |
| --- | --- |
| Figure 1 Scanning order and reference pixel for copy above mode |  |

Figure 1 shows the reference pixels when the scanning order is horizontal (a) and vertical (b). In the case of horizontal scanning, the reference of ‘Copy Above’ prediction mode is a pixel above current pixel and in the case of vertical scan, the reference of ‘Copy Above’ prediction mode is a pixel left of current pixel.

When the current pixel is at the first row in the case of horizontal scan or at the first column in the case of vertical scan, it doesnot have any reference pixel within the CU. Therefore its prediction mode is derived as ‘Index Mode’ prediction without signalling.

# Proposed Method

When the current pixel is at the first row in the case of horizontal scan or at the first column in the case of vertical scan, it can refer neighbouring pixels in neighbouring CUs if they are available.[1] In Figure 2, grey pixels indicate pixels in the current CU which are predicted by ‘Copy Above’ mode and orange pixels indicate neighbouring pixels in the neighbouring CU.

| - Horizontal scan | - Vertical scan |
| --- | --- |
| Figure 2 scanning order and reference pixels for copy above mode when the pixel is at the boundaries |  |

In order to decide the index of neighbouring pixels, the palette of current CU is used to increase of the correlation between indices and to avoid keeping the palette of neighbouring CUs. Also, ESCAPE index which indicates the value of the pixel cannot be decided by the palette table is not used for neighbouring pixels. Indices of neighbouring pixels are derived as following :

Idxi=argmin0≤n<Nk=0CN[i,k]−P[n,k]

Where Idxi means index of i-th neighbouring pixel, N[i,k] indicates k-th component value of i-th neighbouring pixel and P[n,k] indicates k-th component value of n-th entry of Palette table. C indicated number of colour components used to determine the index of neighbouring pixels in current picture.

Since ‘Copy Above’ mode is available at the first line, prediction modes are signalled for the pixels at the first line if necessary. Other signalling methods remain same as before.

# Simulation Results

The proposed method is implemented on top of SCM2.0 under the common test conditions in CE6 [2, 3].

## 3 components and maximum palette size

To decide indices of neighbouring pixels, values of 3 components of neighbouring pixel are compared to 3 components of every entry in the palette.

Table 1 3 colour components, max.31 palette entries : lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.1% | -2.1% | -2.1% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.5% | -2.5% |
| YUV, text & graphics with motion,720p | -0.8% | -0.9% | -1.0% |
| YUV, mixed content, 1440p | -0.5% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -0.5% | -0.7% | -0.6% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 115% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.1% | -1.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% | -0.5% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.5% | -1.4% |
| YUV, text & graphics with motion,720p | -0.5% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -0.2% | -0.6% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.1% | -1.0% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.5% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.4% | -0.7% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.1% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 104% |  |  |

Table 2 3 colour components, max.31 palette entries : lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.0% | 0.3% | 1.4% |
| RGB, text & graphics with motion,720p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.3% | 1.7% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.1% | 0.5% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.3% | 0.9% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.7% | 0.3% | 1.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.3% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.3% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

## luma only and maximum palette size

To decide indices of neighbouring pixels, values of luma component of neighbouring pixel are compared to luma components of every entry in the palette.

Table 3 luma only, max.31 palette entries : lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.4% | -2.3% | -2.3% |
| YUV, text & graphics with motion,720p | -0.7% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -0.5% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.4% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| YUV, mixed content, 1440p | -0.3% | -0.5% | -0.5% |
| YUV, mixed content, 1080p | -0.1% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.7% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% | -0.5% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.4% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -1.1% | -1.0% |
| YUV, text & graphics with motion,720p | -0.4% | -0.3% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.2% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4 luma only, max.31 palette entries : lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.7% | 0.7% | 0.3% | 1.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.3% | 1.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.5% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.2% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.7% | 0.3% | 1.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.2% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.3% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

# Reference

- Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, “Non-RCE4: Cross-CU major color index prediction” ”, JCTVC-P0093, San Jose, U.S.A., Jan. 2014.

- Yu-Wen Huang, Patrice Onno, Robert Cohen, Vadim Seregin, Xiaoyu Xiu, Zhan Ma, “Description of Core Experiment 6 (CE6): Improvements of palette mode”, JCTVC-R1106, 18th Meeting: Sapporo, June-July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, 18th Meeting: Sapporo, June-July 2014.

# Patent rights declaration(s)

MediaTek Inc. USA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).