| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0115 |
| --- | --- |

| Title: | CE6-related : Clarifying decoder’s ambiguous behaviour for escape index in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim1, Shan Liu1, Tzu-Der Chuang2,Yu-Wen Huang2, Shawmin Lei2 1 2860 Junction Ave. San Jose, CA, U.S.A. 2 No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jungsun.kim, shan.liu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two approaches to clarify decoder’s behaviour for escape index handling in palette mode. The horizontal traverse scan is assumed for describing the approaches as follows. One approach is preventing each escape index from being a reference pixel of ‘copy above’ mode by limiting the number of runs of ‘copy above’ mode at decoder side as it works at encoder side. It also derives prediction mode of a pixel below escape index as ‘index mode’ without signalling the mode. The other approach is allowing each escape index being a reference pixel of ‘copy above’ mode by copying the escape values of the above escape pixel. The two approaches for vertical traverse scan can be easily understood by analogy, and both the approaches can solve the ambiguity issue successfully with very little impacts on coding efficiency.

# Introduction

An escape index indicates that the corresponding pixel has a value which is not included in the palette table. The values of individual colour samples are signalled after signalling an escape index. In general, an escape index implies that the pixel value is eccentric in the block. Therefore, it is not used as a reference index. Figure 1 shows an example. If the gray pixel has an escape index, the orange pixel below the escape index cannot have ‘copy above’ mode. Currently, this process is controlled by the encoder. The encoder prevents pixels are predicted from escape index by not including a pixel below the escape index into ‘copy above’ mode set. However, the decoder’s behaviour on referring to escape indices is not defined.

Figure 1.  An example of SCM-2.0 encoder – an escape pixel cannot be a reference pixel.

# Proposed methods

This contribution proposes two methods to complete the undefined description of decoder’s behaviour in the case that an escape index is present.

## Preventing an escape index from being a reference index

The current encoder prevents pixels with ‘copy above’ mode from referring to escape indices. We can impose the same restriction to the decoder as the current encoder does.

### Derivation of prediction mode

If a pixel is below (to the left of) an escape index when the horizontal (vertical) traverse scan is selected, its prediction mode is not signalled and is derived as ‘index mode’ since the pixel should not refer to the above (left) index. Figure 2 shows an example for each of the two scanning orders.

Figure 2.  Examples of deriving prediction modes.

### The text is modified as Table 1 when the prediction mode is derived for the pixel below ESCAPE index.

Table 1 Text modification

...

if( scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE && palette_mode[xC_prev][yC_prev-1] != ESCAPE)

palette_mode[ xC ][ yC ]

…

### Limiting number of run for ‘copy above’ mode

In order to prevent pixels from referring to escape indices, the number of run of ‘copy above’ mode should be limited. Figure 3 shows an example of horizontal traverse scan. When the gray pixel has an escape index and ‘copy above’ mode starts from the green pixel, its run should end before the orange pixel. In this example, therefore, maximum run length of the ‘copy above’ mode is 7. Since the maximum number can be decided by the location of the escape index scanned before the current pixel, binarization of the number of run can be changed with the corresponding maximum value.

Figure 3.  Examples of limiting number of run.

Given the position in the scanning order of the start of ‘copy above mode’ Ic (the green pixel in Figure 3), the position in the scanning order of ‘escape’ index Ie1 (the gray pixel in Figure 3), and the position in the scanning order of the pixel below the ‘escape’ index Ie2 (the orange pixel in Figure 3), we know Ic is larger than Ie1 and smaller than Ie2, and the maximum run value is Ie2 – Ic – 1.

With the maximum possible run, the decoder finishes ‘copy above’ prediction before the pixel right below the escape index. In addition, the number of run is binarized with Exponential Golomb and truncated binary code to reduce bits.

## Allowing an escape index to be a reference index

The other proposed method to handle the undefined decoding behaviour on referring to escape indices is to define the decoding process for this situation. When the above (left) pixel has an escape index and ‘copy above’ mode is selected, the index of current pixel is set to the escape index and the escape values of three color components of the current pixel are copied from that of the above escape pixel. The syntax table modification is shown in Table 1.

Table 2 Syntax table modification

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| if( palette_mode[ xC ][ yC ] = = INDEX ) { |  |
| palette_mode[ xC ][ yC ] = INDEX |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } else { |  |
| palette_mode[ xC ][ yC ] = COPY_ABOVE |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ y − 1 ] |  |
| if (palette_mode[ xC ][ yC ] == ESCAPE) { |  |
| palette_mode[ xC ][ yC ] = ESCAPE |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = paletteEscapeVal[ cIdx ][ xC ][ yC-1 ] |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

# Simulation results

The proposed methods are implemented on top of SCM-2.0 under the common test conditions in [2]. The proposed method described in 2.1.2 can be applied to other types of binarization for number of run as studied in CE6 [3].

## Preventing an escape index from being a reference index

Table 2 and Table 3 show the results for lossy and lossless coding respectively when the binarization for number of run is changed by the restriction. It shows maximum 0.1%, 0.1%, 0.1% gains for each case. Encoding and decoding time are inaccurate.

## Allowing an escape index to be a reference index

The proposed method can be implemented as only changing the decoding process without modifying the encoder searching process. The coding efficiency is not changed in this way. If we allow encoder to use copy above mode when the escape values of two vertical adjacent pixels are the same, Table 6 and Table 7 show the results for lossy and lossless coding respectively. The runtimes are not accurate. The results reportedly show negligible coding performance change by modifying the encoding searching process. The runtimes are not accurate.

# Conclusion

This contribution proposes two approaches to clarify decoder’s behaviour for referring to escape indices. One approach is preventing any escape index from being a reference pixel of ‘copy above’ mode by limiting number of runs of ‘copy above’ mode at decoder side. The other approach is to define the decoding process when an escape index is referred. The palette index and the escape values of three color components are copied from the above pixel.

# Reference

- R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, “CE6 subtest A.6: Binarization for run coding in palette mode”, JCTVC-S0039, October, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, "Common Test Conditions for Screen Content Coding", JCTVC-R1015, August, 2014.

- Y.-W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 6 (CE6): Improvements of palette mode,” JCTVC-R1106, August, 2014.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 3 Lossy results of preventing ESCAPE being reference index

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.4% | 0.4% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.4% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 105% |  |  |

Table 4 Lossless results of preventing ESCAPE being reference index

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.1% | 0.1% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.1% | 0.0% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 6.  Lossy results of allowing ESCAPE being reference index

Table 7.  Lossless results of allowing ESCAPE being reference index