| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0117 |
| --- | --- |

| Title: | CE3: Test 1.3 PU Intra block copy with flipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye Xiaozhong Xu Shan Liu Kai Zhang Shawmin Lei | Tel: Email: | +1-408-526-1899 ext 88134 jing.ye@mediatek.com xiaozhong.xu@mediatek.com shan.liu@mediatek.com |
| Source: | Mediatek Inc. |  |  |

_____________________________

# Abstract

This proposal presents the CE3 test 1.3 “PU intra block copy flipping”. It proposes 2 methods. Method 1 applies PU level flipping with PU level IBC on/off. Method 2 applies PU level flipping with CU level IBC on/off. It is reported that the proposed Method 1 achieves 1.4%, 2.1% and 2.2% BR-rate savings for 1080p text and graphics YUV AI RA LB cases respectively against the SCM 2.0 anchor in the full frame intra BC test condition; Method 2 achieves 1.4%, 0.8% and 0.6% BR-rate savings for 1080p text and graphics YUV AI RA LB cases respectively against the SCM 2.0 anchor in the full frame intra BC test condition.

# Introduction

In the Sapporo meeting, CE3 [1] was established to study different sub-block partition and flipping for intra block copy. In this proposal, it mainly evaluates the PU level flipping method in intra block copy.

# Proposed PU Intra block copy with flipping

It is proposed to enable PU level intra block copy flipping, intra block copy could be applied on 2Nx2N, 2NxN, Nx2N, and NxN blocks. The search range for the proposed method is full frame search. Both horizontal flipping and vertical flipping is enabled for intra block copy predicted blocks as shown in Fig. 1. ibc_flipping_flag and ibc_flipping_direction will be signalled for each PU. The details about the signalling could be found in the text specification.

Fig. 1 vertical flipping and horizontal flipping

## Method 1 PU level Intra block copy with flipping with IBC signal on/off at PU level

PU level flipping could be combined with IBC signal on/off at PU level [3]. The signalling of IBC will change from CU level to PU level. The performance of lossy and lossless coding is shown in Table 1 and Table 2.

## Method 2 PU level Intra block copy with flipping with IBC signal on/off at CU level

PU level flipping uses IBC signal on/off at CU level. The performance of lossy and lossless coding is shown in Table 3 and Table 4.

# Simulation results

The proposed scheme is implemented on top of SCM2.0 and tested using the common test condition defined in [2]. Table 1, Table 2 demonstrate the lossy and lossless coding performance under full frame intra BC against SCM 2.0 software of Method 1. Table 3 and Table 4 demonstrate the lossy and lossless coding performance under full frame intra BC against SCM 2.0 software of Method 2. The encoding time is inaccurate.

Table 1. Lossy results of Method 1 in Full frame Intra BC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -0.6% | -0.5% | -0.6% |
| RGB, mixed content, 1440p | -1.3% | -1.3% | -1.3% |
| RGB, mixed content, 1080p | -1.1% | -0.9% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.4% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.6% |
| YUV, mixed content, 1440p | -1.3% | -1.4% | -1.4% |
| YUV, mixed content, 1080p | -1.0% | -1.2% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.2% | -2.0% | -2.0% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.1% | -1.9% | -1.9% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.9% | -0.9% | -0.8% |
| YUV, mixed content, 1080p | -0.8% | -1.0% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.2% | -2.0% | -2.0% |
| RGB, text & graphics with motion,720p | -0.8% | -1.0% | -0.7% |
| RGB, mixed content, 1440p | -0.7% | -1.0% | -0.6% |
| RGB, mixed content, 1080p | -0.7% | -0.5% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -2.2% | -1.9% | -2.0% |
| YUV, text & graphics with motion,720p | -1.0% | -0.6% | -0.9% |
| YUV, mixed content, 1440p | -0.7% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -0.5% | -0.5% | -0.7% |
| YUV, Animation, 720p | 0.1% | -0.6% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2. Lossless results of Method 1 in Full frame Intra BC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.4% | 1.2% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.1% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.5% | 1.2% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.1% | 0.9% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.1% | 0.4% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.1% | 1.1% | 0.9% | 1.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.1% | 0.9% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 1.2% | 1.0% | 1.3% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.2% | 1.2% | 0.9% | 1.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 0.9% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.3% | 1.2% | 1.0% | 1.4% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

Table 3. Lossy results of Method 2 in Full frame Intra BC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -1.3% | -1.3% | -1.3% |
| RGB, mixed content, 1080p | -1.1% | -1.0% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.4% | -1.2% | -1.3% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -0.6% |
| YUV, mixed content, 1440p | -1.3% | -1.4% | -1.4% |
| YUV, mixed content, 1080p | -1.0% | -1.2% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.7% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -0.6% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -0.6% | -0.7% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -0.3% |
| YUV, mixed content, 1440p | -0.6% | -0.7% | -0.6% |
| YUV, mixed content, 1080p | -0.5% | -0.9% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.4% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.5% | -0.3% | -0.9% |
| YUV, mixed content, 1440p | -0.3% | -0.8% | -0.4% |
| YUV, mixed content, 1080p | 0.0% | -0.4% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4. Lossless results of Method 2 in Full frame Intra BC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.4% | 1.2% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.1% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.6% | 1.2% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.1% | 0.9% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.1% | 0.4% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 0.9% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.0% | 0.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.6% | 0.4% | 0.8% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.2% | 0.7% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.0% | 0.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 0.5% | 0.3% | 0.7% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusion

The performance change (BD rate reduction) of CE3 test 1.3 “PU intra block copy flipping” is reported in this contribution.

# References

[1] S. Liu, “Description of Core Experiment 3 (CE3): Sub-block partitioning and flipping for Intra block copy”, JCTVC-R1103, 18th Meeting: Sapporo, June-July 2014.

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, 18th Meeting: Sapporo, June-July 2014.

[3] X. Xu, S. Liu, S. Lei “CE2 Test 2: PU level IntraBC signaling” JCTVC_S0121, 19th meeting Strasbourg, Oct 2014.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).