| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0120 |
| --- | --- |

| Title: | Non CE6: Copy previous mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye Jungsun Kim Shan Liu Polin Lai Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-526-1899 ext 88134 jing.ye@mediatek.com jungsun.kim@mediatek.com shan.liu@mediatek.com PoLin.Lai@mediatek.com |
| Source: | Mediatek Inc. |  |  |

_____________________________

# Abstract

This proposal presents a “copy previous” mode with the use of neighbour indices for palette coding. The proposed “copy previous” mode is signalled by the “copy above” mode flag in current SCM, followed by an offset which specifies the distance between the previous row and the current row. It is proposed that the “copy previous” mode starts from the first row in the CU. The simulation results report that the proposed method achieves 3.9%, 1.9% and 1.5% BR-rate savings for “YUV 1080p text and graphics” AI, RA and LB configurations, respectively against the SCM 2.0 anchor.

# Introduction

In the Sapporo meeting, CE6 [1] was established to study different palette coding methods. In CE6 category C “Additional color index representation modes”, there are several methods proposed to further improve the coding efficiency of palette index map coding. Test C.3 Copy-from-previous-row mode[2] introduced new prediction mode, in which other previously coded pixels rows in the current CU can be utilized as reference in Copy from previous mode.

# Proposed copy previous mode

It is proposed to have copy previous mode instead of copy above mode in palette index coding. When the offset in copy previous mode equals to 1, it is the same to copy above mode. Thus copy previous mode will cover copy above mode. The proposed method enables copy previous mode for the first two rows. In this proposal, it is proposed to utilize the samples in above CU or left CU as reference for coding the palette indices of the first two rows when they are available. The index of pixels in neighbouring CU is derived by applying current color palette on reconstructed neighbouring pixels. When the above CU or left CU are not available such as frame boundary, slice boundary, or tile boundary, the first two rows use index 0 as reference. An illustration of the proposed mode is shown in Fig. 1. In Fig.1 row 0 is copied from neighbor pixels.

Fig. 1. Copy from previous row example

# Redundancy removal for the proposed method

Index redundancy removal can be applied when the current pixel is located from second line when its left pixel has copy above mode or from the third line when its left pixel has copy previous mode. Since decoder may not have the actual indices of predictors in neighboring CU at the parsing stage, actual indices of the current index map are calculated after reconstructing neighbor’s indices.

# Simulation results

The proposed scheme is implemented on top of SCM2.0 and tested using the common test condition defined in [3]. Table 1 and Table 2 demonstrates the lossy and lossless coding performance against SCM 2.0 software

Table 1. Lossy results in Full frame Intra BC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.3% | -3.5% | -3.5% |
| RGB, text & graphics with motion,720p | -1.3% | -1.5% | -1.4% |
| RGB, mixed content, 1440p | -0.5% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -1.5% | -2.0% | -1.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.9% | -4.0% | -4.0% |
| YUV, text & graphics with motion,720p | -1.3% | -1.6% | -1.9% |
| YUV, mixed content, 1440p | -0.7% | -1.6% | -1.4% |
| YUV, mixed content, 1080p | -1.7% | -3.8% | -3.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.9% | -1.8% |
| RGB, text & graphics with motion,720p | -1.0% | -1.1% | -1.1% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -1.0% | -1.5% | -1.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.9% | -2.3% | -2.3% |
| YUV, text & graphics with motion,720p | -1.0% | -1.4% | -1.7% |
| YUV, mixed content, 1440p | -0.5% | -1.3% | -0.8% |
| YUV, mixed content, 1080p | -1.2% | -3.6% | -2.8% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.5% | -1.7% | -1.6% |
| RGB, text & graphics with motion,720p | -1.2% | -1.1% | -1.1% |
| RGB, mixed content, 1440p | -0.3% | -0.5% | -0.3% |
| RGB, mixed content, 1080p | -1.0% | -1.4% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.5% | -2.0% | -1.9% |
| YUV, text & graphics with motion,720p | -0.8% | -1.0% | -1.5% |
| YUV, mixed content, 1440p | -0.5% | -1.8% | -1.6% |
| YUV, mixed content, 1080p | -1.1% | -3.5% | -3.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Lossless results in Full frame Intra BC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.5% | 1.7% | 0.5% | 2.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 0.8% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 2.1% | 0.5% | 3.0% |
| YUV, text & graphics with motion,720p | 0.4% | 0.5% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.9% | 0.4% | 1.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 1.2% | 0.4% | 1.8% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.6% | 0.4% | 1.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.9% | 0.3% | 1.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusion

The proposed copy previous mode enables prediction beyond immediately above rows, which improves the performance.

# References

- Yu-Wen Huang, Patrice Onno, Robert Cohen, Vadim Seregin, Xiaoyu Xiu, Zhan Ma, “Description of Core Experiment 6 (CE6): Improvements of palette mode”, JCTVC-R1106, 18th Meeting: Sapporo, June-July 2014.

- F. Zou, W. Pu, M. Karczewicz, R. Joshi, V. Sergin, “CE6: Test C.3 Copy previous row mode for palette coding”, JCTVC-S0174, 19th Meeting: Strasbourg, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, 18th Meeting: Sapporo, June-July 2014.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).