| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0121 |
| --- | --- |

| Title: | CE2 Test 2 – Intra BC signalled at PU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek |  |  |

_____________________________

# Abstract

This document reports the experimental results of test 2 in CE2. In this test, Intra BC mode is signalled at PU level. In test 2a, Intra BC mode is coded as an Intra mode; in test 2b, Intra BC mode is coded as an Inter mode. The experimental results show the proposed method in test 2a brings average 1.4% and 1.7% bitrate saving for RGB TGM 1080p RA and LB lossy coding, respectively; the proposed method in test 2b brings average 1.7% and 2.0% bitrate saving for RGB TGM 1080p RA and LB lossy coding, respectively. Performance gain is also observed for the other cases.

# Introduction

In current SCM-2.0, the signalling of Intra BC mode is done at CU level, if intra_bc_flag is equal to 1, all the PUs inside this CU should be coded in Intra BC mode.

It is observed that signalling of this mode at PU level can be a more flexible syntax structure, where each PU is allowed to decide its coding mode (Inter or Intra BC) when applicable [1].

# Solution

The signalling flag of Intra BC mode is moved from CU syntax to PU syntax. In CE2 [2] test 2a, Intra BC mode is coded as an Intra mode; in CE2 test 2b, Intra BC mode is coded as an Inter mode.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 default coding structure is used as the anchor. The common test conditions specified in [3] are followed. The summarized experimental results of test 2a for lossy and lossless coding are shown in Table 1 and 2, respectively. The summarized experimental results of test 2b for lossy and lossless coding are shown in Table 3 and 4, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding in test 2a

Table 2 Experimental results for lossless coding in test 2a

Table 3 Experimental results for lossy coding in test 2b

Table 4 Experimental results for lossless coding in test 2b

# Conclusion

The results of CE2 Test 2- Intra BC signalled at PU level are presented in this proposal.

# References

- JCTVC-R0356, “PU level IntraBC on/off and signalling,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R1102, “Description of Core Experiment 2 (CE2): IBC signalling and partitioning,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-R1005_v3.

Please refer to documents for test 2a and 2b respectively.