| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0123 |
| --- | --- |

| Title: | Non-CE2: Intra BC merge mode with default candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Tzu-Der Chuang Shan Liu Shawmin Lei | Email: | xiaozhong.xu@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek |  |  |

_____________________________

# Abstract

This document proposes to add default BVs to the merge candidate list if some of the entries in the list are empty. In this proposal, the syntax structure is the same as in CE2 test 5b, the changes made is to add some preset values as additional Intra BC merge candidates to fill up the merge candidate list. The experimental results show that the proposed method brings on average 5.3%, 6.7% and 7.0% bitrate saving as against SCM2.0 anchor for RGB TGM 1080p AI, RA and LB lossy coding, respectively; the proposed method brings on average 3.4%, 6.3% and 6.9% bitrate saving as against SCM2.0 anchor for YUV TGM 1080p AI, RA and LB lossy coding, respectively. Performance gain is also observed for the other cases.

# Introduction

In CE 2 [1] test 5b [2], the merge candidate is derived using spatial/temporal neighboring coded blocks. If merge mode is used for the current prediction block and the referred candidate block (by merge index) is an Intra BC coded one, the current prediction block is also coded in Intra BC mode. The BV from the referred Intra BC coded block will also be used for the current prediction block. If the referred block is an inter coded block, the current prediction block will also be an inter coded block, as described in HEVC v1.

In HEVC, when the merge candidate list is not full, (0, 0) vector of different reference pictures will be used to fill the empty entries in the merge list. In this proposal, it is proposed to use some preset BV values to fill these empty entries.

# Solution

It is proposed in this contribution that default BVs are inserted in to the following position of the merge list construction:

- Merge candidate list construction:

  - Spatial and temporal merge candidates

  - Bi-candidates, e.g. (L0_of_cand_0, L1_of_cand_1)

  - Zero-candidate with refIdx = 0, e.g. (0,0) refIdx = 0

  - Default BVs of (-w,0), (-2w,0), (0,-h), (0,-2h), (-w,-h)

  - Zero-candidates with different refIdx, e.g. (0,0) refIdx = 1, 2, …

  - Zero-candidates with refIdx = 0, e.g. (0,0) refIdx = 0

In the above, w and h are the current CU width and height.

For the BV = (BV_x, BV_y) from each Intra BC merge candidate, if it points to some overlapped area with the current CU, it is clipped as follows:

diff_x = CU_x – PU_x – BV_x – PU_Width;

diff_y = CU_y – PU_y – BV_y – PU_Height;

if( diff_x < 0 && diff_y < 0 )

{

if( diff_x >= diff_y )

{

BV_x = CU_x - PU_x - PU_Width;

}

else

{

BV_y = CU_y - PU_y - PU_Height;

}

}

Where in the above, CU_x and CU_y are the top left pixel position of the current CU; PU_x and PU_y are the top left pixel position of the current PU; PU_Width and PU_Height are the width and height of the current PU.

# Experimental results

HM-15.0+RExt-8.0+SCM-2.0 is used as the anchor. The common test conditions specified in [3] are followed. The summarized experimental results for lossy and lossless coding are shown in Table 1 and 2, respectively. More detailed experimental results could be found in the attached spreadsheets. In addition, results of an encoder improved version (fast version) of this work are also provided, where the encoder runtime is reduced. Results of fast version are shown for lossy and lossless coding in table 3 and 4 respectively.

Table 1 Experimental results for lossy coding in test 5b + default merge candidate setting

Table 2 Experimental results for lossless coding in test 5b + default merge candidate setting

Table 3 Experimental results for lossy coding in test 5b + default merge candidate setting + encoder improvement

Table 4 Experimental results for lossless coding in test 5b + default merge candidate setting+ encoder improvement

# Conclusion

This contribution proposes to add some default BVs as additional Intra BC merge candidates for merge mode. Additional BD rate reduction on top of Intra BC merge mode in CE2 Test 5b is observed.

# References

- JCTVC-R1102, “Description of Core Experiment 2 (CE2): IBC signalling and partitioning,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-S0xxx, “CE2 Test 5: Unification of intra BC and inter modes using intra_bc_flag,” Strasburg, FR, 17 Oct. – 24 Oct. 2014.

- JCTVC-R1015, “Common conditions for screen content coding tests,” Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).