| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0129 |
| --- | --- |

| Title: | On indication of IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Martin Pettersson, Jonatan Samuelsson, Andrey Norkin Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

It is proposed to signal a maximum IBC CU size to enable the possibility to avoid signaling local indications of Intra block copy usage for CUs that are larger than the maximum IBC CU size. When setting the maximum IBC CU size to 16x16 it is reported that the bitrate reduction is 0.1% for the common test conditions.

# Introduction

It has been observed that intra block copy (IBC) is mostly used for smaller CU sizes (8x8, 16x16). However, the IBC mode flag is also signaled for larger CUs (32x32, 64x64) that are seldom used for IBC.

# Proposal

It is proposed to signal a maximum IBC CU size in SPS to avoid signaling the intra_bc_flag for CUs larger than the maximum IBC CU size. The approach has been integrated into SCM-2.0. Block vector search is restricted according to the maximum IBC CU size.

It is proposed to add the following text to the HEVC screen content coding extension specification. The JCTVC-R01005-v3.doc text [1] has been used as base for the text integration.

7.3.2.2.3 Sequence parameter set scc extension syntax

| intra_block_copy_enabled_flag | u(1) |
| --- | --- |
| if(intra_block_copy_enabled_flag) |  |
| intra_bc_log2_max_cu_size | ue(v) |

7.3.8.5 Coding unit syntax

| if( intra_block_copy_enabled_flag && ( log2CbSize <= intra_bc_log2_max_cu_size)) |  |
| --- | --- |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |

7.4.3.2.3 Sequence parameter set extension syntax

intra_bc_log2_max_cu_size specifies the maximum coding unit block size at which the intra_bc_flag is present.

# Results

The results for setting the maximum IBC CU size to 16 in comparison with SCM-2.0 [2] are shown in Table 1. Encoding and decoding times are derived on a linux cluster and are unreliable.

Table 1 Results in comparison with SCM-2.0 for lossy common conditions

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0,1% | -0,1% | -0,1% |
| RGB, text & graphics with motion,720p | -0,1% | 0,0% | -0,1% |
| RGB, mixed content, 1440p | 0,0% | 0,0% | 0,0% |
| RGB, mixed content, 1080p | 0,0% | 0,0% | 0,0% |
| RGB, Animation, 720p | 0,0% | 0,0% | 0,0% |
| RGB, camera captured, 1080p | 0,0% | 0,0% | 0,0% |
| YUV, text & graphics with motion, 1080p | -0,1% | -0,1% | -0,1% |
| YUV, text & graphics with motion,720p | -0,1% | -0,1% | -0,1% |
| YUV, mixed content, 1440p | -0,1% | -0,1% | -0,2% |
| YUV, mixed content, 1080p | -0,1% | -0,1% | -0,1% |
| YUV, Animation, 720p | 0,0% | 0,0% | 0,0% |
| YUV, camera captured, 1080p | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 125% |  |  |
| Dec Time[%] | 128% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0,0% | 0,0% | 0,0% |
| RGB, text & graphics with motion,720p | 0,0% | -0,1% | -0,1% |
| RGB, mixed content, 1440p | -0,1% | 0,0% | -0,1% |
| RGB, mixed content, 1080p | 0,1% | 0,0% | 0,0% |
| RGB, Animation, 720p | 0,0% | 0,0% | -0,1% |
| RGB, camera captured, 1080p | 0,0% | 0,0% | 0,0% |
| YUV, text & graphics with motion, 1080p | 0,0% | -0,1% | 0,0% |
| YUV, text & graphics with motion,720p | -0,2% | -0,2% | -0,1% |
| YUV, mixed content, 1440p | 0,0% | 0,0% | 0,0% |
| YUV, mixed content, 1080p | -0,1% | -0,3% | -0,3% |
| YUV, Animation, 720p | 0,0% | 0,0% | 0,0% |
| YUV, camera captured, 1080p | 0,0% | 0,1% | 0,0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0,1% | 0,0% | 0,0% |
| RGB, text & graphics with motion,720p | 0,0% | -0,1% | -0,1% |
| RGB, mixed content, 1440p | -0,1% | -0,1% | -0,2% |
| RGB, mixed content, 1080p | -0,3% | 0,4% | 0,3% |
| RGB, Animation, 720p | -0,1% | -0,1% | 0,1% |
| RGB, camera captured, 1080p | 0,0% | 0,0% | 0,0% |
| YUV, text & graphics with motion, 1080p | -0,1% | 0,0% | 0,0% |
| YUV, text & graphics with motion,720p | -0,2% | 0,0% | -0,2% |
| YUV, mixed content, 1440p | -0,2% | -0,4% | 0,1% |
| YUV, mixed content, 1080p | 0,1% | 0,5% | -0,4% |
| YUV, Animation, 720p | -0,1% | -0,2% | 0,3% |
| YUV, camera captured, 1080p | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 105% |  |  |

# Conclusions

We recommend that a maximum IBC CU size is signaled in the SPS to enable the possibility to avoid signaling the IBC mode flag for large CU sizes that are seldom used for the IBC mode.

# References

[1] R. Joshi and J. Xu, ” High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 1”, JCTVC-R1005-v3, Sapporo, Japan, July. 2014,

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, Japan, July. 2014.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).