| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-S0133r1 |
| --- | --- |

| Title: | Adaptive Colour Transforms For Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krishnan, Pankaj Topiwala 3097 Cortona Dr. Melbourne, FL 32940 | Tel: Email: | +1-321-355-7376 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

Techniques for adaptive integer color transforms are usually developed for RGB 4:4:4 input data, converting the RGB data to one of several (YUV-like) integer color spaces prior to encoding/decoding. The decoded data is converted back to RGB 4:4:4, where performance quality is measured. In this proposal, the color-space transform converts prediction error in 4:4:4 chroma format into several YUV-like color spaces. Experiments shows that redundancy among three color components is further reduced.

# Color Transforms

At the 17th JCT-VC meeting, Li Zhang and Xiaoyu Xiu etc. proposed that the color-space transform is performed in the residual domain, i.e., just before invoking the cross-component prediction (CCP) and ,conventional transform during the encoding process. In their proposal, each CU adaptively enables or disables color-space transform based on a rate-distortion criterion [14]. In this proposal, the adaptive color-space transform converts prediction error in 4:4:4 chroma format into several YUV-like color spaces. Although there are many efficient color transforms, we considering only 4 color transforms: YCgCo, YFbFr1, YFbFr2 and YFbFr3. YCgCo is now well established, both in AVC, as well as anticipated in HEVC. The YFbFr1 – YfbFr3 are related variants.

## Text Description of Transforms

As a preliminary, the adaptive colour transform in the decoder is applied according to the diagram below, as described in the SCC Test Model, Figure 7.

FastVDO has designed a variety of colour transforms that are analogous to YCgCo.

In R1014, it is shown that the lifting implementation of the reversible version of YCgCo (YCgCo-R) is given by

- Forward reversible colour transform

- Inverse reversible colour transform

The FastVDO colour transforms are very similarly defined.

### YCgCo:

The forward conversion for lossless coding:

Cg = R − B;

t    = B + (Cg >> 1);

Co = G − t;

t   = Round (t + (Co >>1))	;

Cg = Round (Cg >> 1);

Co = Round (Co >> 1);

t = Round (t);

Y = Clip3_FVDO (t);

Co = Clip3_FVDO (Co + (1 << (BitDepthC – 1)));

Cg = Clip3_FVDO (Cg + (1 << (BitDepthC – 1)));

The corresponding matrix implementation for lossy coding:

1412−14120−12−1412−14

The inverse conversion for the above three equations for lossless coding should be computed as:

t   = Y – (Co – (1 << (BitDepthC – 1)));

G = Clip3_FVDO (t +2* (Co − (1 << ( BitDepthC – 1))));

B = Clip3_FVDO (t − (Cg − (1 << ( BitDepthC – 1))));

R = Clip3_FVDO (t + (Cg − (1 << ( BitDepthC – 1))));

The inverse matrix implementation for lossy coding:

11−11011−1−1

### YFbFr1:

The forward conversion:

Fr = R − B;

t    = B + (Fr >> 1);

Fb = G − t;

t   = t + ((3*Fb) >> 3);

Y = Clip3_FVDO (t);

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)));

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)));

The corresponding matrix implementation for lossy coding:

56580−8−48−416

The inverse conversion for the above three equations should be computed as:

t   = Y – (3*(Fb – (1 << (BitDepthC – 1))) >> 3);

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))));

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))));

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))));

The inverse matrix implementation for lossy coding:

84−38058−4−38

### YFbFrv2 :

The forward conversion:

Fr = R − B;

t    = B + (Fr >> 1);

Fb = G − t;

t   = t + ((5*Fb) >> 3);

Y = Clip3_FVDO (t);

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)));

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)));

The corresponding matrix implementation for lossy coding:

310380−8−48−416

The inverse conversion for the above three equations should be computed as:

t   = Y – (5*(Fb – (1 << (BitDepthC – 1))) >> 3);

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))));

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))));

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))));

The inverse matrix implementation for lossy coding:

84−58038−4−58

### YFbFrv3 :

The forward conversion:

Fr = R − B;

t    = B + (Fr >> 1);

Fb = G − t;

t   = t + ((23*Fb) >> 5);

Y = Clip3_FVDO (t);

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)));

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)));

The corresponding matrix implementation for lossy coding:

9469320−32−1632−1664

The inverse conversion for the above three equations should be computed as:

t   = Y – (23*(Fb – (1 << (BitDepthC – 1))) >> 5);

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))));

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))));

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))));

The inverse matrix implementation for lossy coding:

816−2332098−16−2332

### Macro Functions

Clip3_FVDO( MinVal, MaxVal, a)   ( ((a)<(MinVal)) ? (MinVal) : (((a)>(MaxVal)) ? (MaxVal) :(a)) )

Sign(Val) ((Val < 0) ? -1 : ((Val > 0) ? 1 : 0))

Floor(x) ((Int)(x))

Abs(x) ((x >= 0) ? x : -x)

Round(x) (Sign(x)*Floor(Abs(x) + 0.5))

# Experiments

There are two different coding modes discussed in JCTVC-R1015. The experiments were conducted on some selected categories of test sequences for the lossy mode with SCC specific encoder settings enabled in SCM-1.0 reference software.

The proposed solution achieves average {G, B, R} BD-rate reductions of {-2.7%      -2.2%      -2.3%}, {-2.5%      -3.2%      -3.1%} and {-3.2%      -3.7%      -3.4%} for AI, RA and LD respectively on top of the results shown in JCTVC-R0147 for the selected RGB sequences. For the corresponding YUV sequences BD-rate reductions are {-0.2%   -1.0%   -1.1%}, {-0.1%    -1.2%    -1.3%} and {-0.1%   -0.9%   -0.5%} for AI, RA and LD respectively.

Table 1 Coding gains compared to JCTVC-R0147

|  | All Intra |  |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p |  | -2.9% | -1.4% | -1.6% | -1.9% | -1.5% | -1.6% | -2.1% | -1.7% | -1.6% |
| RGB, text & graphics with motion,720p |  | -3.0% | -2.2% | -2.2% | -3.1% | -3.0% | -3.0% | -3.6% | -3.8% | -3.6% |
| RGB, mixed content, 1440p |  |  |  |  |  |  |  |  |  |  |
| RGB, mixed content, 1080p |  |  |  |  |  |  |  |  |  |  |
| RGB, Animation, 720p |  | -1.0% | -2.6% | -2.9% | -0.5% | -5.4% | -4.9% | -3.1% | -5.1% | -4.8% |
| RGB, camera captured, 1080p |  |  |  |  |  |  |  |  |  |  |
| YUV, text & graphics with motion, 1080p |  | -0.7% | -0.8% | -0.8% | -0.6% | -1.0% | -0.9% | -0.6% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p |  | -0.3% | -0.6% | -0.8% | -0.1% | -0.8% | -1.3% | 0.2% | -0.5% | -0.4% |
| YUV, mixed content, 1440p |  |  |  |  |  |  |  |  |  |  |
| YUV, mixed content, 1080p |  |  |  |  |  |  |  |  |  |  |
| YUV, Animation, 720p |  | 0.5% | -1.7% | -1.6% | 0.5% | -2.0% | -1.6% | 0.2% | -1.3% | -0.4% |
| YUV, camera captured, 1080p |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |

# Conclusions

Our tests indicate that a useful coding gain can be obtained by performing adaptive color transforms in the residual domain, beyond that achieved by R0147.

# Draft Text as High-Level Syntax Proposal

We propose the following high level syntax features to support adaptive color transforms:

residual_color_space_conversion_enabled_flag and cu_residual_csc_flag are inherited from the joint proposal by Qualcomm and InterDigital Communications [14].

- We propose a sequence level flag residual_color_space_conversion_enabled_flag, specifies that the residual color space conversion process may be invoked in the decoding process. residual_color_space_conversion_enabled_flag equal to 1 specifies the residual samples of the current coding unit are represented in the converted color space. cu_residual_csc_flag equal to 0 specifies that the residual samples of the current coding unit are not represented in the converted color space.  When cu_residual_csc_flag is not  present, it is inferred to be equal to 0.

- A CU level flag cu_residual_csc_flag, indicating that residual samples of the current coding unit are represented in the converted color space. cu_residual_csc_flag equal to 0 specifies that the residual samples of the current coding unit are not represented in the converted color space.  When cu_residual_csc_flag is not  present, it is inferred to be equal to 0.

- If cu_residual_csc_flag equals to 1, cu_transform_id is used to specify which color space is used. cu_transform_idx equal to 0 indicate YCoCg is used. cu_transform_idx equal to 1 indicate YFbFr1 is used. cu_transform_idx equal to 2 indicate YFbFr2 is used. cu_transform_idx equal to 3 indicate YFbFr3 is used.

# Proposed Syntax and Semantics

All the changes on top of JCTVC-R1013_v5 are highlighted in yellow.

##### Sequence parameter set range extensions syntax

| sps_range_extensions( ) { | Descriptor |
| --- | --- |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| persistent_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| residual_color_space_conversion_enabled_flag | u(1) |
| } |  |

#### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_color_space_conversion_enabled_flag&& (CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ !intra_chroma_pred_mode[ x0 ][ y0 ])) |  |
| cu_residual_csc_flag | ae(v) |
| If(cu_residula_csc_flag) |  |
| cu_transform_idx | ae(v) |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

References

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “High-performance,  low-complexity re-sampling filters for SVC”, JVT-T041, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 20th meeting, Klagenfurt, AT, Jul. 2006.

- Thierry Blu, Philippe Thevenaz, and Michael Unser, “MOMS: Maximal-order interpolation of minimal support,” IEEE Trans. On Image Processing, vol. 10, No. 7, pp. 1069-1080, Jul. 2001.

- P. P. Vaidyanathan and Bojan Vrcelj, “Biorthogonal partners and applications”, IEEE Trans. on Signal Processing, vol. 49, No. 5, pp. 1013-1028, May 2001.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Dyadic Spatial Re-sampling Filters For SVC”, JVT-V031, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Gary Sullivan, “Resampling filters for SVC Upsampling”, JVT-R066, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 18th meeting, Bangkok, Jan. 2006.

- Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

- Thomas Davies, Pankaj Topiwala, “AHG18: Adaptive Resolution Coding (ARC),” JCTVC-G264, Geneva, CH, 21-30 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, JCTVC-F618, Torino, IT, 14-22 July, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for HEVC Applications”, JCTVC-G862, Geneva, CH, 14-22 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Integer Color Transforms and Resampling Filters for HEVC Extensions: Scalability, 3DV, and Higher Chroma Coding”, JCTVC-J0127, Stockholm, SE, 10-20 July, 2012.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “AHG7: Colour Spaces and Chroma Sampling Methods For Higher Chroma Format Coding”, JCTVC-K0211, Shanghai, CN, 10-19 October, 2012.

- Takeshi Chujoh, et al, “Additional VUI and SEI for chroma sampling filter,” JCTVC-L0077r2, Geneva, CH, 14-23 January, 2013.

- Li Zhang, Jianle Chen, Joel Sole, Marta Karczewicz, Xiaoyu Xiu, Yuwen Hen and Yan Ye, “SCCE5 Test 3.2.1: In-loop color-space transform”, JCTVC-R0147, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

- Jill Boyce etc, “Draft high efficiency video coding (HEVC) version 2, combined format range extensions (RExt), scalability (SHVC), and multi-view (MV-HEVC) extensions”, JCTVC-R1013_v5, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).