| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR , 17 Oct. – 24 Oct. 2014 | Document: JCTVC-S0134 |
| --- | --- |

| Title: | Non-CE6: Simplified palette size coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Seung-Hwan Kim, Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 jzhao@sharplabs.com kimse@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This contribution proposes to change the binarization of “palette_num_signalled_entries”.  When the palette_num_signalled_entries is less than 16, there is no change to the binarization, it still uses the existing TR code. When   palette_num_signalled_entries is equal to or greater than 16, the proposed binarization uses a concatenated TR binarization of a number 16 and a fixed length code. The benefit of the proposed method is that it reduces the worst case coded bins from 31 to 20 bins, which is 35.5% reduction. It has no coding loss for both lossy and lossless coding conditions.

# Introduction

In the 18th JCT-VC meeting in Sapporo, palette coding is adopted into SCM-2.0 software and draft text. In the draft text, when palette_share_flag  is equal to 0, the palette size consists of number of entries reused from previous palette table “numPredPreviousPalette” and the number entries in the palette that are explicitly signalled for the current coding unit --- “palette_num_signalled_entries”.

As shown in Table 9-34 “Syntax elements and associated binarizations” in the SCM2.0 draft text specification, the binarization of palette_num_signalled_entries uses Truncated Rice (TR) code with cMax=31 and cRiceRaram=0. In worst case, coding palette_num_signalled_entries take the same number of bins as the maximum palette size, which is equal to 31.

Table 9-1 – Syntax elements and associated binarizations (in SCM2.0 draft text)

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |

| alette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if(palette_share_flag[ x0 ][ y0 ]){ |  |
| palette_size = previousPaletteSize |  |
| for( n = 0; n < palette_size; n++ ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ){ |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][n] |  |
| } |  |
| } |  |
| } else { |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize; i++ ) |  |
| previous_palette_entry_flag[ i ] = 0 |  |
| palette_last_group = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize && !palette_last_group && numPredPreviousPalette < max_palette_size; i++) { |  |
| ……… |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| palette_num_signalled_entries | ae(v) |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| for( i = 0; i < palette_num_signalled_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + palette_num_signalled_entries |  |
| } |  |
| …………. |  |
| } |  |

# Proposed

In this proposal, we propose to change the binarization of a palette_num_signalled_entries  to a concatenated TR code and an FLC code when palette_num_signalled_entries  is equal to or greater than a specified threshold. The goal is to reduce the number of coded bins for large palette_num_signalled_entries  numbers. When palette_num_signalled_entries  is below the threshold, its binarization is kept the same as the TR code in the current draft text.

Specifically, for max_palette size 31, the threshold is chosen as 16. When the palette_num_signalled_entries is less than 16, the palette_num_signalled_entries is binarized using the existing TR code. When the palette_num_signalled_entries is equal to or greater than 16, the number 16 is firstly binarized using TR code with cMax=16 and cRiceParam=0, (i.e. a binary code of 16 zeros), then the remaining difference between palette_num_signalled_entries and 16 is binarized using a 4-bit FLC.

At decoder side, at the first step, if the received value by TR decoding with cMax=16  is less than 16, it is the final value of palette_num_signalled_entries. If the received value equals to 16, it means the final palette_num_signalled_entries  is equal to or greater than 16. Then the difference between the final palette_num_signalled_entries   and 16 is decoded using 4-bit FLC.

With the proposed method, there is no change to the syntax table. It only changes the binarization description of palette_num_signalled_entries. Note that truncated rice (TR) code (binarization) with rice parameter equal to 0 is the same as truncated unary (TU) code (binarization).

#### [Propsoed Draft Text]

Table 9-2 – Syntax elements and associated binarizations

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | Section a.b.c.d. |  |

#### a.b.c.d. Binarization process for palette_num_signalled_entries

Inputs to this process are a request for a binarization for the syntax element palette_num_signalled_entries.

Output of this process is the binarization of the syntax element.

Depending on the value of palette_num_signalled_entries, the following applies:

- If palette_num_signalled_entries <16

  - Binaize the palette_num_signalled_entries using using TR binarization with cMax=16 and cRiceParam=0

else

  - Binaize the value 16 using using TR binarization with cMax=16 and cRiceParam=0

  - Binarize the value of palette_num_signalled_entries-16 using 4-bit FLC

[End Propsoed Draft Text]

## Considering the maximum palette size in current palette structure is 31, the existing TR binarization will take 31 bins to code palette_num_signalled_entries in the worst case. The proposed method will take 20 bins instead. This includes 16 bins to code the threshold 16 and 4 bins to code the remainder. This reduces the worst case number of bins by 11, which is 35.5% reduction. In addition, in the aspect of complexity, fixed length code is simpler than TR code. The proposed method will reduce the total number of conditional checking required for TR code, when palette_num_signalled_entries is large.

Here, the threshold 16 and the FLC length 4 are determined from the maximum palette size 31. If the maximum palette size is larger than 31, the threshold and FLC length need to be adjusted as well. For example, when the maximum palette size is 63, the threshold would be 32 and the FLC length would be 5. In this case, using existing TR binarization to code palette_num_signalled_entries would take 63 bins in the worst case. While using the proposed method, the number of bins would be 37 in the worst case. It saves 26 bins. Therefore with the proposed method, the number of bins saved is even more when the maximum pallet size is larger.

# Experimental results

The experimental conditions are performed following the screen content coding test conditions [2] and CE6 description [3] . Full results are provided in the attached Excel sheets. Table 1 and Table 2 summarize lossy and lossless coding results with respect to the SCM2.0 screen content coding anchor with for full frame Intra Block Copy.

The coding performance impact is 0.0% for both lossy and lossless cases.

Table 1: Results with respect to the SCM2.0 anchor for full frame Intra Block Copy (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot,1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & grap. with mot,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot.,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 2: Results with respect to the SCM2.0 anchor for full frame Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

In conclusion, we proposed a change to the binarization of “palette_num_signalled_entries” to reduce the worst case number of coded bins. When the palette_num_signalled_entries is less than 16, it still uses the existing TR code. When  palette_num_signalled_entries  is equal to or greater than 16, the proposed binarization uses a  TR code of 16  followed by a 4-bit FLC code. For maximum palette size 31, this method reduces the worst case coded bins from 31 bins to 20 bins. The benefit of saving will be even larger when the maximum palette size becomes larger.  The proposed method has no coding loss for both lossy and lossless test conditions. Therefore we propose to have it adopt into SCM2.0 draft text.

# Reference

[1] P. Onno, X. Xiu, Y.-W. Huang, R. Joshi, “Suggested combined software and text for run-based palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0348, 18th Meeting: Sapporo, JP, June-July 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, June-July. 2014.

[3] Y.W Huang, R. Onno, R. Cohen, V. Seregin, X. Xiu, and Z. Ma, “Description of Screen Content Core Experiment 6 (CE6): Improvements of palette mode”. JCTVC-R1106, June-July. 2014.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).