| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, Fr, 14 Oct. – 24 Oct. 2014 | Document: JCTVC-S0135 |
| --- | --- |

| Title: | CE4: Results of Test A on Search Range of Intra Line Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

In the 18th JCT-VC meeting, the Core Experiment 4 (JCTVC-R1104) was formed to study intra line copy (IntraLC) techniques. This contribution is based on JCTVC-R0132 to evaluate the coding performance of IntraLC under two different configurations (4 CTUs and full frame) of search range. The coding results are reported as follows:

- (Subtest 1) 4-CTU search range:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.1%,	(RA) -3.7%, 	(LB) -2.4%;

  - RGB, text & graphics with motion, 720p		: (AI) -3.3%, 	(RA) -2.6%, 	(LB) -0.9%;

  - RGB, mixed content, 1440p			: (AI) -2.5%, 	(RA) -1.7%, 	(LB) -1.0%;

  - RGB, mixed content, 1080p			: (AI) -3.7%, 	(RA) -3.0%, 	(LB) -1.5%;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.2%,	(RA) -3.3%, 	(LB) -2.0%;

  - YUV, text & graphics with motion, 720p 	: (AI) -3.0%, 	(RA) -2.5%, 	(LB) -0.9%;

  - YUV, mixed content, 1440p			: (AI) -2.4%, 	(RA) -1.8%, 	(LB) -0.9%;

  - YUV, mixed content, 1080p			: (AI) -3.7%, 	(RA) -2.9%, 	(LB) -1.6%;

  - Encoding time					: (AI) 119%, 	(RA) 103%, 	(LB) 102%;

  - Decoding time					: (AI) 106%, 	(RA) 102%, 	(LB) 102%.

- (Subtest 2) full-frame search range:

  - RGB, text & graphics with motion, 1080p	: (AI) -3.8%,	(RA) -2.2%,	(LB) -1.5%;

  - RGB, text & graphics with motion, 720p		: (AI) -1.3%, 	(RA) -1.1%, 	(LB) -0.7%;

  - RGB, mixed content, 1440p			: (AI) -1.5%, 	(RA) -1.1%, 	(LB) -0.6%;

  - RGB, mixed content, 1080p			: (AI) -2.2%, 	(RA) -1.6%, 	(LB) -0.6%;

  - YUV, text & graphics with motion, 1080p	: (AI) -3.7%, 	(RA) -2.0%, 	(LB) -1.3%;

  - YUV, text & graphics with motion, 720p	: (AI) -1.2%, 	(RA) -0.9%, 	(LB) -0.5%;

  - YUV, mixed content, 1440p			: (AI) -1.4%, 	(RA) -1.0%, 	(LB) -0.7%;

  - YUV, mixed content, 1080p			: (AI) -2.1%, 	(RA) -1.6%, 	(LB) -0.7%;

  - Encoding time					: (AI) 117%, 	(RA) 103%, 	(LB) 101%;

  - Decoding time					: (AI) 101%, 	(RA) 101%, 	(LB) 103%;

# Introduction

The IntraLC mode [1] divides a prediction unit into multiple 1-pixel-wide rows or columns, termed as lines in general, and performs block copying within the same search range as that of intra block copy (IntraBC). The difference between IntraBC and IntraLC is that the basic processing unit for the latter is not a block but a line. The searchable area is restricted to part of the reconstructed pixels inside the current frame and pixels within current CU are not allowed for reference. A flag is signaled at PU level to indicate whether the IntraLC mode is used or not. For each IntraLC PU, another flag is sent to indicate whether a PU is split row-wisely or column-wisely into lines (Figure 1). The block vector (BV) coding technique of IntraLC is the same as that of IntraBC that is to take a BV as reference from a BV candidate list formed by using the BVs of two spatially neighboring blocks, to be left and on the top of the current PU, and two previously-decoded BVs .

Besides, four fast algorithms are applied for speeding up the IntraLC search. First, IntraLC is evaluated only at the smallest CU level. Second, Nx2N/2NxN IntraLC modes will not be evaluated if the encoder skips the evaluation for the Nx2N/2NxN IntraBC modes. Third, the encoder skips the evaluation for the NxN IntraLC mode. Four, a fast searching process is also applied to IntraLC BV search and is described as follows:

- searching horizontally and vertically within the given search range;

- searching the pixel positions indicated by using a hash-based technique [2] with the modification that only the most significant 4 bits of each sample are involved in the calculation of hash keys.

Figure 1. An example of IntraLC with PU-level splitting.

# Experimental Results

The experiments were conducted using the HM-15.0+RExt-8.0+SCM-2.0 software [3] and the screen content common test conditions [4] to collect the BD-rate and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions. Two different settings to the search range are described as follows:

- Subset 1: 4-CTU search range:

In this subset, the search range of IntraLC is restricted within current CTU and 3 CTUs to the left which is the same as that of IntraBC (Figure 2).

Figure 2. Search range of Subset 1.

- Subset 2: full-frame search range:

In this subset, the search range of IntraLC is restricted within current CTU and left CTU which is the same as that of Nx2N/2NxN/NxN IntraBC mode (Figure 3).

Figure 3. Search range of Subset 2.

## Coding Performance

Table 1. BD-rate reduction and processing time ratio of Subset 1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.4% | -6.1% | -6.2% | -3.7% | -3.3% | -3.4% | -2.4% | -2.1% | -2.2% |
| RGB, text & graphics with motion,720p | -3.3% | -3.0% | -3.0% | -2.6% | -2.3% | -2.3% | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -2.5% | -2.3% | -2.4% | -1.7% | -1.5% | -1.6% | -1.0% | -0.8% | -0.9% |
| RGB, mixed content, 1080p | -3.7% | -3.4% | -3.4% | -3.0% | -2.4% | -2.5% | -1.5% | -1.2% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.2% | -5.7% | -5.8% | -3.3% | -2.9% | -2.9% | -2.0% | -1.7% | -1.8% |
| YUV, text & graphics with motion,720p | -3.0% | -2.7% | -2.9% | -2.5% | -2.1% | -2.2% | -0.9% | -0.7% | -0.5% |
| YUV, mixed content, 1440p | -2.4% | -2.4% | -2.4% | -1.8% | -1.4% | -1.7% | -0.9% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | -3.7% | -3.1% | -3.4% | -2.9% | -2.4% | -2.7% | -1.6% | -1.1% | -1.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 119% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  | 102% |  |  |

Table 2. BD-rate reduction and processing time ratio of Subset 1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.1% | 7.1% | 4.8% | 11.1% | 4.7% | 5.3% | 2.7% | 8.3% | 4.6% | 4.4% | 2.5% | 6.0% |
| RGB, text & graphics with motion,720p | 1.7% | 2.1% | 0.5% | 4.6% | 0.4% | 1.2% | 0.3% | 3.6% | 0.3% | 0.6% | 0.2% | 1.7% |
| RGB, mixed content, 1440p | 0.9% | 1.1% | 0.4% | 1.7% | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.4% | 7.4% | 5.2% | 11.2% | 5.0% | 5.6% | 3.3% | 8.5% | 4.9% | 4.9% | 3.1% | 6.4% |
| YUV, text & graphics with motion,720p | 1.9% | 2.5% | 0.6% | 5.8% | 0.4% | 1.5% | 0.3% | 4.6% | 0.3% | 0.8% | 0.2% | 2.2% |
| YUV, mixed content, 1440p | 1.1% | 1.2% | 0.5% | 1.9% | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.8% | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  | 103% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 103% |  |  |  | 100% |  |  |  |

Table 3. BD-rate reduction and processing time ratio of Subset 2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.8% | -3.6% | -3.6% | -2.2% | -1.9% | -2.0% | -1.5% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.1% | -1.2% | -1.1% | -0.9% | -0.9% | -0.7% | -0.6% | -0.4% |
| RGB, mixed content, 1440p | -1.5% | -1.3% | -1.4% | -1.1% | -0.9% | -1.0% | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -2.2% | -1.9% | -2.0% | -1.6% | -1.3% | -1.4% | -0.6% | 0.1% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.7% | -3.4% | -3.4% | -2.0% | -1.7% | -1.8% | -1.3% | -1.0% | -1.1% |
| YUV, text & graphics with motion,720p | -1.2% | -1.1% | -1.2% | -0.9% | -1.1% | -1.0% | -0.5% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -1.4% | -1.3% | -1.5% | -1.0% | -1.2% | -0.9% | -0.7% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -2.1% | -1.7% | -2.0% | -1.6% | -1.4% | -1.6% | -0.7% | -0.4% | -1.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 103% |  |  |

Table 4. BD-rate reduction and processing time ratio of Subset 2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.5% | 4.6% | 3.0% | 7.6% | 2.5% | 3.2% | 1.9% | 5.3% | 2.4% | 2.7% | 1.8% | 3.7% |
| RGB, text & graphics with motion,720p | 1.0% | 1.0% | 0.3% | 1.7% | 0.3% | 0.5% | 0.2% | 1.0% | 0.2% | 0.3% | 0.2% | 0.7% |
| RGB, mixed content, 1440p | 0.6% | 0.7% | 0.3% | 1.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.8% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.5% | 4.6% | 3.3% | 7.1% | 2.7% | 3.4% | 2.1% | 5.2% | 2.6% | 2.8% | 2.0% | 3.8% |
| YUV, text & graphics with motion,720p | 1.1% | 1.1% | 0.4% | 1.8% | 0.3% | 0.6% | 0.2% | 1.3% | 0.2% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.7% | 0.7% | 0.3% | 1.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  | 104% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  | 100% |  |  |  |

## Memory Access Bandwidth

Table 5. p-value of inter, IntraBC, and IntraLC mode.

|  | Configurations |  |  |  |  |  |  | Per-pixel Memory Access Bandwidth (P) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prediction Mode | Requested Block Size |  | Num. of Samples | IF Length | Bi Pred? | Memory Access Pattern | m= | 4 | 8 | 4 | 8 | 4 |
|  | M | N | MxN | L | Y or N |  | n= | 1 | 1 | 2 | 2 | 4 |
| 8x8, Bi-prediction | 8 | 8 | 64 | 8 | Y |  |  | 9.375 | 11.25 | 10 | 12 | 12.5 |
| 4x4, IntraBC | 4 | 4 | 16 | 1 | N |  |  | 2 | 4 | 3 | 6 | 4 |
| 1x4, IntraLC | 1 | 4 | 4 | 1 | N |  |  | 4 | 8 | 6 | 12 | 8 |

Table 6. Average memory access bandwidth of Subset 1.

| 4 CTUs | All Intra (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 112.2% | 112.2% | 114.0% | 114.0% | 123.3% | 131.3% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 105.9% | 105.9% | 106.8% | 106.8% | 111.6% | 115.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 105.7% | 105.7% | 106.7% | 106.7% | 111.6% | 115.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 108.9% | 108.9% | 110.4% | 110.4% | 118.0% | 123.8% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.5% | 100.5% | 100.6% | 100.6% | 100.9% | 101.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 111.1% | 111.1% | 112.6% | 112.6% | 120.4% | 127.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 105.1% | 105.1% | 105.9% | 105.9% | 109.8% | 113.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 104.9% | 104.9% | 105.7% | 105.7% | 110.0% | 113.8% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 108.0% | 108.0% | 109.4% | 109.4% | 116.1% | 121.8% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.8% | 101.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| 4 CTUs | Random Access (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.8% | 100.8% | 100.9% | 100.9% | 101.6% | 102.6% | 100.1% | 100.1% |
| RGB, text & graphics with motion,720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.4% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.2% | 100.5% | 99.9% | 99.9% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.6% | 100.6% | 100.7% | 100.7% | 101.3% | 102.1% | 100.1% | 100.1% |
| YUV, text & graphics with motion,720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.1% | 100.1% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| 4 CTUs | Low Delay B (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.8% | 100.8% | 100.9% | 100.9% | 101.7% | 102.7% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.3% | 100.3% | 100.1% | 100.1% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.6% | 100.6% | 100.7% | 100.7% | 101.2% | 102.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 99.9% | 99.9% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 99.9% | 99.9% |

| 4 CTUs | All Intra (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 116.1% | 116.1% | 118.2% | 118.2% | 129.2% | 138.8% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 110.2% | 110.2% | 111.6% | 111.6% | 119.0% | 125.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 110.2% | 110.2% | 111.7% | 111.7% | 118.4% | 122.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 112.4% | 112.4% | 114.2% | 114.2% | 121.7% | 125.3% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.7% | 100.7% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 115.5% | 115.5% | 117.5% | 117.5% | 127.6% | 136.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 109.8% | 109.8% | 111.2% | 111.2% | 118.1% | 124.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 109.5% | 109.5% | 110.9% | 110.9% | 116.9% | 120.4% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 112.4% | 112.4% | 114.1% | 114.1% | 121.5% | 124.9% | 100.0% | 100.0% |
| YUV, Animation, 720p | 101.4% | 101.4% | 101.6% | 101.6% | 102.2% | 102.4% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| 4 CTUs | Random Access (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 103.1% | 103.1% | 103.5% | 103.5% | 105.8% | 108.3% | 99.9% | 99.9% |
| RGB, text & graphics with motion,720p | 100.3% | 100.3% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.2% | 100.2% | 100.2% | 100.2% | 100.5% | 100.8% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 103.3% | 103.3% | 103.7% | 103.7% | 106.2% | 108.7% | 99.7% | 99.8% |
| YUV, text & graphics with motion,720p | 100.3% | 100.3% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.4% | 100.8% | 99.9% | 99.9% |
| YUV, mixed content, 1080p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.0% | 100.0% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| 4 CTUs | Low Delay B (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 103.9% | 103.9% | 104.5% | 104.5% | 107.1% | 109.5% | 99.7% | 99.8% |
| RGB, text & graphics with motion,720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.6% | 101.0% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 103.7% | 103.7% | 104.2% | 104.2% | 106.7% | 109.1% | 99.7% | 99.7% |
| YUV, text & graphics with motion,720p | 100.3% | 100.3% | 100.3% | 100.3% | 100.5% | 100.9% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.8% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 99.9% | 99.9% | 100.0% | 100.0% | 99.9% | 99.9% |

Table 7. Average memory access bandwidth of Subset 2.

| Full Frame (FF) | All Intra (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 105.8% | 105.8% | 106.6% | 106.6% | 111.1% | 115.2% | 99.6% | 99.6% |
| RGB, text & graphics with motion,720p | 102.9% | 102.9% | 103.3% | 103.3% | 105.6% | 107.6% | 99.8% | 99.8% |
| RGB, mixed content, 1440p | 103.5% | 103.5% | 104.1% | 104.1% | 107.3% | 109.8% | 99.8% | 99.7% |
| RGB, mixed content, 1080p | 104.9% | 104.9% | 105.7% | 105.7% | 110.1% | 113.4% | 99.6% | 99.5% |
| RGB, Animation, 720p | 100.5% | 100.5% | 100.6% | 100.6% | 100.9% | 101.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 105.0% | 105.0% | 105.7% | 105.7% | 109.3% | 112.4% | 99.6% | 99.5% |
| YUV, text & graphics with motion,720p | 102.5% | 102.5% | 102.8% | 102.8% | 104.7% | 106.4% | 99.9% | 99.9% |
| YUV, mixed content, 1440p | 102.8% | 102.8% | 103.3% | 103.3% | 105.9% | 108.2% | 99.8% | 99.7% |
| YUV, mixed content, 1080p | 104.4% | 104.4% | 105.1% | 105.1% | 108.9% | 112.0% | 99.6% | 99.5% |
| YUV, Animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.8% | 101.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| Full Frame (FF) | Random Access (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.5% | 100.5% | 100.9% | 101.5% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.4% | 100.4% | 100.7% | 101.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.1% | 100.1% |
| YUV, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| Full Frame (FF) | Low Delay B (Lossy) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 100.5% | 100.5% | 100.6% | 100.6% | 101.0% | 101.7% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.2% | 100.1% | 100.1% |
| RGB, mixed content, 1080p | 99.9% | 99.9% | 99.9% | 99.9% | 100.0% | 100.0% | 99.9% | 99.9% |
| RGB, Animation, 720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.2% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.1% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 100.4% | 100.4% | 100.4% | 100.4% | 100.8% | 101.2% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% | 99.9% |
| YUV, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.4% | 100.4% | 100.4% | 100.4% | 100.4% | 100.4% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 99.9% |

| Full Frame (FF) | All Intra (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 108.1% | 108.1% | 109.1% | 109.1% | 114.9% | 120.2% | 99.4% | 99.4% |
| RGB, text & graphics with motion,720p | 105.9% | 105.9% | 106.7% | 106.7% | 110.8% | 114.6% | 99.8% | 99.8% |
| RGB, mixed content, 1440p | 106.4% | 106.4% | 107.3% | 107.3% | 111.9% | 114.9% | 99.7% | 99.7% |
| RGB, mixed content, 1080p | 107.4% | 107.4% | 108.6% | 108.6% | 113.5% | 115.9% | 99.6% | 99.6% |
| RGB, Animation, 720p | 100.5% | 100.5% | 100.5% | 100.5% | 100.7% | 100.7% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 107.7% | 107.7% | 108.6% | 108.6% | 113.9% | 118.5% | 99.5% | 99.5% |
| YUV, text & graphics with motion,720p | 105.8% | 105.8% | 106.6% | 106.6% | 110.6% | 114.2% | 99.8% | 99.7% |
| YUV, mixed content, 1440p | 105.9% | 105.9% | 106.8% | 106.8% | 111.0% | 113.5% | 99.6% | 99.5% |
| YUV, mixed content, 1080p | 107.1% | 107.1% | 108.2% | 108.2% | 112.9% | 115.2% | 99.5% | 99.5% |
| YUV, Animation, 720p | 101.4% | 101.4% | 101.6% | 101.6% | 102.2% | 102.3% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| Full Frame (FF) | Random Access (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 101.7% | 101.7% | 101.9% | 101.9% | 103.2% | 104.7% | 99.8% | 99.8% |
| RGB, text & graphics with motion,720p | 100.2% | 100.2% | 100.3% | 100.3% | 100.4% | 100.8% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 101.9% | 101.9% | 102.2% | 102.2% | 103.6% | 105.1% | 99.7% | 99.7% |
| YUV, text & graphics with motion,720p | 100.2% | 100.2% | 100.3% | 100.3% | 100.4% | 100.8% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.2% | 100.2% | 100.2% | 100.2% | 100.3% | 100.6% | 100.0% | 100.0% |
| YUV, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |

| Full Frame (FF) | Low Delay B (Lossless) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8-8 | 32-64 | 32-128 | 64-128 | 64-256 | 64-512 | 64-256 Cache | 64-512 Cache |
| RGB, text & graphics with motion, 1080p | 102.2% | 102.2% | 102.4% | 102.4% | 104.0% | 105.5% | 99.7% | 99.7% |
| RGB, text & graphics with motion,720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.7% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 100.1% | 100.1% | 100.2% | 100.2% | 100.3% | 100.5% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 100.1% | 100.1% | 100.1% | 100.1% | 100.2% | 100.3% | 100.0% | 100.0% |
| RGB, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 99.9% | 99.9% | 99.9% |
| YUV, text & graphics with motion, 1080p | 102.1% | 102.1% | 102.4% | 102.4% | 103.9% | 105.4% | 99.6% | 99.6% |
| YUV, text & graphics with motion,720p | 100.2% | 100.2% | 100.2% | 100.2% | 100.4% | 100.7% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 100.1% | 100.1% | 100.1% | 100.1% | 100.3% | 100.5% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.2% | 100.0% | 99.9% |
| YUV, Animation, 720p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 100.1% | 100.1% |

## CABAC Throughput

Table 8. CABAC throughput of inter, IntraBC, and IntraLC mode.

| Prediction Mode (Note: CU Size = 8x8) | 4x8, Uni Prediction (SCM-2.0) | 4x4, IntraBC (SCM-2.0) | 1x4, IntraLC |
| --- | --- | --- | --- |
| Number of Context-coded Bins | 488 | 484 | 528 |
| Bins per Pixel | 7.63 | 7.56 | 8.25 |

# Conclusion

In this contribution, the coding performance of IntraLC under two different configurations (4 CTUs and full frame) of the search range is tested. With 4-CTU search range, the BD-rate saving is up to 6.2% in the All-Intra configuration whereas 3.8% coding gain is observed with full-frame search range. According to the interesting gains, we would recommend the group to consider further studying this mode in a CE.

# References

- C. C. Chen, X. Xu, L. Zhang, and T. Lin, “Description of Core Experiment 4 (CE4): Intra Line Copy,” JCTVC-R1104, Sapporo, Jul. 2014.

- B. Li, J. Xu, F Wu, X. Guo, and G. J. Sullivan, “Description of screen content coding technology proposal by Microsoft,” JCTVC-Q0035, Valencia, Mar. 2014.

- JCT-VC, “HM-15.0+RExt-8.0+SCM-2.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-15.0+RExt-8.0+SCM-2.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, Jul. 2014.

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).