| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 14–24 Oct. 2014 | Document: JCTVC-S0136 |
| --- | --- |

| Title: | Non-CE4: On CABAC Throughput of Intra Line Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

In CE4 Test A (JCTVC-S0135), the Intra Line Copy (IntraLC) mode is studied to investigate the benefit in comparison to restricted and full-frame Intra Block Copy (IntraBC). As JCTVC-S0135 reported, IntraLC requires 528 context-coded bins in its worst-case scenario, with 40 bins and 44 bins more than that for Inter modes (488 bins) and IntraBC (484 bins), respectively. In this contribution, three modifications are applied to IntraLC for reducing the number of context-coded bins as follows: (a) bypass-coding the row_splitting_flag; (b) taking the most-recently decoded BV for BV prediction except the first line of each PU; (c) removing RDPCM for IntraLC. The experimental results show that the number of context-coded bins for IntraLC is reduced to 488 bins. When using the 4-CTU configuration, the average BD-rate reductions of (AI)0.3%/(RA)0.1%/(LB)0.0% and (AI)0.5%/(RA)0.1%/(LB)0.1% relative to CE4 Test A are achieved respectively for RGB and YUV TGM 1080p sequences in lossy coding. When using the full-frame configuration, the average BD-rate reductions of (AI)0.6%/(RA)0.2%/(LB)0.0% and (AI)0.7%/(RA)0.2%/(LB)0.0% relative to CE4 Test A are achieved respectively for RGB and YUV TGM 1080p sequences in lossy coding. The BD-rate reductions relative to the SCM-2.0 anchor in lossy coding are summarized as follows:

- 4-CTU configuration:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.7,	(RA) -3.8, 	(LB) -2.4;

  - RGB, text & graphics with motion, 720p		: (AI) -3.3, 	(RA) -2.6, 	(LB) -1.1;

  - RGB, mixed content, 1440p			: (AI) -2.6, 	(RA) -1.8, 	(LB) -1.0;

  - RGB, mixed content, 1080p			: (AI) -3.9, 	(RA) -3.0, 	(LB) -1.4;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.7,	(RA) -3.4, 	(LB) -2.1;

  - YUV, text & graphics with motion, 720p 	: (AI) -3.2, 	(RA) -2.5, 	(LB) -0.9;

  - YUV, mixed content, 1440p			: (AI) -2.7, 	(RA) -1.9, 	(LB) -0.9;

  - YUV, mixed content, 1080p			: (AI) -4.0, 	(RA) -2.9, 	(LB) -1.6;

  - Encoding time					: (AI) 120%, 	(RA) 104%, 	(LB) 102%;

  - Decoding time					: (AI) 103%, 	(RA) 102%, 	(LB) 103%;

- full-frame configuration:

  - RGB, text & graphics with motion, 1080p	: (AI) -4.4;	(RA) -2.4, 	(LB) -1.5;

  - RGB, text & graphics with motion, 720p		: (AI) -1.5; 	(RA) -1.2, 	(LB) -0.5;

  - RGB, mixed content, 1440p			: (AI) -1.7; 	(RA) -1.1, 	(LB) -0.6;

  - RGB, mixed content, 1080p			: (AI) -2.5; 	(RA) -1.9, 	(LB) -0.7;

  - YUV, text & graphics with motion, 1080p	: (AI) -4.4;	(RA) -2.2, 	(LB) -1.3;

  - YUV, text & graphics with motion, 720p 	: (AI) -1.5; 	(RA) -1.1, 	(LB) -0.7;

  - YUV, mixed content, 1440p			: (AI) -1.6; 	(RA) -1.1, 	(LB) -0.6;

  - YUV, mixed content, 1080p			: (AI) -2.5; 	(RA) -1.8, 	(LB) -0.7;

  - Encoding time					: (AI) 128%, 	(RA) 107%, 	(LB) 108%;

  - Decoding time					: (AI) 102%, 	(RA) 104%, 	(LB) 104%;

# Introduction

The following three modifications are applied to CE4 Test A to reduce the number of context-coded bins required for IntraLC.

- The flag, row_splitting_flag, which is used to indicate whether a PU is row-wise or column-wise splitting into lines is bypass-coded.

- For the first line in a PU, BV prediction is the same as that of IntraBC mode; for the rest, the most-recently decoded BV of each line is used as BVp;

- RDPCM is disabled when the top-left sample within a TU is coded by using the IntraLC mode.

# Experimental Results

The experiments were conducted using the HM-15.0+RExt-8.0+SCM-2.0 software [1] and the screen content common test conditions [2] to collect the BD-rate and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions.

## Coding Performance

Table 1. 4-CTU configuration in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.7% | -6.3% | -6.5% | -3.8% | -3.4% | -3.5% | -2.4% | -2.1% | -2.2% |
| RGB, text & graphics with motion,720p | -3.3% | -3.0% | -3.0% | -2.6% | -2.2% | -2.3% | -1.1% | -0.9% | -0.9% |
| RGB, mixed content, 1440p | -2.6% | -2.4% | -2.5% | -1.8% | -1.5% | -1.6% | -1.0% | -0.5% | -0.9% |
| RGB, mixed content, 1080p | -3.9% | -3.4% | -3.6% | -3.0% | -2.4% | -2.5% | -1.4% | -1.2% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -6.7% | -6.2% | -6.3% | -3.4% | -2.9% | -3.0% | -2.1% | -1.7% | -1.8% |
| YUV, text & graphics with motion,720p | -3.2% | -2.9% | -3.0% | -2.5% | -2.2% | -2.3% | -0.9% | -0.8% | -0.4% |
| YUV, mixed content, 1440p | -2.7% | -2.5% | -2.6% | -1.9% | -1.8% | -1.8% | -0.9% | -0.7% | -0.6% |
| YUV, mixed content, 1080p | -4.0% | -3.4% | -3.6% | -2.9% | -2.5% | -2.7% | -1.6% | -0.6% | -0.7% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 120% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 103% |  |  |

Table 2. 4-CTU configuration in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.3% | 7.2% | 5.0% | 11.0% | 4.9% | 5.3% | 2.8% | 8.2% | 4.8% | 4.5% | 2.6% | 5.9% |
| RGB, text & graphics with motion,720p | 1.7% | 2.1% | 0.4% | 4.7% | 0.4% | 1.2% | 0.3% | 3.7% | 0.2% | 0.6% | 0.2% | 1.7% |
| RGB, mixed content, 1440p | 0.8% | 1.0% | 0.4% | 1.5% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.8% | 7.8% | 5.5% | 11.4% | 5.2% | 5.8% | 3.3% | 8.6% | 5.1% | 5.0% | 3.1% | 6.4% |
| YUV, text & graphics with motion,720p | 2.0% | 2.6% | 0.6% | 6.2% | 0.4% | 1.5% | 0.3% | 4.9% | 0.3% | 0.8% | 0.2% | 2.2% |
| YUV, mixed content, 1440p | 1.0% | 1.1% | 0.5% | 1.8% | 0.2% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 1.8% | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 118% |  |  |  | 104% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  | 100% |  |  |  |

Table 3. Full-frame configuration in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.4% | -4.0% | -4.2% | -2.4% | -2.1% | -2.2% | -1.5% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -1.5% | -1.3% | -1.4% | -1.2% | -0.9% | -1.1% | -0.5% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -1.7% | -1.5% | -1.6% | -1.1% | -0.9% | -1.0% | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -2.5% | -2.1% | -2.4% | -1.9% | -1.5% | -1.7% | -0.7% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.4% | -4.0% | -4.1% | -2.2% | -1.8% | -1.9% | -1.3% | -1.1% | -1.1% |
| YUV, text & graphics with motion,720p | -1.5% | -1.3% | -1.6% | -1.1% | -1.0% | -1.0% | -0.7% | -0.2% | -0.5% |
| YUV, mixed content, 1440p | -1.6% | -1.6% | -1.7% | -1.1% | -0.9% | -0.9% | -0.6% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -2.5% | -2.1% | -2.4% | -1.8% | -1.3% | -1.7% | -0.7% | 0.8% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  | 104% |  |  |

Table 4. Full-frame configuration in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.8% | 4.9% | 3.4% | 7.8% | 2.8% | 3.4% | 1.9% | 5.5% | 2.7% | 2.8% | 1.8% | 3.7% |
| RGB, text & graphics with motion,720p | 1.0% | 1.0% | 0.3% | 1.6% | 0.2% | 0.5% | 0.2% | 1.1% | 0.2% | 0.3% | 0.2% | 0.7% |
| RGB, mixed content, 1440p | 0.5% | 0.6% | 0.3% | 1.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.8% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.0% | 5.1% | 3.6% | 7.6% | 3.0% | 3.6% | 2.2% | 5.4% | 2.9% | 3.0% | 2.0% | 3.9% |
| YUV, text & graphics with motion,720p | 1.2% | 1.2% | 0.4% | 1.9% | 0.3% | 0.6% | 0.2% | 1.6% | 0.2% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.6% | 0.7% | 0.3% | 1.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  | 103% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  | 100% |  |  |  |

## CABAC Throughput

Table 5. CABAC throughput of inter, IntraBC, and IntraLC mode.

| Prediction Mode (Note: CU Size = 8x8) | 4x8, Uni-Prediction (SCM-2.0) | 4x4, IntraBC (SCM-2.0) | 1x4, IntraLC (Test A) | 1x4, IntraLC (S0136) |
| --- | --- | --- | --- | --- |
| Number of Context-coded Bins | 488 | 484 | 528 | 488 |
| Bins per Pixel | 7.63 | 7.56 | 8.25 | 7.63 |

# Conclusion

In this contribution, the CABAC throughput of IntraLC is reduced to the same level as that of IntraBC and Inter modes. Only 4 additional context-coded bins are required in Intra frames and no impact is observed in Inter frames. As compared with CE4 Test A, this contribution brings additional 0.6% gain in BD-rate saving. According to the observations, we would recommend the committee to consider further studying this mode in a CE.

# References

- JCT-VC, “HM-15.0+RExt-8.0+SCM-2.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-15.0+RExt-8.0+SCM-2.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, Jul. 2014.

# Appendix: Text

The cyan markers indicate the necessary changes to the text of CE4 Test A.

7.3.8.6	Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| … |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { /* Intra BC*/ |  |
| intra_lc_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_lc_flag[ x0 ][ y0 ] ) |  |
| row_splitting_flag[ x0 ][ y0 ] | ae(v) |
| for( i = 0; i < (intra_lc_flag[ x0 ][ y0 ] ? ( row_splitting_flag[ x0 ][ y0 ] ? nPbH : nPbW ) : 1 ); i++ ) { |  |
| bvd_coding( x0, y0, 2 ) |  |
| if ( i == 0) |  |
| bvp_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |

7.3.8.13	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ ( intra_bc_flag[ x0 ][ y0 ] & !intra_lc_flag[x0][y0] ) ) && explicit_rdpcm_enabled_flag && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| explicit_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| … |  |

7.4.9.7	Prediction unit semantics

intra_lc_flag[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is coded in intra line copying mode. intra_lc_flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is coded in intra block copying mode. When not present, the value of intra_lc_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

row_splitting_flag[ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is row-wise splitting. row_splitting_flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is column-wise splitting. When not present, the value of row_splitting_flag is inferred to be equal to 0.

7.4.9.10	Motion vector difference semantics

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of −128 to 128, inclusive [Ed. (RJ): Please check the range]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. When the considered prediction block is coded in intra line copying mode, the array indices i, j specify the location (i, j) of the top-left luma sample of the considered block relative to the top-left luma sample of the prediction block. Otherwise, i and j are inferred to be equal to 0.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise, if refList is equal to 1, MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 + i ][ y0 + j ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1 [Ed. (RJ): Please check this part.]

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).