| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 14–24 Oct. 2014 | Document: JCTVC-S0137 |
| --- | --- |

| Title: | Non-CE4: Intra Line Copy with Extended Full-frame Search for Test A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

In CE4 Test A (JCTVC-S0135), the intra line copy (IntraLC) mode was tested under full-frame configuration for search range. The search range of IntraLC and 4x4/4x8/8x4 IntraBC are both restricted to 2 CTUs (the current plus 1 left CTU). In this contribution, the full-frame search ranges were evaluated for IntraLC and 4x4/4x8/8x4 IntraBC. Three tests are conducted as follows: (Subset 1) 2-CTU search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC; (Subset 2) full-frame search for 8x4/4x8/4x4 IntraBC and no IntraLC; (Subset 3) full-frame search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC . Experimental results are reported as follows:

- (Subtest 1) full-frame search of IntraLC vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -10.1,	(RA) -5.4, 	(LB) -3.6;

  - RGB, text & graphics with motion, 720p		: (AI) -4.6, 	(RA) -3.2, 	(LB) -1.6;

  - RGB, mixed content, 1440p			: (AI) -4.0, 	(RA) -2.5, 	(LB) -1.2;

  - RGB, mixed content, 1080p			: (AI) -7.0, 	(RA) -4.9, 	(LB) -2.1;

  - YUV, text & graphics with motion, 1080p	: (AI) -9.9,	(RA) -4.7, 	(LB) -2.9;

  - YUV, text & graphics with motion, 720p 	: (AI) -4.5, 	(RA) -3.2, 	(LB) -1.5;

  - YUV, mixed content, 1440p			: (AI) -4.1, 	(RA) -2.6, 	(LB) -1.4;

  - YUV, mixed content, 1080p			: (AI) -7.1, 	(RA) -5.3, 	(LB) -2.0;

  - Encoding time					: (AI) 128%, 	(RA) 109%, 	(LB) 108%;

  - Decoding time					: (AI) 102%, 	(RA) 102%, 	(LB) 103%;

- (Subtest 2) full-frame search of 8x4/4x8/4x4 IntraBC vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.0,	(RA) -3.1, 	(LB) -2.3;

  - RGB, text & graphics with motion, 720p		: (AI) -3.5, 	(RA) -2.4, 	(LB) -1.2;

  - RGB, mixed content, 1440p			: (AI) -2.6, 	(RA) -1.7, 	(LB) -1.0;

  - RGB, mixed content, 1080p			: (AI) -4.7, 	(RA) -3.3, 	(LB) -1.6;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.1,	(RA) -2.8, 	(LB) -1.8;

  - YUV, text & graphics with motion, 720p 	: (AI) -3.6, 	(RA) -2.5, 	(LB) -1.3;

  - YUV, mixed content, 1440p			: (AI) -2.8, 	(RA) -1.5, 	(LB) -0.8;

  - YUV, mixed content, 1080p			: (AI) -4.9, 	(RA) -3.4, 	(LB) -1.2;

  - Encoding time					: (AI) 114%, 	(RA) 108%, 	(LB) 107%;

  - Decoding time					: (AI) 100%, 	(RA) 101%, 	(LB) 101%;

- (Subtest 3) full-frame search of IntraLC on top of Subset 2 vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -12.9,	(RA) -7.0, 	(LB) -4.8;

  - RGB, text & graphics with motion, 720p		: (AI) -6.2, 	(RA) -4.3, 	(LB) -2.3;

  - RGB, mixed content, 1440p			: (AI) -5.0, 	(RA) -3.2, 	(LB) -1.7;

  - RGB, mixed content, 1080p			: (AI) -8.2, 	(RA) -5.7, 	(LB) -2.5;

  - YUV, text & graphics with motion, 1080p	: (AI) -13.0,	(RA) -6.3, 	(LB) -4.0;

  - YUV, text & graphics with motion, 720p 	: (AI) -6.3, 	(RA) -4.5, 	(LB) -2.2;

  - YUV, mixed content, 1440p			: (AI) -5.2, 	(RA) -3.3, 	(LB) -1.5;

  - YUV, mixed content, 1080p			: (AI) -8.5, 	(RA) -6.2, 	(LB) -2.6;

  - Encoding time					: (AI) 143%, 	(RA) 116%, 	(LB) 115%;

  - Decoding time					: (AI) 102%, 	(RA) 102%, 	(LB) 103%;

- (Subtest 3) full-frame search of IntraLC on top of Subset 2 vs. Subset 2:

  - RGB, text & graphics with motion, 1080p	: (AI) -7.3,	(RA) -4.0, 	(LB) -2.6;

  - RGB, text & graphics with motion, 720p		: (AI) -2.9, 	(RA) -2.0, 	(LB) -1.1;

  - RGB, mixed content, 1440p			: (AI) -2.5, 	(RA) -1.6, 	(LB) -0.7;

  - RGB, mixed content, 1080p			: (AI) -3.6, 	(RA) -2.5, 	(LB) -0.9;

  - YUV, text & graphics with motion, 1080p	: (AI) -7.3,	(RA) -3.6, 	(LB) -2.2;

  - YUV, text & graphics with motion, 720p 	: (AI) -2.9, 	(RA) -2.1, 	(LB) -0.9;

  - YUV, mixed content, 1440p			: (AI) -2.5, 	(RA) -1.8, 	(LB) -0.8;

  - YUV, mixed content, 1080p			: (AI) -3.8, 	(RA) -2.9, 	(LB) -1.4;

  - Encoding time					: (AI) 125%, 	(RA) 107%, 	(LB) 108%;

  - Decoding time					: (AI) 103%, 	(RA) 102%, 	(LB) 102%;

# Introduction

The search range of IntraLC under full-frame configuration is restricted within the current and left CTU, as shown in Figure 1. In this contribution, three subtests are evaluated to the coding performance of full-frame IntraLC and full-frame search for 8x4/4x8/4x4 IntraBC blocks. Besides, the search range of IntraLC is extended to full frame under full frame configuration (Error: Reference source not found).

Figure 1. The search range of IntraLC under full-frame configuration described in CE4 Test A.

# Experimental Results

The experiments were conducted based on the JCTVC-S0136 software and the screen content common test conditions [1] to collect the BD-rate and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions.

In SCM-2.0 software, the full-frame search is only applied to 16x16 and 8x8 IntraBC PUs while 2-CTU search is applied to 8x4/4x8/4x4 IntraBC PUs. In this contribution, three subtests will be conducted to evaluate the performance-complexity trade-offs between full-frame and 4-CTU search ranges for 8x4/4x8/4x4 IntraBC and IntraLC. All the subsets are described as follows:

- Subset 1: 2-CTU search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC;

- Subset 2: full-frame search for 8x4/4x8/4x4 IntraBC and no IntraLC;

- Subset 3: full-frame search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC .

Table 1. Results of Subset 1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -10.1% | -9.7% | -9.8% | -5.4% | -4.9% | -5.1% | -3.6% | -3.1% | -3.2% |
| RGB, text & graphics with motion,720p | -4.6% | -4.1% | -4.2% | -3.2% | -2.8% | -2.8% | -1.6% | -1.2% | -1.2% |
| RGB, mixed content, 1440p | -4.0% | -3.6% | -3.8% | -2.5% | -2.2% | -2.3% | -1.2% | -0.7% | -1.1% |
| RGB, mixed content, 1080p | -7.0% | -6.5% | -6.7% | -4.9% | -4.2% | -4.5% | -2.1% | -0.8% | -1.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -9.9% | -9.4% | -9.5% | -4.7% | -4.3% | -4.4% | -2.9% | -2.5% | -2.6% |
| YUV, text & graphics with motion,720p | -4.5% | -4.1% | -4.4% | -3.2% | -2.9% | -3.2% | -1.5% | -1.0% | -1.3% |
| YUV, mixed content, 1440p | -4.1% | -3.7% | -3.8% | -2.6% | -2.2% | -2.1% | -1.4% | -0.6% | -1.2% |
| YUV, mixed content, 1080p | -7.1% | -6.5% | -7.1% | -5.3% | -4.5% | -4.9% | -2.0% | -0.3% | -1.9% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.5% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  | 109% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 103% |  |  |

Table 2. Results of Subset 1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.1% | 11.0% | 6.4% | 16.0% | 8.5% | 8.0% | 3.7% | 11.1% | 8.3% | 6.8% | 3.5% | 9.0% |
| RGB, text & graphics with motion,720p | 2.4% | 2.4% | 0.5% | 4.3% | 0.5% | 1.2% | 0.2% | 3.5% | 0.4% | 0.8% | 0.2% | 2.3% |
| RGB, mixed content, 1440p | 2.2% | 2.2% | 2.2% | 2.2% | 0.4% | 0.4% | 0.2% | 0.7% | 0.2% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 2.8% | 2.8% | 2.8% | 2.8% | 0.5% | 0.5% | 0.5% | 0.5% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.8% | 11.8% | 7.4% | 16.6% | 8.8% | 8.6% | 4.7% | 11.5% | 8.6% | 7.4% | 4.4% | 9.3% |
| YUV, text & graphics with motion,720p | 2.6% | 2.9% | 0.5% | 5.4% | 0.5% | 1.5% | 0.3% | 4.6% | 0.3% | 1.0% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 2.2% | 2.3% | 1.8% | 2.7% | 0.4% | 0.4% | 0.2% | 0.7% | 0.3% | 0.3% | 0.1% | 0.5% |
| YUV, mixed content, 1080p | 3.3% | 3.3% | 3.3% | 3.3% | 0.6% | 0.6% | 0.6% | 0.6% | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  |  | 104% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |

Table 3. Results of Subset 2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.0% | -5.9% | -5.9% | -3.1% | -2.9% | -3.0% | -2.3% | -2.0% | -2.1% |
| RGB, text & graphics with motion,720p | -3.5% | -3.4% | -3.3% | -2.4% | -2.3% | -2.1% | -1.2% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -2.6% | -2.2% | -2.3% | -1.7% | -1.3% | -1.4% | -1.0% | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -4.7% | -4.6% | -4.4% | -3.3% | -3.0% | -3.1% | -1.6% | -1.0% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.1% | -6.0% | -6.1% | -2.8% | -2.7% | -2.7% | -1.8% | -1.7% | -1.6% |
| YUV, text & graphics with motion,720p | -3.6% | -3.7% | -3.6% | -2.5% | -2.6% | -2.4% | -1.3% | -1.0% | -1.3% |
| YUV, mixed content, 1440p | -2.8% | -2.4% | -2.6% | -1.5% | -1.4% | -1.3% | -0.8% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -4.9% | -4.6% | -4.8% | -3.4% | -3.1% | -3.1% | -1.2% | -0.7% | -1.4% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 114% |  |  | 108% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |

Table 4. Results of Subset 2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.7% | 5.4% | 3.1% | 6.7% | 5.4% | 4.1% | 2.1% | 5.9% | 5.4% | 3.6% | 1.9% | 5.9% |
| RGB, text & graphics with motion,720p | 1.2% | 1.3% | 0.2% | 2.5% | 0.2% | 0.7% | 0.1% | 2.0% | 0.2% | 0.4% | 0.1% | 1.1% |
| RGB, mixed content, 1440p | 0.7% | 0.8% | 0.4% | 1.1% | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.8% | 5.7% | 3.4% | 6.9% | 5.2% | 4.1% | 2.4% | 5.7% | 5.2% | 3.6% | 2.3% | 5.7% |
| YUV, text & graphics with motion,720p | 1.4% | 1.7% | 0.2% | 3.5% | 0.3% | 0.8% | 0.1% | 2.8% | 0.2% | 0.5% | 0.1% | 1.4% |
| YUV, mixed content, 1440p | 0.9% | 1.0% | 0.5% | 1.5% | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 2.0% | 2.0% | 2.0% | 2.0% | 0.4% | 0.4% | 0.4% | 0.4% | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 119% |  |  |  | 105% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |

Table 5. Results of Subset 3 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.3% | -7.1% | -7.1% | -4.0% | -3.7% | -3.7% | -2.6% | -2.4% | -2.4% |
| RGB, text & graphics with motion,720p | -2.9% | -2.5% | -2.6% | -2.0% | -1.7% | -1.9% | -1.1% | -1.1% | -1.0% |
| RGB, mixed content, 1440p | -2.5% | -2.3% | -2.5% | -1.6% | -1.3% | -1.4% | -0.7% | -0.5% | -0.8% |
| RGB, mixed content, 1080p | -3.6% | -3.4% | -3.5% | -2.5% | -2.4% | -2.4% | -0.9% | -0.4% | -1.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -7.3% | -6.9% | -7.0% | -3.6% | -3.2% | -3.3% | -2.2% | -1.8% | -2.0% |
| YUV, text & graphics with motion,720p | -2.9% | -2.3% | -2.7% | -2.1% | -1.9% | -1.9% | -0.9% | -0.9% | -0.7% |
| YUV, mixed content, 1440p | -2.5% | -2.4% | -2.4% | -1.8% | -1.7% | -1.6% | -0.8% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -3.8% | -3.5% | -3.8% | -2.9% | -2.7% | -2.9% | -1.4% | -0.4% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 125% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 102% |  |  |

Table 6. Results of Subset 3 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.7% | 7.8% | 5.4% | 12.0% | 5.0% | 5.6% | 3.0% | 8.6% | 4.8% | 4.7% | 2.7% | 6.3% |
| RGB, text & graphics with motion,720p | 1.5% | 1.6% | 0.4% | 2.6% | 0.4% | 0.8% | 0.2% | 2.1% | 0.3% | 0.6% | 0.2% | 1.5% |
| RGB, mixed content, 1440p | 1.7% | 1.6% | 1.4% | 1.9% | 0.3% | 0.3% | 0.1% | 0.5% | 0.2% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 8.4% | 8.5% | 6.2% | 12.6% | 5.5% | 6.2% | 3.7% | 9.1% | 5.2% | 5.3% | 3.5% | 6.9% |
| YUV, text & graphics with motion,720p | 1.6% | 1.8% | 0.5% | 3.3% | 0.4% | 1.0% | 0.2% | 2.8% | 0.3% | 0.7% | 0.2% | 1.9% |
| YUV, mixed content, 1440p | 1.5% | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.1% | 0.5% | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | 1.7% | 1.7% | 1.7% | 1.7% | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 119% |  |  |  | 104% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |

# Conclusion

In this contribution, the full-frame search is applied to IntraLC under full-frame configuration. It is observed that 10.1% coding gain can be achieved when applying full-frame search to IntraLC. Additionally, on top of enlarged full-frame search range for 8x4/4x8/4x4 IntraBC PUs, 7.3% BD-rate saving of IntraLC with full-frame search is observed. According to the interesting gains, we would recommend the committee to consider further studying this technique in a CE.

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-R1015, Sapporo, Jul. 2014.

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).