| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0138 |
| --- | --- |

| Title: | Non-CE6: Exponential Golomb binarization for Palette Run |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a concatenated binarization based on fixed length code and exponential golomb code for the syntax element ‘palette_run.’ Specifically, greater_than_2_flag was replaced by greater_than_5_flag. Here, if greater_than_5_flag is equal to 0 then fixed length code with length of 2 is used to signal the palette_run information between 2 and 5. If greater_than_5_flag is equal to 1, exponential-golomb binarization is employed to code ‘palette_run-6’. It is reported that the proposed approach provides bit rate reductions of 0.3 %, 0.4% for All Intra 1080p text and graphics RGB and YUV categories, respectively.

# Introduction

Palette coding is an important tool which operates in the pixel domain. For each CU, pixel values within the CU are signalled using a palette table. Specifically, each pixel value is represented by an index in the palette table. The spatial correlation in the index map is exploited for compression efficiency by using spatial prediction such as horizontal prediction (‘INDEX_MODE’) and vertical prediction (‘COPY_ABOVE_MODE’). In order to remove the redundancy in the index map, a syntax element called ‘palette_run’ is used. The syntax element ‘palette_run’ represents the number of consecutive locations minus 1 with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_MODE or represents the number of consecutive locations minus 1 with the same palette index as the current pixel when palette_run_type_flag is equal to INDEX_MODE. The structure of palette_run coding is dipicted in Figure 1.

Figure 1 The coding structure of ‘palette_run’

# Proposed Method

The proposed palette_run coding structure is described in Table 1. As shown in Table 1, greater_than_2_flag was replaced by greater_than_5_flag. If greater_than_5_flag is equal to 0 then fixed length code with length of 2 is used to signal between 2 and 5. If greater_than_5_flag is equal to 1, exponential-golomb binarization is employed. The proposed EG3 binarization is currently being used for existing syntax elements such as intra block vector coding.

Table 1 proposed palette coding structure

| Current Palette_run coding | Proposed |
| --- | --- |
| Code significant flag (context coded) if(significant flag ==1) Code GR_1 flag (context coded) if (GR_1 flag ==1) Code GR_2 flag (context coded) if(GR_2 flag ==1) Code Golomb-rice (‘Run-3’) | Code significant flag (context coded) if(significant flag ==1) Code GR_1 flag (context coded) if (GR_1 flag ==1) Code GR_2 flag (context coded) if(GR_5 flag ==0) Code Fixed Length Code (‘Run-2’) else Code Exp-Golomb (‘Run-6’) |

# Experimental Results

The compression performance is measured using BD rate compared with CE6 anchors, using the CE6 test conditions [1]. Table 2 and Error: Reference source not found give the detailed average BD rate reduction for lossy and lossless coding, respectively.

Table 2: Results with respect to the SCM2.0 anchor for full frame Intra Block Copy (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot,1080p | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |
| RGB, text & grap. with mot,720p | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| YUV, text & grap. with mot, 1080p | -0.4% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |
| YUV, text & grap. with mot.,720p | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | -0.1% | -0.7% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% | 0.1% | -0.2% | -0.2% | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.0% |

Table 3: Results with respect to the SCM2.0 anchor for full frame Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.1% | 0.1% |  |  | 0.0% | 0.2% |  |  | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | 0.0% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.1% | 0.1% |  |  | 0.0% | 0.2% |  |  | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | 0.0% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

The proposed concatenated binarization for the syntax element ‘palette_run’ is based on fixed length code and exponential golomb code. In the given binarization, existing greater_than_2_flag was replaced by greater_than_5_flag. Here, if greater_than_5_flag is equal to 0 then fixed length code with length of 2 is used to signal the palette_run information between 2 and 5. If greater_than_5_flag is equal to 1, exponential-golomb binarization is employed to code ‘palette_run-6’. The results show that BD rate changes are up to -0.4%, -0.2%, - 0.3% for AI, RA, LD, respectively. The proposed binarization is only based on the existing binarization methods such as FLC and EG3 and was verified to provide better coding performance. Therefore we propose to have it adopt into SCM2.0 draft text.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 6 (CE6): Improvements of palette mode”, JCTVC-R1106, Jul. 2014, Sapporo, JP.

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 1”, JCTVC-R1005, Jul. 2014, Sapporo, JP.

# Draft specification changes

Modifications are highlighted in yellow.

#### 9.3.3.13 Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run.

Output of this process is the binarization of palette_run as specified in Table 9-1.

Table 9-1 – Binarization for palette_run

| palette_run | Codeword |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 1100 |
| 3 | 1101 |
| 4 | 1110 |
| 5 | 1111 |
| >5 | Prefix = '111', Suffix EG3 |