| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0140_r1 |
| --- | --- |

| Title: | On transform coefficient scaling for adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution identifies a problem for the adaptive colour transform where the quantization parameter (qP) may take on negative values. This is a result of absorbing the scaling associated with adaptive colour transform within the transform coefficient scaling process. Note, that the transform coefficient scaling is undefined for negative qP values since the modulus operation is undefined for negative arguments. To avoid this issue it is proposed that for blocks where the effective qP may take on negative values, adaptive colour transform is disabled. Alternatively, one can clip qP to a valid range.

# Introduction

In the current HEVC screen content coding (SCC) draft the quantization parameter is adjusted to absorb the scaling associated with adaptive colour transform as follows:

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr	 + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0)	(8-3)

However when Qp′Y or Qp′Cb is less than 5, or Qp′Cr is less than 3 the resulting qP takes on negative value. In such an event the highlighted part of the transform coefficient scaling equation is undefined:

d[ x ][ y ] = Clip3( coeffMin, coeffMax, ( ( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * m[ x ][ y ] * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift )	(8-4)

To resolve this issue it is proposed that either:

- Whenever Qp′Y or Qp′Cb is less than 5, or Qp′Cr is less than 3 adaptive colour transform is disabled

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| … |  |
| else { |  |
| … |  |
| else { |  |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| if( rqt_root_cbf ) { |  |
| if(!( Qp′Y < 5 ¦¦ Qp′Cb < 5 ¦¦ Qp′Cr < 3) && residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

OR (b) Clip qP to a valid range

–	If cIdx is equal to 0,

qP = Max(0,Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0))

–	Otherwise, if cIdx is equal to 1,

qP = Max(0,Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0))

–	Otherwise (cIdx is equal to 2),

qP = Max(0,Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0))

# Experimental Results

To illustrate the detrimental effect of negative qP, SCM-2.0 was run for 16 frames with quantization parameters {1, 2, 3, 4}. We picked an encoder configuration and sequence at random and include the resulting bitrates and PSNR (for G channel) below in Table 1 for comparison.

Table 1 Bitrates and PSNR comparison (“FlyingGraphics_RGB” in AI )

|  | SCM 2.0 |  | Option (a) |  | Option (b) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Quantization parameter | Bitrates | PSNR (G) | Bitrates | PSNR (G) | Bitrates | PSNR (G) |
| 1 | 204865.4700 | 33.6777 | 228095.8500 | 84.9411 | 284879.5200 | 78.8354 |
| 2 | 377669.5800 | 39.7507 | 218379.3000 | 80.4757 | 257264.1600 | 77.0936 |
| 3 | 326218.4100 | 63.2395 | 202108.1400 | 77.1090 | 225076.8900 | 75.0776 |
| 4 | 208964.7000 | 69.4524 | 178521.4200 | 71.6991 | 189562.9800 | 70.6202 |

# As can be see both proposed modifications remove the loss in coding efficiency due to negative qP at lower quantization parameters. Similar behavior is observed for other sequences and encoder configurations.

# Conclusion

It was demonstrated that negative qP resulting from use of adaptive colour transform at low quantization parameters can result in significant degradation in performance. It is recommended to adopt in SCC draft:

Either (a) Disabling adaptive colour transform when Qp′Y or Qp′Cb is less than 5, or Qp′Cr is less than 3; Or (b) Clip qP to a valid range

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Rajan Joshi, Jizheng Xu, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 1,” 18th meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Sapporo, JP, 30 June – 9 July 2014