| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0141_r1 |
| --- | --- |

| Title: | Using the wavefront store-and-sync design for palette table prediction variables |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the use of the original HEVC design of store-and-sync for palette table prediction variables when wavefront parallel processing (WPP) is enabled. This approach was first proposed in JCTVC-R0233 (albeit without BD-rate performance data). In this contribution, the performance of the proposed approach is compared versus a bug-fixed SCM-2.0 anchor with WPP enabled and used as anchors. The bug-fix is necessary to prevent decoder crashes when WPP is enabled and incorporates changes from r4061. The issue and its resolution were reported in JCT-VC HEVC bug-tracker issue #1317.

Average BD rate improvement of YUV Text & graphics with motion 1080P for the proposed approach is:  0.9%, 0.4%, 0.4% for AI, RA, LD in lossy case and 0.8%, 0.2%, 0.2% for AI, RA, LD in lossless case..

# Introduction

In the current HEVC screen content coding draft [1] palette predictors are reset at the start of a wavefront. This is a departure from the original HEVC design where: (a) CABAC context variables and (b) rice parameter initialization variables; are copied from the CTB row above.

This contribution proposes that consistent with the original HEVC store-and-sync design, palette table prediction variables be copied from the CTB row above. It is demonstrated that the proposed change improves compression efficiency.

The text modifications in the draft specification, corresponding to the proposed change, are listed under section 6.

# Experimental Results

The compression performance is measured using BD rate compared with SCM-2.0 anchor WPP enabled. Here, bug-fix patch (r4061) is incorporated. Note that the bug-fix is necessary to prevent decoder crashes when WPP is enabled and incorporates changes from r4061. The issue and its resolution were reported in JCT-VC HEVC bug-tracker issue #1317 and the bug-fix.

Lossy:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.9% | -0.8% |
| RGB, text & graphics with motion,720p | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.4% | -0.4% | -0.7% |
| YUV, mixed content, 1440p | -0.3% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -0.1% | 0.0% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | -0.8% | -0.4% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | 0.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.5% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | 0.3% | 0.9% | 0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

Lossless:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.6% | 0.1% | 1.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.1% | 1.4% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.3% | 0.1% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.1% | 0.8% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.1% | 0.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

This contribution proposes the use of the original HEVC design of store-and-sync for palette table prediction variables when wavefront parallel processing (WPP) is enabled. The proposed method contributes BD rate improvement of 0.3%, 0.2%, 0.1% for AI, RA, LD in lossy case and 0.2%, 0.1%, 0.1% for AI, RA, LD in lossless case.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Rajan Joshi, Jizheng Xu, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 1,” 18th meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Sapporo, JP, 30 June – 9 July 2014

# Draft specification changes

Modifications are highlighted in yellow.

9.3	CABAC parsing process for slice segment data

9.3.1	General

…

The storage process for context variables is applied as follows:

  - When ending the parsing of the coding tree unit syntax in subclause 7.3.8.2, entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 1 or both CtbAddrInRs is greater than 1 and TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 2 ] ], the storage process for context variables, Palette table prediction variables, and Rice parameter initialization states as specified in subclause 9.3.2.3 is invoked with TableStateIdxWpp, TableMpsValWpp, TablePreviousPaletteEntriesWpp, PreviousPaletteSizeWpp and TableStatCoeffWpp as outputs.

…

9.3.2	Initialization process

9.3.2.1	General

Outputs of this process are initialized CABAC internal variables, and the initialized Rice parameter initialization states StatCoeff, previous palette table previousPaletteEntries, and previous palette size previousPaletteSize.

…

The context variables of the arithmetic decoding engine are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the following applies:

  - The initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 0 or TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 1 ] ], the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 9-2) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-3)

  - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables and Rice parameter initialization states as specified in subclause 9.3.2.4 is invoked with TableStateIdxWpp, TableMpsValWpp, TablePreviousPaletteEntriesWpp, PreviousPaletteSizeWpp, and TableStatCoeffWpp as inputs.

  - Otherwise, the following applies:

  - The initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

…

9.3.2.3	Storage process for context variables, Palette table prediction variables and Rice parameter initialization states

Inputs to this process are:

–	The CABAC context variables indexed by ctxTable and ctxIdx.

–	The previous palette table indexed by cIdx and n, and the size of the previous index table.

–	The Rice parameter initialization states indexed by k.

Outputs of this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the initialization process of context variables and Rice parameter initialization states that are assigned to all syntax elements in subclauses 7.3.8.1 through 7.3.8.11, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variables tablePreviousPaletteEntriesSync, PreviousPaletteSizeSync containing the values of the variables previousPaletteEntries[ cIdx ][ n ], previousPaletteSize used in the initialization process of context variables, Palette table predictor and Rice parameter initialization states.

–	The variables tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the initialization process of context variables, Palette table prediction variables, and Rice parameter initialization states.

For each context variable, the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMps.

For each Previous palette table entry, where cIdx varies from 0 to ( ChromaArrayType != 0 ? 2 : 0 ), inclusive, and n varies from 0 to previous PaletteSize-1, inclusive, each entry of the table tablePreviousPaletteEntriesSync is initialized to the corresponding value of previousPaletteEntries[ cIdx ][ n ].

For each Rice parameter initialization state k, each entry of the table tableStatCoeffSync is initialized to the corresponding value of StatCoeff[ k ].

The storage process for context variables is illustrated in the flowchart of Figure 9 4.

…

- 9.3.2.4	Synchronization process for context variables, Palette table prediction variables and Rice parameter initialization states

Inputs to this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the storage process of context variables that are assigned to all syntax elements in subclauses 7.3.8.1 through 7.3.8.11, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variable tablePreviousPaletteEntriesSync and previousPaletteSizeSync containing the values of the variables previousPaletteEntries[ cIdx ][ n ] and previousPaletteSize used in the storage process of context variables, Palette table prediction variables and Rice parameter initialization states.

–	The variable tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the storage process of context variables, Palette table prediction variables and Rice parameter initialization states.

Outputs of this process are:

–	The initialized CABAC context variables indexed by ctxTable and ctxIdx.

–	The initialized Previous palette table entries indexed by cIdx and n, and Previous palette table size.

–	The initialized Rice parameter initialization states StatCoeff indexed by k.

For each context variable, the corresponding context variables pStateIdx and valMps are initialized to the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync.

For each Previous palette table entry n, where cIdx varies from 0 to ( ChromaArrayType != 0 ? 2 : 0 ), inclusive,, each variable previousPaletteEntries[ cIdx ][ n ] is initialized to the corresponding entry of table tablePreviousPaletteEntriesSync. The variable previousPaletteSize is initialized to previousPaletteSizeSync.

For each Rice parameter initialization state, each variable StatCoeff[ k ] is initialized to the corresponding entry of table tableStatCoeffSync.