| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0142r3 |
| --- | --- |

| Title: | CE1: Results of Test 1.1, Test 2.1 and Test 3.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka, Marta Karczewicz, Chao Pang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com martak@qti.qualcomm.com cpang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports experimental results of CE1 Test 1.1, Test 2.1 and Test 3.1 related to intra block vector difference (BVD) and motion vector difference (MVD) coding techniques proposed in JCTVC-R0181. In Test 1.1, a BVD coding method is proposed for intra block copy mode. In this method, two regular bins are coded to indicate if the BVD value is greater than 0 and 4 respectively. The remaining bins are bypass coded using a Exp-Golomb(EG) or fixed length code. In Test 2.1, a MVD coding method is proposed for motion vector coding same as Test1.1 but applied to motion vector difference.  In Test 3.1, a BVD and MVD coding method is proposed which combines the methods in Test 1.1 and Tests 2.1. It is reported that the proposed approach in Test 1.1 provides objective bit rate reductions of -0.3 %, -0.3% for All Intra 1080p text and graphics RGB and YUV categories, respectively over SCM-2.0 anchor under lossy CTC. It is reported that the proposed approach in Test 2.1 provides objective bit rate reductions of -2.3 %, -2.6% for random access 1080p text and graphics RGB and YUV categories, respectively and -3.0% and -3.3% for low delay 1080p text and graphics RGB and YUV categories over SCM-2.0 anchor under lossy CTC. It is reported that the proposed approach in Test 3.1 provides objective bit rate reductions of -2.4 %, -2.7% for random access 1080p text and graphics RGB and YUV categories, respectively and -3.1% and -3.4% for low delay 1080p text and graphics RGB and YUV categories over SCM-2.0 anchor under lossy CTC.

In the revision 1 of this document, worst-case cabac throughput calculations for Test 1.1 and test 2.1 are uploaded as a part of package.

In the revision 2 of this document, further results are provided that were requested by experts during the BoG on complexity assessment of IBC block vector coding. Following additional results were requested,

- Performance of proposed method for local search test conditions with no hash for intraBC and inter.

It is reported that the proposed approach under above test conditions provides objective bit rate reductions of -1.8 %, -2.2% for random access 1080p text and graphics RGB and YUV categories, respectively and -2.4% and -2.8% for low delay 1080p text and graphics RGB and YUV categories over SCM-2.0 anchor under lossy CTC.Performance of proposed method when incorrect bit estimation (that of HEVC MVD coding) is used.

It is reported that the proposed approach provides objective bit rate reductions of –x.x2.2 %, -2.5% for random access 1080p text and graphics RGB and YUV categories, respectively and –x.x2.7% and –x.x3.1% for low delay 1080p text and graphics RGB and YUV categories over SCM-2.0 anchor under lossy CTC.

In the revision 3 of the document, a performance results of 4x4 and Test 1.1 are show

# Proposed method

It is observed that the distributions of BVD’s and MVD’s for screen content sequences were different from that of camera captured sequences. It was asserted that the current binarization methods were inefficient in adapting to these characteristics and CE1 was organized to study binarization methods. The following tests related to BVD and MVD coding identified and evaluated are discussed below

## Test 1.1

This method is applied for coding block vector differences. First a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than 4. If BVD is greater than 4, the remaining absolute level is coded using bypass Exp-Golomb codes with order 4. Otherwise, the remaining absolute level is coded using fixed length bypass code of length 2. One flag is coded to indicate sign of the BVD.

## Test 2.1

This method is applied for coding motion vector difference. First a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than 4. If MVD is greater than 4, the remaining absolute level is coded using bypass Exp-Golomb codes with order 4. Otherwise, the remaining absolute level is coded using fixed length bypass code of length 2. One flag is coded to indicate sign of the MVD.

## Test 3.1

This method combines Test 1.1 and Test 2.1 and is applied for coding block vector and motion vector difference.

# Results

Experimental results are provided for the methods Test 1.1, Test 2.1 and Test 3.1 implemented over SCM2.0 under SCC test conditions.

## Experimental Results of Test 1.1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1080p | -0.3% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 100% |  |  |

## Experimental Results of Test 2.1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.3% | -2.3% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.6% | -2.5% | -2.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.5% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.0% | -2.9% | -2.9% |
| RGB, text & graphics with motion,720p | -1.1% | -1.0% | -0.9% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -3.3% | -3.1% | -3.1% |
| YUV, text & graphics with motion,720p | -1.3% | -1.1% | -1.2% |
| YUV, mixed content, 1440p | 0.0% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | -0.3% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | -0.2% | -0.1% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 100% |  |  |

## Experimental Results of Test 3.1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.4% | -2.4% | -2.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -2.7% | -2.6% | -2.6% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.1% | -3.0% | -2.9% |
| RGB, text & graphics with motion,720p | -1.2% | -1.1% | -0.9% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.6% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.4% | -3.2% | -3.2% |
| YUV, text & graphics with motion,720p | -1.4% | -1.4% | -1.5% |
| YUV, mixed content, 1440p | -0.3% | -0.4% | 0.1% |
| YUV, mixed content, 1080p | -0.4% | -0.4% | -1.1% |
| YUV, Animation, 720p | -0.1% | -0.7% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 100% |  |  |

## Additional results for following configuration are provided

Performance of proposed method for local search test conditions with no hash for intraBC and inter.

Performance of proposed method when incorrect bit estimation (that of HEVC MVD coding) is used.

Below table provides side by side comparison of SCM2.0 vs SCM2.0_no4x4IBC and SCM2.0 vs Test 1.1

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Text specification

CE1 Test 1.1

The text changes for the proposed method are marked in yellow.

### IntraBC Block vector difference syntax

| bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_bvd_greater0_flag[ 0 ] ) |  |
| abs_bvd_greater4_flag[ 0 ] | ae(v) |
| if(abs_bvd_greater0_flag[ 1 ] ) |  |
| abs_bvd_greater4_flag[ 1 ] | ae(v) |
| if(abs_bvd_greater0_flag[ 0 ] ) { |  |
| if(abs_bvd_greater4_flag[ 0 ] ) |  |
| abs_bvd_ minus5[ 0 ] | ae(v) |
| else |  |
| abs_bvd_minus1[ 0 ] | ae(v) |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_bvd_greater0_flag[ 1 ] ) { |  |
| if(abs_bvd_greater4_flag[ 1 ] ) |  |
| abs_bvd_ minus5[ 1 ] | ae(v) |
| else |  |
| abs_bvd_minus1[ 1 ] | ae(v) |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

Semantics

abs_bvd_greater0_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 0.

abs_bvd_greater4_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 4.

When abs_bvd_greater4_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_bvd_minus1[ compIdx ] plus 1 specifies the absolute value of a block vector component difference.

When abs_bvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

abs_bvd_ minus5[ compIdx ] plus 5 specifies the absolute value of a block vector component difference.

When abs_bvd_minus5[ compIdx ] is not present, it is inferred to be equal to −5.

bvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

If bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

Otherwise (bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference lBvd [ compIdx ] for compIdx = 0..1 is derived as follows:

lBvd [ compIdx ] = abs_bvd_greater0_flag [ compIdx ]  *  (  abs_bvd_greater4_flag? : (abs_bvd_minus5 [ compIdx ] + 5 )  :  ( abs_bvd_minus1 [ compIdx ] + 1 ) ) * ( 1 − 2 * bvd_sign_flag [ compIdx ] );

| Table 9-xx – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| bvd_coding ( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_greater4_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_minus1 [ ] | FL | cMax =4 |
|  | abs_bvd_minus5 | EG4 |  |
|  | bvd_sign_flag [ ] | FL | cMax = 1 |

CE1 Test 2.1

### Motion vector difference syntax

The text changes for the proposed MVD coding method are marked in yellow.

| mvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater4_flag[ 0 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater4_flag[ 1 ] | ae(v) |
| if(abs_mvd_greater0_flag[ 0 ] ) { |  |
| if(abs_mvd_greater4_flag[ 0 ] ) |  |
| abs_mvd_ minus5[ 0 ] | ae(v) |
| else |  |
| abs_mvd_minus1[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag[ 1 ] ) { |  |
| if(abs_mvd_greater4_flag[ 1 ] ) |  |
| abs_mvd_ minus5[ 1 ] | ae(v) |
| else |  |
| abs_mvd_minus1[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

Semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 0.

abs_mvd_greater4_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 4.

When abs_mvd_greater4_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus1[ compIdx ] plus 1 specifies the absolute value of a block vector component difference.

When abs_mvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

abs_mvd_ minus5[ compIdx ] plus 5 specifies the absolute value of a block vector component difference.

When abs_mvd_minus5[ compIdx ] is not present, it is inferred to be equal to −5.

mvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference lMvd [ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd [ compIdx ] = abs_mvd_greater0_flag [ compIdx ]  *  (  abs_mvd_greater4_flag? : (abs_mvd_minus5 [ compIdx ] + 5 )  :  ( abs_mvd_minus1 [ compIdx ] + 1 ) ) * ( 1 − 2 * mvd_sign_flag [ compIdx ] );

| Table 9-xx – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| mvd_coding ( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater4_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus1 [ ] | FL | cMax =4 |
|  | abs_mvd_minus5 | EG4 |  |
|  | mvd_sign_flag [ ] | FL | cMax = 1 |