| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0144 |
| --- | --- |

| Title: | Qp derivation for adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Li Zhang Rajan Joshi Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the 18th JCTVC meeting, the adaptive color transform was adopted into the test model of SCC extension. The tool adaptively transforms prediction residuals on one color space into another and signals a flag to indicate its color space at CU level. This contribution identifies various issues in the current test model related to quantization parameter derivation when adaptive color transform is used and proposes solutions to fix them.

# Introduction and Problem statement

It is asserted that in the current test, the QP derivation when cu_residual_act_flag is enabled has following problems. In the current working draft when adaptive color transform is enabled, during the scaling and transformation process, a QP offset of -5 is added for luma and Cb chroma component, and -3 is added for Cr chroma component. It is asserted that the resultant value of Qp may underflow the allowed Qp range. For example, in the current test model it is possible that the resultant Qp may underflow to -5 when the range of HEVC is between (0, 51). It is also asserted that adaptive QP offset signalling when adaptive color transform is enabled may provide additional flexibility. The Qp derivation in the test model is as follows

### 8.6.2 Scaling and transformation process

-----

The quantization parameter qP is derived as follows in the current test model,

If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0)	(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0)	(8-3)

where cIdx specifies the colour component of the current block and cu_residual_act_flag specifies whether adaptive colour transform is applied to the residual samples of the current coding unit.

# Proposal and Technical Discussion

This section presents solutions that address the problems detailed in the above section.

## Clip the Qp’s within the HEVC Qp’s range

In order to keep the allowed Qp range same to that of HEVC when adaptive color transform is used, it is proposed to clip the range of the Qp values to that of HEVC Qp range. The proposed changes to the test model are highlighted in yellow text.

### 8.6.2 Scaling and transformation process

-----

- If cIdx is equal to 0,

qP  = Clip3( 0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5  : 0))

When ChromaArrayType is not equal to 0,

- if cIdx is equal to 1,

qP  = Clip3( 0, 51 + QpBdOffsetC, Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 :  0))	(8-4)

- Otherwise (cIdx is equal to 2),

qP  = Clip3( 0, 51 + QpBdOffsetC , Qp′Cr	 + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 :  0)	(8-5)

## Flexible signalling of QP offset for adaptive color transform

| pps_scc_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| if(residual_adaptive_colour_transform_enabled_flag){ |  |
| pps_slice_act_qp_offsets_present_flag |  |
| pps_ act _y_qp_offset | se(v) |
| pps_ act _cb_qp_offset |  |
| pps_ act _cr_qp_offset |  |
| } |  |
| … |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| …………….. |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if(pps_slice_act_qp_offsets_present_flag){ |  |
| slice_ act _y_qp_offset | u(1) |
| slice_ act _cb_qp_offset |  |
| slice_ act _cr_qp_offset |  |
| } |  |
| …… |  |

It is proposed to clip the range of the QP values for luma and chroma component.

If cIdx is equal to 0,

qP  = Clip3( 0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] ?   pps_ act _y_qp_offset + slice_act_y_qp_offset : 0))

When ChromaArrayType is not equal to 0,

if cIdx is equal to 1,

qP  = Clip3( 0, 51 + QpBdOffsetC, Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? pps_act_ pps_cb_qp_offset + slice_act_cb_qp_offset : 0))	(8-6)

Otherwise (cIdx is equal to 2),

qP  = Clip3( 0, 51 + QpBdOffsetC , Qp′Cr	 + (cu_residual_act_flag[ xTbY ][ yTbY ] ? pps_act_ pps_cr_qp_offset + slice_act_cr_qp_offset : 0)	(8-7)

pps_ act _y_qp_offset ,pps_act_cb_qp_offset and pps_act_cr_qp_offset specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of pps_ act _y_qp_offset, pps_cb_qp_offset and pps_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_act_cb_qp_offset and pps_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value.

pps_slice_act_qp_offsets_present_flag equal to 1 specifies that slice_act_y_qp_offset, slice_act_cb_qp_offset, slice_act_cr_qp_offset are present in the slice header. pps_slice_act_qp_offsets_present_flag equal to 0 specifies that slice_act_y_qp_offset, slice_act_cb_qp_offset, slice_ act _cr_qp_offset are not present in the slice header. When not present, the value of pps_slice_act_qp_offsets_present_flag is inferred to be equal to 0.

slice_ act _y_qp_offset ,slice_cb_qp_offset and slice_cr_qp_offset specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of slice_ act _y_qp_offset, slice_cb_qp_offset and slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, slice_act_cb_qp_offset and slice_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value.

## Signalling the presence of QP offset for adaptive color transform

As discussed in section 1 , fixed negative QP offset when adaptive color transform is enabled narrows the Qp range at the higher Qp’s. For example, with the current definition when adaptive color transform is enabled it is impossible to reach QP’s over 46 + QpBdOffsetY, which in some scenarios are necessary to meet target bitrate. In the below solution, it is proposed to signal a flag to indicate whether Qp offset shall be added or not. The proposed changes to the test model are highlighted in yellow text.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| …………….. |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( residual_adaptive_colour_transform_enabled_flag) |  |
| slice_act_qp_offset_present_flag | u(1) |
| …… |  |

slice_act_qp_offset_present_flag equal to 1 specifies that a Qp offset is applied for the coding units with cu_residual_act_flag equal to 1. slice_act_qp_offset_present_flag equal to 0 specifies that a Qp offset is not applied for the coding units with cu_residual_act_flag equal to 1. When not present, the value of cu_chroma_qp_offset_enabled_flag is inferred to be equal to 0.

If cIdx is equal to 0,

qP  = Clip3( 0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag[ xTbY ][ yTbY ] && slice_act_qp_offset_present_flag?  -5 : 0))

When ChromaArrayType is not equal to 0,

if cIdx is equal to 1,

qP  = Clip3( 0, 51 + QpBdOffsetC,  Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] && slice_act_qp_offset_present_flag? -5: 0))	(8-8)

Otherwise (cIdx is equal to 2),

qP  = Clip3( 0, 51 + QpBdOffsetC , Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] && slice_act_qp_offset_present_flag? -3 : 0)	(8-9)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).