| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0145 |
| --- | --- |

| Title: | Bandwidth reduction method for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Ted Hsieh Chao Pang Vadim Seregin Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce average bandwidth when intra block copy (IBC) mode is used for prediction in the current picture. IBC mode uses previously decoded unfiltered samples within the same picture for its prediction. In the current test model, for the IBC mode the search range is unrestricted and can use any unfiltered decoded samples of the current picture (full search IBC). This introduces additional bandwidth required to store unfiltered decoded samples of the current picture (in addition to filtered decoded samples required for temporal prediction). It is observed that on average not all previously decoded unfiltered samples of the current picture are used for prediction in IBC mode and the usage depends on the characteristics of content, for example, the usage peaks for text and graphic sequences and is minimal for camera captured content. Hence, it is argued that always storing the previously decoded unfiltered samples for the current picture is inefficient from the bandwidth perspective. In this contribution it is proposed to indicate which of the previously decoded coded block tree’s(CTBs) are used for IBC prediction so that only those CTBs could be stored thereby reducing the average bandwidth.

# Introduction and Problem statement

Unlike the inter modes, the IBC mode uses already decoded unfiltered samples within the same picture for its prediction. In the current test model, for the IBC mode the search range is unrestricted (full search IBC). As shown in the figure1, the current blocks marked in green could predict from any of the previously decoded unfiltered samples (blocks marked in yellow). Since, the usage information for yellow blocks are not known until all the blocks in the current pictures are decoded, IBC introduces additional store of unfiltered samples for all the decoded blocks. However it is observed that some regions (marked in Red) are not used for IBC prediction and this usage depends on the characteristics of the content.  Hence, it is argued that always storing the previously decoded unfiltered samples (including the red region) for the current picture is inefficient from the bandwidth perspective.

Figure 1: An example IBC prediction distribution for a CU in a picture

# Proposal

In this contribution we propose to indicate the IBC prediction usage information such that decoders can selectively store only the regions that are used for prediction using IBC mode. It is proposed to signal a flag for each CTB to indicate whether a particular CTB is referenced for prediction using IBC mode atleast by one block.

## Text modifications

The proposed text changes are detailed below

#### General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …. | u(1) |
| if(pps_ibc_ref_usage_present_flag) { |  |
| num_ctb_in_slice | ue(v) |
| for( i = 0; i < num_ctb_in_slice; i++ ) |  |
| ibc_ref_usage_info[ i ] | u(1) |
| } |  |
| …. |  |
| } |  |

num_ctb_in_slice specifies the number of ibc_ref_usage_info[ i ]syntax elements in the slice header. When not present, the value of num_ctb_in_slice is inferred to be equal to 0.

ibc_ref_usage_info[ i ] equal to 0 specifies that the no sample within the coded block tree  i are used for prediction for intra block copy mode in the current picture. ibc_ref_usage_info[ i ] equal to 0 specifies that the sample withn the coded block tree  i may be used for prediction for intra block copy mode in the current picture. When ibc_ref_usage_info[ i ]  is not present, it is inferred to be 1.

#### General picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(intra_block_copy_enabled_flag) |  |
| pps_ibc_ref_usage_present_flag | u(1) |
| …. |  |
| } |  |

pps_ibc_ref_usage_present_flag equal to 1 specifies that intra block copy reference usage info is present in the slice segment header for all the coded tree blocks of the slice. pps_ibc_ref_usage_present_flag equal to 0 specifies that intra block copy reference usage info is not present in the slice segment header  When pps_ibc_ref_usage_present_flag is not present, it is inferred to be 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).