| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0148 |
| --- | --- |

| Title: | Indication of the end of coded data for pictures and partial-picture regions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjun Wu Lihua Zhu Shyam Sadhwani Gary J. Sullivan Microsoft Corp. One Microsoft Way Redmond WA 98052-6399, USA | Tel: Email: | +1-425-703-5308 yongjunw@microsoft.com lizhu@microsoft.com shyams@microsoft.com garysull@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This contribution proposes the specification of an SEI message to indicate the slice segment address of the next slice header (when present) for decoding latency minimization. It is asserted that it would be beneficial, especially in ultra-low latency use cases, to have an earlier indication of when the coded picture is complete, and particularly to have such an indication within the same access unit as that coded picture itself. Some decoders may be designed to wait for reception of the complete decoded picture before performing the parsing and decoding processes for the picture's VCL NAL units. Requiring such decoders to wait until the next access unit begins is asserted to sometimes add significant delay.

Moreover, for some decoding architectures that are based on region segmentation, it is asserted that it would be similarly beneficial for a decoder to have an indication of how much of the decoded picture has been sent in the preceding VCL NAL units (without waiting for the next slice header). To provide such an indication, an SEI message is proposed for HEVC to identify the slice segment address of the next slice header (when present). The SEI message is primarily proposed as a suffix SEI message.

The same problem is reported to be present in the AVC context, for which the contribution proposes using a previously-reserved NAL unit type (type code 22) for the same basic purpose, since suffix SEI messages are not (at least not yet) defined in AVC. More generally, it would be possible to add the general concept of suffix SEI messages to AVC, for which this message could be the first to be specified.

# Introduction / Problem Statement

In extremely low-latency usage, it is asserted that there may be significant "dead air" gaps (or filler data) between the transmission of different VCL NAL units, and between the last VCL NAL unit of a coded picture and the first NAL unit of the next coded picture. These gaps can add latency for decoders that are designed to wait for reception of an entire picture, as the detection of the completion of the picture may need to wait until the next access unit begins.

It is asserted that with the current HEVC syntax, the decoder must either fully parse the coded data of the slice segments it has received or it may need to wait until the next access unit begins (and possibly a subsequent slice header has been received) before it can determine whether or not the slice segments it has received contain the complete coded picture.

In some system environments, the data of the next access unit (such as an access unit delimiter) is handled in a way that prevents sending it early (e.g., by having it locked to the timing information or data packets of the next picture). It is also undesirable to need to alter the position of the access unit boundaries just so that an indication of the end of a picture can be sent sooner.

Moreover, access unit boundaries only have whole-picture granularity, whereas it may be beneficial to also be able to indicate the completion of partial-picture regions.

- Proposed syntax and semantics

| coded_region_completion( payloadSize ) { | Descriptor |
| --- | --- |
| next_segment_address | ue(v) |
| } |  |

next_segment_address identifies the value of the slice_segment_address in the next slice header in the bitstream (when present). When the next slice header has first_slice_in_pic_flag equal to 1 or no subsequent slice header is present in the bitstream, the value of next_segment_address shall be equal to 0.

- Additional considerations

  - Prefix or suffix presence

In the HEVC context, it is proposed that this SEI message be allowed to be either a prefix or suffix SEI message, so that it may be placed either in the same access unit as the prior slice segment or in the access unit with the next slice segment. The usage as a suffix SEI message seems the more useful of the two, but we see no harm in allowing either usage.

  - Representation of the next_segment_address

Note that the proposed coding of the next_segment_address uses ue(v) coding rather than u(v) coding, for two reasons:

- It avoids SPS dependency, and

- It also minimizes the number of bits needed to send the value 0, which is the usage we expect to be the most common usage (as it is the value sent to indicate that the next slice starts a new coded picture).

  - Usage with dependent slice segments

When considering the possibility of the use of dependent slice segments, two approaches are feasible:

- Prohibiting the present of the SEI message when the next slice segment is a dependent slice segment, or

- Specifying that when the next slice segment is a dependent slice segment, the SEI message identifies the position of the first CTU of the dependent slice segment.

We suggest approach #1, for the sake of conceptual simplicity, since some decoders may prefer to only decode complete slices, and this constraint would ensure that the entire slice has been completed.

  - Consideration of AVC

AVC would benefit from having the same indication capability. However, in AVC, there is no definition of suffix SEI messages. We therefore suggest using a previously-reserved NAL unit type code (type code 22) in the AVC context. The NAL unit type approach is also available in HEVC (e.g., with type code 45).

More generally, it would be possible to add the general concept of suffix SEI messages to AVC, for which this message could be the first to be specified.

It may also be desirable to define a NAL unit type extension mechanism for AVC.

- Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).