| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17 Oct. – 24 Oct. 2014 | Document: JCTVC-S0149 |
| --- | --- |

| Title: | Fast intra coding mode decision for screen contents coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong-Jo Ahn Xiangjian Wu Woong Lim Jonghyun Ma Donggyu Sim Kwangwoon University 20, Gwangun-ro, Nowon-gu Seoul, South Korea | Tel: Email: | +82-2-940-5470 yongjoahn@kw.ac.kr xjw0118@kw.ac.kr limwoong@kw.ac.kr mday519@kw.ac.k dgsim@kw.ac.kr |
| Source: | KWU (Kwangwoon University) |  |  |

_____________________________

# Abstract

In this contribution, an early termination of intra coding mode decision is proposed to reduce an encoding complexity of the screen contents coding (SCC) extensions. Comparing to HEVC version 1, the computational complexity of SCC extensions has been increased due to newly added intra coding mode as intra block copy (IBC). To reduce complexity from additional mode decision in encoder, adjustment of decision order for intra coding modes and early termination based on coding information of IBC 2Nx2N mode are proposed in this contribution. The proposed method achieved about 10% reduction in encoding time with BD-bitrate gain of 0.28% compared with that of SCC extensions test model 2.0 encoder in all intra (AI) case.

# Introduction

Intra block copy (IBC) was adopted as a new intra coding mode for HEVC Range Extensions and it leads to large bitrate saving for screen contents. However, additional intra coding mode decision raises the encoding complexity. To reduce encoder complexity from additional intra coding mode, a fast encoding methods [1-2] are introduced before. In [1], a fast IBC search method was proposed by using 1-D search for 16x16 CUs and 2-D search for 8x8 CUs. In addition, the method was disabled IBC search for 64x64 and 32x32 CUs. In [2], early skipping decision methods of IBC search using intra mode cost and CU activity for 8x8 CUs were proposed. These fast encoding methods was adopted in screen contents coding (SCC) extensions test model (SCM) software.

To achieve additional reduction of encoding complexity with adopted fast encoding methods in SCM, this contribution proposes an early termination of intra coding mode decision by checking the coding information of an IBC 2Nx2N mode.

# Proposed techniques

This contribution proposes an early termination of intra coding mode decision to reduce the encoding complexity of screen contents coding (SCC) extensions by simply checking the block vector difference (BVD) and a coded block flag (CBF) after encoding the IBC 2Nx2N mode. For checking the coding information of the IBC 2Nx2N, the coding order of the intra coding modes is adjusted in this contribution. The flowcharts of the mode decision process of SCM 2.0 and that of the proposed method are shown in Fig. 1 and Fig. 2, respectively. As shown in Fig. 2, the current CU checks IBC 2Nx2N mode before checking the Intra 2Nx2N mode. After encoding the IBC 2Nx2N mode, the proposed method checks its BVD and CBF. If BVD and CBF of the IBC 2Nx2N mode are respectively equal to (0, 0) and zero, the encoding process of the next intra coding modes (e.g., Intra 2Nx2N, Intra NxN, Intra PCM, IBC 2NxN, IBC Nx2N, and IBC NxN) is terminated.

Figure 1. Mode decision process of SCM 2.0

Figure 2. Mode decision process of the proposed method

# Performance evaluation

The proposed method was evaluated under the common test conditions for screen content coding [3], and it is integrated on HM-15.0+RExt-8.0+SCM-2.0 [4].

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.1% | -2.1% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.3% | 0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.3% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.2% | 0.0% | 0.1% |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.6% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.4% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.2% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.2% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.5% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution proposes a fast intra coding mode decision for SCC in SCM 2.0. It is reported that the proposed method has a reduction of encoding time by about 10% on average with 0.28% BD-BR gain compared to SCM 2.0 in all intra (AI) case with already adopted fast encoding methods for IBC.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra motion compensation with 2D MVs,” JCTVC-N0256, 14th JCT-VC meeting, Vienna, AT, August 2013.

- D.-K. Kwon and M. Budagavi, “AHG5: Fast encoding using early skipping of intra block copy (IntraBC) search,” JCTVC-O245, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” JCTVC-R1015, 18th JCT-VC meeting, Sapporo, JP, July 2014.

- HM-15.0+RExt-8.0+SCM-2.0 (SCM 2.0),
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0

# Patent rights declaration(s)

KWU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).