| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0150 |
| --- | --- |

| Title: | Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Zhan Ma, Wei Wang, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution simplifies the coding method for Escape colors in the palette mode by treating Escape index as one of normal color indices and moves the actual coding of the Escape colors to the end of index map processing.  This method helps COPY_ABOVE and INDEX_MODE modes and also improves processing throughput without performance loss on average (It even provides performance improvement for several categories using All Intra configuration).

# Description

Color palette mode was recently introduced into HEVC SCC WD and reference software SCM 2.0 and has shown noticeable gains over SCM 1.0. It applies to all CU depths in a recursive quad-tree fashion.  For each CU, it generally involves two main parts, i.e., color table processing and index map coding. In the current design of index map coding, Escape colors and color indices are processed in an interleaved manner. Neither INDEX_MODE nor COPY_ABOVE process supports Escape color index.

Alternatively, this contribution proposes to separate the processing of Escape color from the color index processing.

- Escape color index will be processed as a normal index in the INDEX_MODE and COPY_ABOVE operations.  It only stops whenever it meets an unmatched index.

- All Escape colors will be processed together after the encoding of color index map is complete.

This simplification does not introduce any new syntax element for palette mode, just re-orders the Escape color coding so as to enable the parallel processing for color index map and Escape colors at encoder. Intuitively, unless there is only one Escape color, this simplification should also improve the coding efficiency by counting the RUN continuously (instead of stopping it).

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [1] (w/ full frame intra block copy). Results for lossy and lossless are summarized in Table 1 and 2.

Table 1 Lossy Coding efficiency of Proposed Escape Coding Simplification versus SCM 2.0

Table 2 Lossless Coding efficiency of Proposed Escape Coding Simplification versus SCM 2.0

Additional experiments have been carried out when the max. sizes of current palette table and palette predictor table are reduced to 15 and 16, respectively.  More coding efficiency has been observed from Table 3 and 4. It demonstrates that the coding efficiency could be further improved by this simplification at the case that smaller size of color palette and reference table is applied.

Table 3 Lossy Coding efficiency of Proposed Escape Coding Simplification versus SCM 2.0 (w/ size of max. palette size = 15 and palette predictor size = 16)

Table 4 Lossless Coding efficiency of Proposed Escape Coding Simplification versus SCM 2.0 (w/ size of max. palette size = 15 and palette predictor size = 16)

Another set of data is also provided to demonstrate the Escape Coding simplification by using CU dependent maximum palette size as describe in JCTVC-S0201 [2]. Here the anchor is SCM 2.0 + CU dependent max. size of palette and palette predictor.

Table 5 Lossy Coding efficiency of Proposed Escape Coding Simplification versus JCTVC-S0201 (SCM 2.0 + CU dependent max. size of palette and palette predictor)

Table 6 Lossless Coding efficiency of Proposed Escape Coding Simplification versus JCTVC-S0201 (SCM 2.0 + CU dependent max. size of palette and palette predictor)

# Recommendation

It is recommended to add this simplification into HEVC SCC WD and SCM 3.0, given that performance is well maintained without loss for this simplification (and enable the potential parallel processing at encoder).

Modified HEVC SCC WD is attached.

# References

[1] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

[2] W. Wang, Z. Ma, M. Xu and H. Yu, “Non-CE5: CU dependent color palette maximum size”, JCTVC-S0201, Strasbourg, FR, Oct. 2014.

# 5  Patent rights declarations(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).