| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0151 |
| --- | --- |

| Title: | Non-CE6: 2-D Index Map Coding of Palette Mode in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents a 2-D index map coding method designed for improving the performance of the palette mode in HEVC SCC WD and SCM 2.0. The proposed method has been harmonized with the current palette mode. Its performance reported in this contribution was evaluated under common test conditions with various search range configurations. Experimental results demonstrate BD-Rate gain of 3.3% (IBC-1x4/2D-1x4), 20.2% (IBC-FF/2D-1x4), 14.8% (IBC-2x4/2D-2x4) and 18.4% (IBC-3x5/2D-3x5), over Anchor SCM-2.0+IBC-1x4, for lossy 1080p text graphics content encoded using All intra configuration, respectively. More specifically:

- BD-Rate gain of IBC FF over IBC-1x4 is AI-18.5%, RA-10.0%, LB-7.0%.

- BD-Rate gain of IBC-3x5/2D-3x5 over IBC 1x4 is AI-18.4%, RA-11.8%, LB-9.9%

# Introduction

Color palette mode was recently introduced into HEVC SCC WD and reference software SCM 2.0 and has shown noticeable gains over SCM 1.0. It applies to all CUs in a recursive quad-tree fashion and basically performs the 1-D string search in index map coding to find the matched string represented by RUN.  In this contribution, a 2-D string copy method is proposed to further improve the coding efficiency and has been harmonized with the current index map coding.

To demonstrate the efficiency of proposed 2-D index map coding, local search window is supported, such as

- 1x4 CTU: Window left 3 CTUs plus current CTU

- 2x4 CTU Window: left 3 CTUs + 4 upper CTUs + current CTU

- 3x5 CTU Window: left 4 CTUs + 8 CTUs from above two CTU lines + current CTU

Figure 1 illustrates the local search window defined for 2-D index map coding and intra block copy.

The index map coding of palette mode in the current HEVC SCC draft uses RUN based 1D string copy method. For any give location, in addition to the RUN based 1D string search, a 2D string search with flexible width and height is also performed.  If the area covered by the 2D-match result is bigger than 1D RUN length, the 2D string copy mode is selected and its (mvx, mvy, width, height) is encoded into bit-stream, otherwise, the current index map coding mode is selected and its (RUN) is encoded into bit-stream.

The above algorithm is implemented on top of the SCM 2.0 with JCT-VC S0150 [1] with modifications such as simplified Escape Coding (S0150) and reduced context-coded bins.

For any MxN search window, IBC is configured to use Full search on the last CTU row and Hash search on all remaining CTUs.  Full-Frame IBC is configured to use Full search on 2 CTUs and Hash search on all remaining CTUs.

# Proposed Method

A one-bit context coded “2D_flag” is added to the syntax to indicate if the 2D-string copy mode is selected.

- If 2D_flag = 1, COPY_ABOVE flag is skipped,

  - a one-bit context coded mvx0_flag is added to indicate whether its motion vector mvx is equal to zero; and a one-bit context coded mvy0_flag is added to indicate whether its motion vector mvy is equal to 0.

    - When a 2D block is a single color block, both mvx and mvy are set to 0 and its color index is encoded using TBC method.

  - a one-bit context is used to encode the sign of non-zero mvx, followed by the absolute value of mvx, which is encoded using truncated binary coding (TBC) method.

  - a one-bit context is used to encode the sign of non-zero mvy, followed by the absolute value of mvy, which is encoded using truncated binary coding (TBC) method.

  - both width and height are encoded using TBC method.

- If 2D_flag = 0, COPY_ABOVE flag is encoded using one context (2 contexts are used in default RUN based 1D search)

In a summary, a total of 5 context-coded bins are used in the 2D string copy mode. Since the minimum size of a 2D block is 4x4, the number of context-coded bins is 20 in a 8x8 CU (worst case).

Different schemes for coding mvx/mvy have also been tested. For example, a method of coding mvx/mvy with 3-bin context-based fixed-length was also tested, resulting in 0.1% gain over TBC method, however, a total of 6 extra contexts were used to encode mvx and mvy, which means that a total number of context-coded bins is 44 for a 8x8 CU.

Compared to default RUN based 1D search, this contribution applies the simplified Escape Coding in JCTVC-S150 [1] and also reduces the context of COPY_ABOVE flag signaling from 2 to 1. It provides much clean implementation.

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [2] plus the test condition using restricted search range (i.e., 1x4, 2x4 and 3x5 CTUs) for intra block copy (IBC).

Total 4 Tests are performed during simulation. Tests configuration and Results for lossy and lossless are summarized as follows.

Anchor : SCM 2.0 + IBC 1x4.

Test 1 : IBC 1X4 + 2D 1X4

Test 2 : IBC FF + 2D 1X4

Test3 : IBC 2X4 + 2D 2X4

Test4 : IBC 3X5 + 2D 3X5

Table 1.1 : Lossy coding efficiency for test1

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.3% | -4.1% | -3.9% | -2.1% | -2.8% | -2.6% | -1.7% | -2.1% | -2.0% |
| RGB, text & graphics with motion,720p | -2.8% | -3.5% | -3.2% | -2.6% | -3.2% | -3.0% | -1.0% | -1.3% | -1.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.5% |
| RGB, mixed content, 1080p | -0.6% | -0.7% | -0.6% | -0.5% | -0.6% | -0.7% | -0.5% | -0.8% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.8% | -3.3% | -3.5% | -2.1% | -2.7% | -2.8% | -1.8% | -2.3% | -2.4% |
| YUV, text & graphics with motion,720p | -1.9% | -2.8% | -2.7% | -1.9% | -2.6% | -2.9% | -0.5% | -1.1% | -1.2% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.6% | -0.3% | -0.7% | -0.7% | -0.3% | -0.9% | -0.7% |
| YUV, mixed content, 1080p | -0.6% | -2.7% | -2.1% | -0.6% | -2.7% | -2.1% | -0.9% | -3.2% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% |
| Enc Time[%] | 115% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  | 104% |  |  |

Table 1.2 : Lossless coding efficiency for test1

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.9% | 3.1% | 1.4% | 4.4% | 0.8% | 1.8% | 0.6% | 3.3% | 0.6% | 1.3% | 0.6% | 2.3% |
| RGB, text & graphics with motion,720p | 0.7% | 1.5% | 0.0% | 5.2% | 0.1% | 1.0% | 0.0% | 3.8% | 0.0% | 0.2% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.9% | 3.1% | 0.7% | 5.4% | 0.6% | 2.0% | 0.3% | 4.0% | 0.4% | 1.5% | 0.3% | 2.8% |
| YUV, text & graphics with motion,720p | 0.7% | 1.9% | 0.0% | 6.9% | 0.1% | 1.3% | 0.0% | 5.0% | 0.0% | 0.3% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |  | 106% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 104% |  |  |  | 101% |  |  |  | 103% |  |  |  |

Table 2.1 : Lossy coding efficiency for test2

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -20.2% | -20.5% | -20.5% | -11.2% | -11.3% | -11.3% | -8.3% | -8.2% | -8.3% |
| RGB, text & graphics with motion,720p | -11.5% | -11.4% | -11.4% | -7.9% | -7.9% | -7.9% | -3.2% | -3.1% | -3.0% |
| RGB, mixed content, 1440p | -11.1% | -10.7% | -10.7% | -6.6% | -6.0% | -6.3% | -3.7% | -3.2% | -3.5% |
| RGB, mixed content, 1080p | -14.1% | -13.8% | -13.5% | -9.7% | -9.3% | -9.1% | -3.9% | -3.7% | -3.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -20.9% | -20.8% | -20.8% | -11.0% | -11.1% | -11.1% | -7.7% | -7.7% | -7.8% |
| YUV, text & graphics with motion,720p | -12.2% | -12.3% | -12.1% | -8.3% | -8.2% | -8.4% | -2.9% | -3.0% | -2.8% |
| YUV, mixed content, 1440p | -11.0% | -10.4% | -10.4% | -7.3% | -6.8% | -6.8% | -4.3% | -3.3% | -3.4% |
| YUV, mixed content, 1080p | -15.4% | -15.8% | -15.5% | -10.9% | -11.0% | -11.1% | -5.2% | -4.4% | -3.8% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  | 103% |  |  |

Table 2.2 : Lossless coding efficiency for test2

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 18.1% | 17.0% | 7.8% | 23.7% | 15.4% | 12.5% | 3.9% | 16.8% | 15.4% | 10.4% | 3.3% | 16.9% |
| RGB, text & graphics with motion,720p | 3.3% | 4.7% | 0.5% | 12.4% | 0.8% | 2.7% | 0.2% | 9.5% | 0.5% | 1.2% | 0.2% | 3.7% |
| RGB, mixed content, 1440p | 9.8% | 8.8% | 4.9% | 12.8% | 2.0% | 2.0% | 0.4% | 3.7% | 1.2% | 1.3% | 0.1% | 2.5% |
| RGB, mixed content, 1080p | 8.2% | 8.2% | 8.2% | 8.2% | 1.7% | 1.7% | 1.7% | 1.7% | 0.8% | 0.8% | 0.8% | 0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 18.0% | 17.3% | 8.0% | 24.5% | 14.7% | 12.6% | 4.5% | 17.2% | 14.5% | 10.5% | 3.9% | 16.1% |
| YUV, text & graphics with motion,720p | 3.7% | 5.5% | 0.7% | 15.1% | 0.8% | 3.3% | 0.3% | 11.6% | 0.6% | 1.4% | 0.2% | 4.4% |
| YUV, mixed content, 1440p | 10.3% | 9.5% | 5.5% | 13.5% | 2.1% | 2.3% | 0.5% | 4.1% | 1.3% | 1.4% | 0.1% | 2.7% |
| YUV, mixed content, 1080p | 9.2% | 9.2% | 9.2% | 9.2% | 1.9% | 1.9% | 1.9% | 1.9% | 0.9% | 0.9% | 0.9% | 0.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116% |  |  |  | 105% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |

Table 3.1 : Lossy coding efficiency for test3

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -14.8% | -15.5% | -15.3% | -9.2% | -9.7% | -9.5% | -7.6% | -7.9% | -7.7% |
| RGB, text & graphics with motion,720p | -8.8% | -9.1% | -9.0% | -6.2% | -6.5% | -6.4% | -2.1% | -2.4% | -2.3% |
| RGB, mixed content, 1440p | -5.5% | -5.2% | -5.2% | -3.8% | -3.4% | -3.5% | -2.2% | -2.0% | -2.0% |
| RGB, mixed content, 1080p | -7.4% | -7.2% | -7.1% | -5.1% | -5.0% | -5.0% | -2.4% | -2.5% | -2.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -15.0% | -15.0% | -15.0% | -9.0% | -9.1% | -9.2% | -7.2% | -7.4% | -7.5% |
| YUV, text & graphics with motion,720p | -8.6% | -9.1% | -9.0% | -5.9% | -6.2% | -6.4% | -1.8% | -2.2% | -2.2% |
| YUV, mixed content, 1440p | -6.3% | -5.8% | -5.8% | -4.4% | -4.0% | -4.1% | -2.7% | -2.1% | -2.1% |
| YUV, mixed content, 1080p | -8.7% | -9.4% | -8.9% | -6.1% | -6.5% | -6.4% | -2.9% | -2.9% | -2.3% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 130% |  |  | 110% |  |  | 107% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  | 103% |  |  |

Table 3.2 : Lossless coding efficiency for test3

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 12.1% | 11.5% | 7.1% | 14.1% | 10.7% | 9.0% | 3.7% | 11.6% | 10.8% | 8.4% | 3.2% | 11.7% |
| RGB, text & graphics with motion,720p | 2.5% | 3.8% | 0.2% | 10.6% | 0.5% | 2.2% | 0.1% | 7.9% | 0.4% | 0.8% | 0.1% | 2.5% |
| RGB, mixed content, 1440p | 1.7% | 2.0% | 1.0% | 3.0% | 0.5% | 0.5% | 0.2% | 0.8% | 0.4% | 0.4% | 0.1% | 0.8% |
| RGB, mixed content, 1080p | 3.0% | 3.0% | 3.0% | 3.0% | 0.6% | 0.6% | 0.6% | 0.6% | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 12.2% | 11.7% | 6.8% | 14.5% | 10.2% | 9.0% | 3.7% | 12.4% | 10.1% | 8.3% | 3.2% | 11.1% |
| YUV, text & graphics with motion,720p | 2.8% | 4.5% | 0.3% | 13.1% | 0.6% | 2.7% | 0.2% | 9.8% | 0.4% | 0.9% | 0.1% | 3.1% |
| YUV, mixed content, 1440p | 2.0% | 2.3% | 1.2% | 3.4% | 0.6% | 0.6% | 0.3% | 1.0% | 0.5% | 0.5% | 0.1% | 1.0% |
| YUV, mixed content, 1080p | 3.7% | 3.7% | 3.7% | 3.7% | 0.8% | 0.8% | 0.8% | 0.8% | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 160% |  |  |  | 115% |  |  |  | 111% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  | 101% |  |  |  |

Table 4.1 : Lossy coding efficiency for test4

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -18.4% | -19.0% | -18.9% | -11.8% | -12.3% | -12.1% | -9.9% | -10.2% | -10.1% |
| RGB, text & graphics with motion,720p | -11.0% | -11.4% | -11.2% | -7.7% | -8.1% | -7.9% | -3.0% | -3.3% | -3.3% |
| RGB, mixed content, 1440p | -7.5% | -7.1% | -7.1% | -4.8% | -4.3% | -4.4% | -2.6% | -2.3% | -2.6% |
| RGB, mixed content, 1080p | -11.9% | -11.4% | -11.3% | -8.1% | -7.6% | -7.7% | -3.5% | -3.0% | -2.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -18.7% | -18.7% | -18.7% | -11.7% | -11.7% | -11.7% | -9.5% | -9.6% | -9.7% |
| YUV, text & graphics with motion,720p | -11.2% | -11.6% | -11.6% | -7.6% | -7.8% | -8.1% | -2.6% | -2.8% | -2.5% |
| YUV, mixed content, 1440p | -8.2% | -7.7% | -7.5% | -5.5% | -5.0% | -4.9% | -3.1% | -2.6% | -2.1% |
| YUV, mixed content, 1080p | -13.3% | -13.6% | -13.1% | -9.3% | -9.3% | -9.2% | -4.1% | -3.4% | -2.7% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 158% |  |  | 116% |  |  | 112% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  | 101% |  |  |

Table 4.2 : Lossless coding efficiency for test4

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 15.0% | 14.3% | 8.8% | 17.4% | 13.7% | 11.5% | 5.1% | 14.7% | 13.7% | 10.8% | 4.5% | 14.8% |
| RGB, text & graphics with motion,720p | 3.2% | 4.7% | 0.5% | 12.7% | 0.7% | 2.7% | 0.2% | 9.6% | 0.5% | 1.1% | 0.2% | 3.6% |
| RGB, mixed content, 1440p | 3.6% | 3.7% | 3.4% | 3.9% | 0.8% | 0.9% | 0.3% | 1.4% | 0.6% | 0.6% | 0.1% | 1.2% |
| RGB, mixed content, 1080p | 5.2% | 5.2% | 5.2% | 5.2% | 1.0% | 1.0% | 1.0% | 1.0% | 0.5% | 0.5% | 0.5% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 15.2% | 14.6% | 8.5% | 17.8% | 13.0% | 11.6% | 5.1% | 15.4% | 12.9% | 10.8% | 4.6% | 14.1% |
| YUV, text & graphics with motion,720p | 3.5% | 5.5% | 0.6% | 15.6% | 0.7% | 3.3% | 0.3% | 11.9% | 0.5% | 1.3% | 0.2% | 4.5% |
| YUV, mixed content, 1440p | 4.0% | 4.1% | 3.8% | 4.4% | 0.9% | 1.0% | 0.4% | 1.6% | 0.7% | 0.7% | 0.1% | 1.4% |
| YUV, mixed content, 1080p | 6.1% | 6.1% | 6.1% | 6.1% | 1.3% | 1.3% | 1.3% | 1.3% | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 216% |  |  |  | 126% |  |  |  | 120% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  | 100% |  |  |  |

# Recommendation

It is recommended to add 2-D index map coding method into HEVC SCC WD and SCM 3.0.

Modified HEVC SCC WD is attached.

# References

[1] M. Xu, Z. Ma, W. Wang, H. Yu, “Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC”, Strasbourg, FR, Oct. 2014

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).