| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0152 |
| --- | --- |

| Title: | Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents a parallel processing method for index map coding using index coding group. Basically, this method suggests that, to improve the overall system performance, coding of color index map may be done on smaller subgroups, instead of on the original color index map. In particular, it is suggested that this method might be used for 8x8 CU, given that 8x8 CU is the bottleneck for the overall throughput. Experimental results demonstrate negligible performance loss (i.e., averaged 0.2%/0.1%/0.0% lossy BD-Rate loss for all content encoded using All intra, Random Access and Low-delay B configurations), but with potential 4x throughput improvement for 8x8 index map processing. It is also observed small gains for lossless coding scenarios (i.e., 0.5% bit rate reduction for 1080p text/graphics content in RGB format).

# Proposed Method

Color palette mode was recently introduced into HEVC SCC WD and reference software SCM 2.0 and has shown noticeable gains over SCM 1.0. It applies to all CUs in a recursive quad-tree fashion.  In practical encoder system, 8x8 CU typically has very tight clock-cycle budget.  Therefore, this contribution proposes an index coding grouping method to enable the parallel processing.

More specifically, the proposed index coding group includes

- splitting an 8x8 index map into 4 2x8 (shown in Figure 1) parallel groups

- maintaining COPY_ABOVE across different groups

- adding an additional flag for each group at the first index position to indicate whether the RUN equals to the full size of current group (i.e., single color or fully copied from above line), i.e., “full-RUN” flag or ICG flag.

In addition, to further simplify the index map coding, ICG in this proposal also includes (noted as Simple1D palette)

- simplified Escape Coding proposed in JCTVC-S0150

- reduced context for index coding mode (from 2 to 1)

  - In SCM 2.0, it has dependency on previous mode in upper line to code index coding mode, i.e., 2 contexts are used.

Figure 1: 8x8 Index Map as of 4 2x8 Groups for Parallel Processing at encoder

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [1] (w/ full frame intra block copy). Results for lossy and lossless are summarized in Table 1 to 4.

Table 1 Lossy Coding Efficiency of Proposed ICG vs. SCM 2.0

Table 2 Lossless Coding Efficiency of Proposed ICG vs. SCM 2.0

An additional test has also been performed where the ICG flag or “full-RUN” flag was removed while keeping other modifications unchanged.

Table 3 Lossy Coding Efficiency of Proposed ICG (w/ flag removed) vs. SCM 2.0

Table 4 Lossless Coding Efficiency of Proposed ICG (w/ flag removed) vs. SCM 2.0

Besides, another set simulation is also performed to show the performance difference between default index map coding and simplified index map coding using JCTVC-S0150 and with one context removed. Experimental results demonstate negligible loss (0.0% on average) by implementing this simple1D palette. Note that it even shows gains for lossless encoding configuration by this simplification.

Table 5 Lossy Coding Efficiency of Proposed Simple1D Palette vs. SCM 2.0

Table 6 Lossless Coding Efficiency of Proposed Simple1D Palette vs. SCM 2.0

# Recommendation

It is recommended to add this feature into HEVC SCC WD and SCM 3.0 to enable the parallel processing of 8x8 CU for palette mode so as to improve the throughput.

Modified HEVC SCC WD is attached.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).