| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0152 |
| --- | --- |

| Title: | Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents a parallel processing method for index map coding using index coding group. Basically, this method suggests that, to improve the overall system performance, coding of color index map may be done on smaller subgroups, instead of on the original color index map. In particular, it is suggested that this method might be used for 8x8 CU, given that 8x8 CU is the bottleneck for the overall throughput. Experimental results demonstrate negligible performance loss (i.e., 0.4% lossy BD-Rate loss for 1080p text graphics content encoded using All intra configuration), but with potential 4x throughput improvement for 8x8 index map processing.

# Proposed Method

Color palette mode was recently introduced into HEVC SCC WD and reference software SCM 2.0 and has shown noticeable gains over SCM 1.0. It applies to all CUs in a recursive quad-tree fashion.  In practical encoder system, 8x8 CU typically has very tight clock-cycle budget.  Therefore, this contribution proposes an index coding grouping method to enable the parallel processing.

More specifically, the proposed index coding group includes

- splitting an 8x8 index map into 4 2x8 (shown in Figure 1) parallel groups

- maintaining COPY_ABOVE across different groups

- adding an additional flag for each group at the first index position to indicate whether the RUN equals to the full size of current group (i.e., single color or fully copied from above line).

In addition, to further simplify the index map coding, ICG in this proposal also includes

- simplified Escape Coding proposed in JCTVC-S0150

- reduced context for index coding mode (from 2 to 1)

Figure 1: 8x8 Index Map as of 4 2x8 Groups

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [1] (w/ full frame intra block copy). Results for lossy and lossless are summarized in Table 1 to 4.

Table 1 Lossy Coding Efficiency of Proposed ICG vs. SCM 2.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.4% | 0.3% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.6% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.4% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Lossy Coding Efficiency of Proposed ICG (w/ flag removed) vs. SCM 2.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.7% |
| RGB, text & graphics with motion,720p | 0.6% | 0.6% | 0.6% |
| RGB, mixed content, 1440p | 0.2% | 0.4% | 0.3% |
| RGB, mixed content, 1080p | 0.2% | 0.4% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 1.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.7% | 0.8% | 1.0% |
| YUV, mixed content, 1440p | 0.4% | 0.8% | 0.8% |
| YUV, mixed content, 1080p | 0.5% | 0.6% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

An additional test has also been performed where the ICG flag was removed while keeping other modifications unchanged.

# Recommendation

It is recommended to add this feature into HEVC SCC WD and SCM 3.0 to enable the parallel processing of 8x8 CU for palette mode so as to improve the throughput.

Modified HEVC SCC WD is attached.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).