| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0153 |
| --- | --- |

| Title: | CE6: Test D.1 Run-length coding for reuse flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Vadim Seregin, Rajan Joshi, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is related to the signaling of the palette reused flags, indicating which palette entries are predicted from the palette predictor formed from the palette entries of the previous coded blocks. It is proposed to replace the current method with the run-length coding in order to signal which entries are predicted. Reportedly, it provides 0.2% and 0.3% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration, respectively. Additionally, increased palette predictor length to 128 and 256 is tested. Proposed method with the palette predictor length of 128 provides 0.7% luma BD-rate reduction for the same video sequence classes.

# Introduction

Palette entries of the current block can be predicted from the palette predictor consisting of the palette entries of the previously palette coded blocks. The default size of the palette predictor is 64. The prediction can be represented with the binary vector of the size equal to the palette predictor size, where 1 indicates that the entry is reused from the predictor, and 0 otherwise.

In the current method, the binary vector is divided into groups of 4 flags. For each group, two bins are signalled to indicate:

- whether the group contains non-zero flag,

- whether this is the last group containing non-zero flag.

If the first bin indicates that there is at least one non-zero flag in the group, then four flags of the group are directly signaled into the bitstream.

# Proposed run-length coding

This contribution proposes to use run-length coding method to code zero flags in the binary vector, initially proposed in JCTVC-R0228 and consisting of the following steps:

- run-length value equal to 1 indicates end of prediction

- the end of prediction is not signaled for the last 1 in the binary vector

- the number of preceding zero elements is coded for every 1 in the binary vector

- if the number of zero elements is greater than 0, the number plus one is signaled, due to the escape value of 1

- run-length value is coded using 0-order Exponential Golomb code

For example, let consider a binary vector {1100100010000}.

The signaled run-length values are 0-0-3-4-1.

# Simulation results

Run-length coding for the palette prediction was implemented on top of SCM2.0 and tested based on common test conditions JCTVC-R1015. The simulation was performed on the heterogeneous Linux cluster, so the encoding and decoding running time can be inaccurate.  The results for the lossy configuration are summarized in the Table 1, and the complete results can be found in the accompanying Excel tables.

Table 1. Proposed method with the default palette predictor length (64).

Additionally, the palette predictor with the increased length of 128 and 256 is tested for both SCM2.0 anchor and proposed method. The AI results are summarized in the Table 2 and 3.

Table 2. Palette predictor length 128 with anchor (left) and proposed method (right).

Table 3. Palette predictor length 256 with anchor (left) and proposed method (right).

# Conclusion

According to the results, proposed run-length coding is more efficient and it noticeably better for larger palette predictor, where the anchor’s method provides some losses. It is proposed to adopt run-length coding method for the palette prediction signalling into the next version of screen content working draft and software model, and consider increasing palette predictor length to 128.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

| numPredPreviousPalette = 0 |  |
| --- | --- |
| for( i = 0; i < previousPaletteSize && (i ==0 ¦¦ palette_prediction_run[ i–1 ] != 1) && numPredPreviousPalette < max_palette_size; i++ ) { |  |
| palette_prediction_run[ i ] | ae(v) |
| if( palette_prediction_run[ i ] != 1 ) { |  |
| if ( palette_prediction_run[ i ] > 1 ) |  |
| i += palette_prediction_run[ i ] – 1 |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ idx ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| for( i = 0; i < previousPaletteStuffingSize; i++ ) |  |
| previous_palette_entry_flag[ i ] = 0 |  |
| palette_last_group = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize && !palette_last_group && numPredPreviousPalette < max_palette_size; i++ ) { [Ed. YY: max_palette_size probably needs to be signalled] |  |
| lastPossibleGroupFlag = ( i + 4 >= previousPaletteStuffingSize ) |  |
| lastIdx = min( i + 4, previousPaletteStuffingSize ) − 1 |  |
| if( i > 3 && !lastPossibleGroupFlag ) |  |
| palette_all_zeros_in_group | ae(v) |
| else |  |
| palette_all_zeros_in_group = 0 |  |
| if( palette_all_zeros_in_group ) |  |
| i += 4 |  |
| else { |  |
| numOnesInGroup = 0 |  |
| for( idx = i; idx <= lastIdx && numPredPreviousPalette < max_palette_size; idx++ ) { |  |
| if ( idx = = lastIdx && numOnesInGroup = = 0 ) |  |
| previous_palette_entry_flag[ idx ] = 1 |  |
| else |  |
| previous_palette_entry_flag[ idx ] | ae(v) |
| if ( previous_palette_entry_flag[ idx ] ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ idx ] |  |
| numPredPreviousPalette++ |  |
| numOnesInGroup++ |  |
| } |  |
| } |  |
| if( !palette_all_zeros_in_group && !lastPossibleGroupFlag && numPredPreviousPalette < max_palette_size ) |  |
| palette_last_group | ae(v) |
| } |  |

palette_prediction_run[ i ] specifies the difference between indices of the current reused and the next reused palette entries from the previous palette previousPaletteEntries with the following exceptions: palette_prediction_run equal to 0 indicates that the difference between the indices of the current and next reused entries is 1, and palette_prediction_run equals to 1 indicates that no more entries from the previous palette previousPaletteEntries are reused.

palette_all_zeros_in_group equal to 1 specifies that all previous_palette_entry_flag in a group of length 4 are equal to zero. If all_zeros_in_group is not present it is infered to be equal to 0.

palette_last_group equal to 1 specifies that the group of 4 previous_palette_entry_flag is the last group and after this group there is no more previous_palette_entry_flag signalling. If last_group is not present it is infered to be equal to 0.

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag[ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

Table 9-1 – Syntax elements and associated binarizations

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_share_flag | FL | cMax = 1 |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx = = 0 ? ( (1<<BitDepthY) − 1 ) : ( (1<<BitDepthC) − 1 ) |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | palette_escape_val | Error: Reference source not found | cIdx, qP |
|  | escape_val_present_flag | FL | cMax = 1 |
|  | palette_run_type_flag | FL | cMax = 1 |
|  | palette_index | TB | cMax = adjustedIndexMax |
|  | palette_run | Error: Reference source not found | - |
|  | palette_all_zeros_in_group | FL | cMax = 1 |
|  | palette_last_group | FL | cMax = 1 |
|  | palette_prediction_run | EG0 | - |

Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins

| … |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| previous_palette_entry_flag | bypass | na | na | na | na | Na |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_transpose_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | Na |
| palette_escape_val | bypass | bypass | bypass | bypass | bypass | bypass |
| escape_val_present_flag | bypass | na | na | na | na | na |
| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | Na | na | Na | Na |
| palette_index | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_run | 0 | 1 | 2 | bypass | bypass | bypass |
| palette_all_zeros_in_group | bypass | na | na | na | na | na |
| palette_last_group | bypass | na | na | na | na | na |
| palette_prediction_run | bypass | bypass | bypass | bypass | bypass | bypass |
| … |  |  |  |  |  |  |