| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0154 |
| --- | --- |

| Title: | CE6: Test B.1 Context coded CU-level escape colour flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Vadim Seregin, Rajan Joshi, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to use a context coding for CU-level escape flag. Reportedly, it provides 0.1% and 0.2% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration, respectively.

# Introduction

In the current palette mode, a colour triplet (Y, U, and V) can be directly signalled with the so called escape pixel mode. The presence of the escape pixels in the block is identified by the CU-level escape flag palette_escape_val_present_flag coded in bypass mode.

# Proposal and results

In this contribution, it is proposed to use a context coded bin to signal CU-level escape flag as was initially proposed in JCTVC-R0066. There are two configurations for context derivation:

- Depending on the palette size, two contexts are defined by checking whether palette_size < max_palette_size

- Single context

Context coding for the CU-level escape was implemented on top of SCM2.0 and tested based on common test conditions JCTVC-R1015. The simulation was performed on the heterogeneous Linux cluster, so encoding and decoding running time can be inaccurate.

The simulation results for both methods are closed, so only the results for second method for the lossy configuration are summarized in the next table, the complete data including the first method can be found in the accompanying Excel tables.

# Conclusion

Simulation results reveal 0.2% luma BD-rate gain using context coding bin which is only one per coding unit in the second method. It is proposed to adopt a single context coding into the next version of screen content working draft and software model.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

Table 9-1 – Assignment of ctxInc to syntax elements with context coded bins

| … |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| palette_escape_val_present_flag | bypass 0 | na | na | na | na | na |
| … |  |  |  |  |  |  |

Table 9-XX – Values of initValue for palette_escape_val_present_flag

| Initialization variable | ctxIdx of palette_escape_val_present_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |