| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0155 |
| --- | --- |

| Title: | Non-CE6: Palette copy above mode for the first row |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Marta Karczewicz, Wei Pu, Rajan Joshi, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to extend the use of the palette copy above mode for the very first row in the block. It is achieved by assigning the default palette index for the samples outside of the block. Reportedly, it provides 0.6% and 0.8% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration, respectively.

# Introduction

In the current palette, there are two modes to code a sample: index mode and copy above mode. One bin is signalled indicating which mode is used. In the current design, the copy above mode is not used for the very first row in the block, in this case the mode bin is not signalled and the mode is inferred equal to the run mode.

# Proposal and results

In this contribution, it is proposed to enable copy above mode for the very first row in a block, a default palette index equal to 0 is assigned for the palette indices of the samples located outside of the block, and the following changes have been made:

- enable mode signalling for the very first row, additional one context is used to code the mode bin

- extend the palette index adjustment to the first row

Additionally, the following adjustment is done to the palette index when the left palette index is not escape and is equal to the above palette index. In this case, the first palette index cannot be coded with the copy mode, since it would be included into the previous run mode.

So, the copy mode is never signalled and is redundant. In this contribution, the copy mode is signalled for this case, but the first palette index is assigned equal to 0 if the left palette mode is greater than 0, or 1 otherwise. This palette index adjustment is not needed during parsing and can be done later during decoding.

Copy for the first row and the palette index adjustment for the copy mode was implemented on top of SCM2.0 and tested based on common test conditions JCTVC-R1015. Additionally, the encoder modification for the copy and run mode selection from the JCTVC-S0156 test2 was enabled.

The simulation was performed on the heterogeneous Linux cluster, so encoding and decoding running time can be inaccurate.

The simulation results are summarised in the next tables and a complete data can be found in the accompanying Excel tables.

Table 1. Enabling copy mode for the first row (left) and with encoder modification (right)

Table 2. Enabling copy mode for the first row with the index adjustment for the equal palette indices from the left and above (left) and with encoder modification (right).

# Conclusion

According to the results, the copy mode with index adjustment provides 0.6% and with encoder modification 0.8% luma BD-rate improvement for YUV test and graphics 1080p class in lossy configuration. It is proposed to adopt this method into the next version of the screen content working draft and software model.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

The changes related to enabling copy mode for the first row are highlighted with yellow colour, and the changes related to the additional index adjustment are highlighted with green.

| while( scanPos < nCbS * nCbS ) { |  |
| --- | --- |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xC_prev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| yC_prev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( scanPos > = nCbS && scanPos = = 0 ¦¦ palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE ) |  |
| palette_mode[ xC ][ yC ] | ae(v) |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| adjustedIndexMax = indexMax |  |
| adjustedRefIndex = indexMax + 1 |  |
| } |  |
| if( scanPos == 0) { |  |
| adjustedIndexMax −= 1 |  |
| adjustedRefIndex = 0 |  |
| } |  |
| if( scanPos > 0 && palette_mode[xC_prev][yC_prev] ! = ESCAPE ) { |  |
| if( palette_mode[xC_prev][yC_prev] = = INDEX ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = paletteMap[ xC_prev ][ yC_prev ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[ xC_prev ][ yC_prev ] = = COPY_ABOVE && (scanPos < nCbS ¦¦ ( scanPos > = nCbS && palette_mode[ xC ][ yC − 1 ] ! = ESCAPE ) ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = scanPos < nCbS ? 0 : paletteMap[ xC ][ yC − 1 ] |  |
| } |  |
| } |  |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| if( adjustedIndexMax > 0 ) |  |
| palette_index | ae(v) |
| if( palette_index > = adjustedRefIndex ) |  |
| palette_index++ |  |
| if( palette_index = = palette_size ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| palette_mode[ xC ][ yC ] = ESCAPE |  |
| scanPos++ |  |
| } |  |
| } |  |
| if( palette_mode[xC][yC] ! = ESCAPE ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| runMode = palette_mode[ xC ][ yC ] |  |
| while ( runPos < = palette_run ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_mode[ xC ][ yC ] = = INDEX ) { |  |
| palette_mode[ xC ][ yC ] = INDEX |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } else { |  |
| palette_mode[ xC ][ yC ] = COPY_ABOVE |  |
| paletteMap[ xC ][ yC ] = scanPos < nCbS ? 0 : paletteMap[ xC ][ y − 1 ] |  |
| if( scanPos > 0 && palette_mode[ xC – 1 ][ yC ] = = INDEX && palette_mode[ xC – 1 ][ yC ] = = palette_mode[ xC ][ yC – 1] ) |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC ] ? 0 : 1 |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |

Table 9-1 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| palette_mode | scanPos < nCbS ? 2 : (palette_mode[ xC – 1 ][ yC ] = = INDEX ? 0 : 1) | na | na | na | na | na |

Table 9-XX – Values of initValue for ctxIdx of palette_mode

| Initialization variable | ctxIdx of palette_mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |