| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0159 |
| --- | --- |

| Title: | CE 7 Test 2: Constrained 1-D String based Index Coding for Palette Index Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng. Xu, Zhan Ma, Wei Wang, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents a constrained1-D string-based index map coding method on top of current palette mode adopted in HEVC SCC WD and SCM 2.0 to improve the coding efficiency [1, 2].  It shows -1.6% and -2.5% lossy BD-Rate improvements, and 0.9% and 1.6% lossless bit rate reduction on top of current SCM 2.0 with full intra block copy, and constrained 1x4 intra block copy for 1080p text and graphics content encoded using All Intra configuration.

# Proposed method

This test evaluates the color index map coding proposed in [1] using constrained 1D string match based index compression. This constrained 1-D string match is performed within current CU with constrained matched distance and length is inferred as the CU width.

Technical Details:

- 1-D string search is constrained within current CU;

- Distance is constrained at significant position only;

- Length is inferred as the CUWidth without signaling;

- Representation is on the line-by-line basis, for the line with one unmatched index, the entire line is coded as “unmatch”.

  - (line_unmatched_flag = 1, unmatched_idx[array])

  - (line_unmatched_flag = 0, near_neighbor_flag)

- near_neighbor_flag is used to indicate whether the distance is 1 or CUWidth.

- Unmatched_idx is coded using prediction from neighbor indexes and previous Transition Copy. If a match is found, the match index is coded; otherwise, the Unmatched_idx itself is coded.

- CABAC

  - line_unmatched_flag is coded using context mode

  - Unmatched_idx is coded using bypass mode

  - Near_neighbor_flag is context coded

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [3] plus the test condition using restricted search range (i.e., 1x4 CTUs). Results for lossy and lossless are summarized from Table 1 to 4.

Table 1 Lossy coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.6% | -1.6% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.6% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -0.6% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.1% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -1.0% | -0.9% |
| YUV, text & graphics with motion,720p | -0.7% | -1.0% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.4% |
| YUV, mixed content, 1080p | 0.0% | -0.8% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% |
| RGB, text & graphics with motion,720p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.8% | -0.7% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -1.0% |
| YUV, mixed content, 1440p | -0.4% | -0.8% | -0.7% |
| YUV, mixed content, 1080p | -0.4% | -1.1% | -1.3% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 133% |  |  |

Table 2 Lossless coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.0% | 0.4% | 1.4% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.1% | 0.2% | 1.6% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.1% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 131% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.2% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.1% | 1.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 120% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 0.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |

Table 3 Lossy coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.5% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -1.6% | -1.7% | -1.6% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.4% | -2.5% | -2.5% |
| YUV, text & graphics with motion,720p | -1.2% | -1.4% | -1.5% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 129% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.1% | -1.1% |
| RGB, text & graphics with motion,720p | -1.6% | -1.6% | -1.5% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -1.1% | -1.2% | -1.4% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.6% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.3% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.3% | -1.0% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 134% |  |  |

Table 4 Lossless coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.7% | 0.6% | 2.6% |
| RGB, text & graphics with motion,720p | 0.5% | 0.8% | 0.1% | 2.4% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.6% | 1.8% | 0.3% | 3.1% |
| YUV, text & graphics with motion,720p | 0.5% | 0.9% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |  |
| Dec Time[%] | 120% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.9% | 0.3% | 1.8% |
| RGB, text & graphics with motion,720p | 0.1% | 0.5% | 0.0% | 1.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.6% | 0.0% | 2.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 117% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 1.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 121% |  |  |  |

# Recommendation

It is recommended to further study the constrained 1-D string based index coding method in a potential CE.

# Reference

[1] Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu, “Non-SCCE3: Improvements for SCCE3 Test C3”, Doc. JCTVC R0304, Sapporo, JP, July 2014.

[2] Z. Ma, Y.-W. Huang, “Description of Core Experiment 7 (CE7): String Matching for Palette Index Coding”, Doc. JCTVC R1107, Sapporo, JP, July 2014.

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).