| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0160 |
| --- | --- |

| Title: | CE 7 Test 3: Hybrid 1-D/2-D String-based Index Coding for Palette Index Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng. Xu, Zhan Ma, Wei Wang, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

This contribution presents a hybrid1-D/2-D string-based index map coding method on top of current palette mode adopted in HEVC SCC WD and SCM 2.0 to improve the coding efficiency [1, 2, 3].  It shows -1.9% and -3.4% lossy BD-Rate improvements, and 1.1% and 2.5% lossless bit rate reduction on top of current SCM 2.0 with full intra block copy, and constrained 1x4 intra block copy for 1080p text and graphics content encoded using All Intra configuration.

# Proposed method

This test evaluates the color index map coding proposed in [1, 2, 3] using hybrid 1-D/2-D string match based index compression. 1-D string match is performed within current CU, while hybrid 1-D/2-D search can be extended to the left 3 CTUs.

Technical Details:

- 1-D string search is constrained within current CU

- 2-D string search is extended to left 3 CTUs

- String search is represented by pairs, i.e.,

  - (unmatched_flag = 1, unmatched_idx)

  - (unmatched_flag = 0, match_type)

  - (match_type = 1, 2d_matched_distance, 2d_matched_width, 2d_matched_height)

  - (match_type = 0, 1d_matched_distance, 1d_matched_length)

- 1d_matched_distance is signaled using significant distance and non-significant distance which is differentiated by signaling the significant_distance_flag.

  - When significant_distance_flag is true, another near_neighbor_flag is used to indicate whether the distance is 1 or CUWidth.

  - When significant_distance_flag is false, distance is signaled explicitly using CABAC coded bins.

- 1d_matched_length is signaled by differentiating the significant and non-significant lengths using significant_length_flag;

  - When matched length is less than CUWidth, significant_length_flag is true;

- 2d_matched_distance is signaled using the difference between the starting location of search window and the reference match location, in raster scan order.

- 2d_matched_length is signaled by converting 2d_matched_width and 2d_matched_height to

2d_matched_length =

(2d_matched_width - MIN_2D_WIDTH )+

( 2d_matched_height - MIN_2D_HEIGHT )*(1 + minw - MIN_2D_WIDTH)

Where minw is the width of the constraint search windows for current location

- CABAC

  - Unmatched_flag is coded using context mode

  - Unmatched_idx is coded using bypass mode

  - Significant_distance_flag is context coded

  - Near_neighbor_flag is context coded

  - Non_significant_distance is context coded for each bin, which is binarized using fixed length code.

  - significant_length_flag is context coded.

  - Significant_length is context coded for each bin, which is binarized using fixed length code.

  - Non_significant_length is context coded for each bin, which is binarized using fixed length code.

  - 2d_matched_distance is context coded for each bin, which is binarized using fixed length code.

  - 2d_matched_length is context coded for each bin, which is binarized using fixed length code.

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [4] plus the test condition using restricted search range (i.e., 1x4 CTUs). Results for lossy and lossless are summarized from Table 1 to 4.

Table 1 Lossy coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.3% | -2.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.5% | -1.5% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -1.2% | -1.5% | -1.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.7% | -2.2% | -2.1% |
| YUV, text & graphics with motion,720p | -0.8% | -1.3% | -1.2% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -1.1% | -3.0% | -2.5% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 124% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.5% | -1.5% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.9% | -1.3% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -0.8% | -1.3% | -1.3% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.8% | -3.1% | -2.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.4% | -1.3% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -1.1% | -0.9% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.5% | -1.5% |
| YUV, text & graphics with motion,720p | -0.7% | -0.8% | -1.3% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.3% |
| YUV, mixed content, 1080p | -1.1% | -3.1% | -2.6% |
| YUV, Animation, 720p | 0.0% | -0.5% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 133% |  |  |

Table 2 Lossless coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.1% | 1.3% | 0.3% | 1.9% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 1.3% | 0.1% | 2.0% |
| YUV, text & graphics with motion,720p | 0.3% | 0.5% | 0.1% | 1.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 125% |  |  |  |
| Dec Time[%] | 115% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.7% | 0.1% | 1.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.8% | 0.1% | 1.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.1% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.5% | 0.0% | 0.9% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 122% |  |  |  |

Table 3 Lossy coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.4% | -4.2% | -4.0% |
| RGB, text & graphics with motion,720p | -3.2% | -3.9% | -3.7% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.8% | -0.9% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.9% | -3.7% | -3.7% |
| YUV, text & graphics with motion,720p | -2.1% | -2.9% | -2.9% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.6% |
| YUV, mixed content, 1080p | -0.6% | -2.7% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 122% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.0% | -2.7% | -2.5% |
| RGB, text & graphics with motion,720p | -3.0% | -3.6% | -3.5% |
| RGB, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.7% | -0.9% | -0.8% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.9% | -2.7% | -2.7% |
| YUV, text & graphics with motion,720p | -2.1% | -2.8% | -3.2% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | -0.5% | -2.6% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.5% | -2.0% | -1.8% |
| RGB, text & graphics with motion,720p | -1.3% | -1.7% | -1.6% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -0.6% | -0.9% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.4% | -2.2% | -2.2% |
| YUV, text & graphics with motion,720p | -0.8% | -1.6% | -1.7% |
| YUV, mixed content, 1440p | -0.4% | -0.9% | -0.8% |
| YUV, mixed content, 1080p | -0.6% | -2.6% | -1.6% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 133% |  |  |

Table 4 Lossless coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.5% | 2.7% | 0.6% | 4.7% |
| RGB, text & graphics with motion,720p | 0.7% | 1.6% | 0.1% | 5.5% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.7% | 2.9% | 0.2% | 5.8% |
| YUV, text & graphics with motion,720p | 0.8% | 2.0% | 0.1% | 7.2% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 1.5% | 0.2% | 3.3% |
| RGB, text & graphics with motion,720p | 0.1% | 1.1% | 0.0% | 4.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 1.8% | 0.1% | 4.0% |
| YUV, text & graphics with motion,720p | 0.1% | 1.4% | 0.0% | 5.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 116% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 1.2% | 0.1% | 2.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.0% | 1.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 121% |  |  |  |

# Recommendation

It is recommended to further study the hybrid 1-D/2-D string-based index coding method in a potential CE.

# Reference

[1] Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu, “Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer”, JCTVC R0268, Sapporo, JP, July 2014.

[2] Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu, “Non-SCCE3: Improvements for SCCE3 Test C3”, Doc. JCTVC R0304, Sapporo, JP, July 2014.

[3] Z. Ma, Y.-W. Huang, “Description of Core Experiment 7 (CE7): String Matching for Palette Index Coding”, Doc. JCTVC R1107, Sapporo, JP, July 2014.

[4] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).