| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0161 |
| --- | --- |

| Title: | CE 10: Hybrid 1-D/2-D Intra string search for HEVC screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Zhan Ma, Meng Xu, H. Yu 2330 Central Expy, Santa Clara, USA | Tel: Email: | +1-408-330-5142 m.xu@huawei.com |
| Source: | Huawei USA R&D |  |  |

_____________________________

# Abstract

It is proposed to perform the 1-D and 2-D string search on pixels applied in JCTVC-R0145 [1, 2]. Several tests are arranged in CE activities to demonstrate the coding efficiency of proposed solution [3].

# Description

This contribution presents intra string copy:1-D and 2-D string search is conducted upon the reconstructed pixels to the left of current CTU. It uses 3 left CTUs in this proposal.  Residuals between matched string and original string are encoded using HEVC residual engine without change.

# Simulations

Experiments are performed on top of the SCM 2.0 with default settings using common test condition [4] plus the test condition using restricted search range (i.e., 1x4 CTUs).

### Test 2: Matching mode

Table 1 Lossy coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.6% | 0.6% | 0.6% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.2% | -0.2% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.2% | 0.1% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | -0.5% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.6% | -0.4% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 189% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.3% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.6% | -0.2% |
| YUV, mixed content, 1080p | 0.4% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 194% |  |  |

Table 2 Lossless coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.9% | 0.2% | 1.3% |
| RGB, text & graphics with motion,720p | 0.4% | 0.2% | -0.7% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.4% | 0.3% |
| YUV, text & graphics with motion,720p | 0.2% | -0.1% | -1.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 118% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.0% | 0.5% |
| RGB, text & graphics with motion,720p | 0.1% | -0.1% | -0.7% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.2% | -0.4% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | -0.2% | -1.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 164% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.2% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.7% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.7% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.7% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 162% |  |  |  |

Table 3 Lossy coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.6% | -0.6% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.6% |
| YUV, text & graphics with motion,720p | 0.4% | 0.3% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 129% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | 0.1% | -0.5% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 189% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.6% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.2% | -0.2% | 0.3% |
| YUV, mixed content, 1440p | -0.4% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 194% |  |  |

Table 4 Lossless coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.2% | 1.2% | 0.7% | 1.5% |
| RGB, text & graphics with motion,720p | 0.8% | 0.4% | -0.5% | 1.5% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.3% | 0.1% | 0.5% |
| YUV, text & graphics with motion,720p | 0.5% | 0.2% | -0.8% | 1.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.3% | 0.1% | 0.7% |
| RGB, text & graphics with motion,720p | 0.1% | -0.1% | -0.7% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.5% | 0.3% |
| YUV, text & graphics with motion,720p | 0.0% | -0.2% | -0.8% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 160% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | -0.1% | 0.1% |
| RGB, text & graphics with motion,720p | 0.1% | -0.1% | -0.8% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.9% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.6% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 161% |  |  |  |

### Test 3: Residual coding

Table 5 Lossy coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.3% | -1.2% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.3% | 0.4% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.3% |
| YUV, mixed content, 1440p | 0.2% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 126% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.8% | -0.7% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | 0.1% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 188% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | 0.2% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | -0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.7% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.4% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 193% |  |  |

Table 6 Lossy coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.3% | -2.2% |
| RGB, text & graphics with motion,720p | -1.2% | -1.6% | -1.4% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.0% | -0.8% |
| YUV, text & graphics with motion,720p | 0.1% | -0.2% | -0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | 0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 128% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.5% | -1.3% |
| RGB, text & graphics with motion,720p | -1.5% | -1.7% | -1.7% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.3% | -1.2% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.9% |
| YUV, mixed content, 1440p | -0.1% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | 0.0% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 189% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.4% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.5% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -0.5% | -1.1% | -0.9% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 194% |  |  |

### Test 5: Frequent color table

Table 7 Lossy coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.1% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.5% | 0.6% |
| YUV, text & graphics with motion,720p | 0.6% | 0.5% | 0.4% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.2% | 0.0% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 127% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.3% | -0.5% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | -0.4% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 188% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.1% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.3% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | -0.4% | -0.2% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.2% | -0.1% | 0.5% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 195% |  |  |

Table 8 Lossless coding efficiency of proposed solution vs. SCM 2.0 with full frame intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.4% | 1.5% | 0.5% | 2.5% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.0% | 0.6% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.7% | 0.7% | -0.1% | 1.9% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.1% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 119% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.8% | 0.3% | 1.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.2% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.6% | 0.0% | 1.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 165% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 0.5% | 0.3% | 0.7% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.4% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.3% | -0.1% | 0.8% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 166% |  |  |  |

Table 9 Lossy coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -2.1% | -2.0% |
| RGB, text & graphics with motion,720p | -1.1% | -1.4% | -1.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 123% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.4% | -1.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.5% | -1.4% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.1% | -1.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.6% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.4% |
| YUV, mixed content, 1080p | 0.1% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 189% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.1% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | 0.0% |
| YUV, Animation, 720p | 0.2% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 195% |  |  |

Table 10 Lossless coding efficiency of proposed solution vs. SCM 2.0 with constrained 1x4 intra block copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.4% | 2.5% | 0.9% | 4.3% |
| RGB, text & graphics with motion,720p | 0.7% | 1.0% | 0.0% | 2.9% |
| RGB, mixed content, 1440p | 0.1% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 2.1% | 0.4% | 4.8% |
| YUV, text & graphics with motion,720p | 0.5% | 1.1% | 0.0% | 3.8% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |  |
| Dec Time[%] | 117% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.6% | 1.4% | 0.5% | 3.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.5% | 0.0% | 1.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.4% | 1.5% | 0.3% | 3.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.7% | 0.0% | 2.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 160% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.9% | 0.5% | 1.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.4% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 163% |  |  |  |

# Recommendation

It is recommended to further study the hybrid 1-D and 2-D intra string copy in potential CE.

# Reference

[1] W. Wang, Z. Ma, M. Xu, F. Duanmu, H. Yu, “SCCE4: Result of Test 4.3.4 for string search for sample coding”, JCTVC R0145, Sapporo, JP, July 2014.

[2] Y. Chen and J. Xu, “Description of Core Experiment 10 (CE10): Intra String Copy”, Doc. JCTVC R1110, Sapporo, JP, July 2014.

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC R1015, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei USA R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).