| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0162 |
| --- | --- |

| Title: | CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of CE1 Tests on the proposed method in JCTVC-R0133 & JCTVC-R0134. Under the SCC CTCs, Test 1.4 on BVD coding reportedly achieves average Luma BD-rate savings of 0.5%, 0.1%, and 0.1% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively. Test 2.2 on MVD coding reportedly achieves average Luma BD-rate savings of 3.1%, and 4.0% for YUV, text & graphics with motion, 1080p sequences for the RA and LB settings, respectively. Test 3.3 on both BVD & MVD coding reportedly achieves average Luma BD-rate savings of 0.5%, 3.0%, and 3.9% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively.

In revision 1, the results of the proposed method for MVD & BVD coding with the constraint on the max number of regular bins per CU are further provided. It is reportedly shown that, despite an increase in the max number of the regular bins under the worst case by the proposed method, the existing HEVC parsing throughput constraint for a coding unit can still be met without any performance sacrifice using the proposed method.

# Proposed method

The proposed method represents the absolute value, | d |, of each component of the block difference vector in a binary form by its most significant bit (MSB) index followed by its refinement bits. The MSB index for syntax value 0 is assigned to -1. The MSB index plus one, referred to as msb_plus_one, is binarized by the unary code. The remaining refinement bits are represented in the fixed-length binary code with the codeword length determined by the MSB index. Incidentally, this binarization scheme leads to exactly the same bin string as that of the binarization scheme employed in the current HEVC for coding motion vector difference, as compared in Table 1.

The resulting bin stings are coded by context adaptive binary arithmetic coding (CABAC). The three new syntax elements abs_mvd_msb_idx_p1, abs_mvd_refinement_bits, and mvd_sign_flag are introduced for representing msb_plus_one, refinement bits and sign flag of a block vector component difference, respectively. The bin string for abs_bvd_msb_idx_p1 is entropy coded in the CABAC mode if the bin index is smaller than a threshold and in the bypass mode, otherwise.

The context selection is conditioned on bin index and vector component index. Table 2 provides the mapping table for context selection. The first 2 contexts are shared with the existing contexts for MVD coding and the total 4 contexts are added.

Table 1: The bin strings generated by the proposed method and HEVC mvd coding

|  | Proposed Method |  | HEVC mvd_coidng() |  |  |
| --- | --- | --- | --- | --- | --- |
| ¦ d ¦ | msb_plus_one | refinement bins | TU prefix | EG1 prefix | EG suffix |
| 0 | 0 | - | 0 | - | - |
| 1 | 10 | - | 10 | - | - |
| 2 | 110 | 0 | 11 | 0 | 0 |
| 3 | 110 | 1 | 11 | 0 | 1 |
| 4 | 1110 | 00 | 11 | 10 | 00 |
| 5 | 1110 | 01 | 11 | 10 | 01 |
| 6 | 1110 | 10 | 11 | 10 | 10 |
| 7 | 1110 | 11 | 11 | 10 | 11 |

Table 2: Context assignment for entropy coding abs_bvd_msb_idx_p1 of the BVD components

| bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | > 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizontal component | 5 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |
| Vertical component | 0 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |

In this way, the proposed method employs 6 contexts for modeling and may use up to 7 regular CABAC bins for coding a MVD or BVD component, an increase in the number of the regular bins by 5 per vector component. However, practical applications are mainly concerned about the worst-case parsing throughput rate for a processing unit, such as a CU or CTU. We can still easily meet the existing CABAC parsing throughput constraint for a processing unit despite an increase in the max number of the regular bins by the proposed method under the worst case. In our implementation, the encoder will track the number of regular bins consumed in the current CU subject to the rate-distortion optimization test. The encoder simply skips the CU configuration leading to the number of the regular bins greater than the max number of the regular bins subject to the specified throughput constraint.

# Experimental results

The proposed method has been implemented using reference software HM-15.0+RExt-8.0+SCM-2.0 as the code base. The simulation is conducted according to the common test conditions specified in [3]. The simulation results are summarized in Tables 1-6.

Tables 7 & 8 further provide the results of the proposed method for MVD & BVD coding with the constraint on the max number of regular bins per CU compared with those of the proposed method without the constraint. It was tested subject to the constraint that the number of regular bins for each 8x8 CU is less than 450, or 7.03125 bins per pixel. The same per-pixel constraint is also applied to CU’s of other sizes. The max number of the regular bins for an 8x8 CU is equal to 488 under the worst case, according to our estimation as show in the included spread sheet.

The detailed simulation results can be found in the included spread sheets.

Table 1: Lossy BD bitrate results compared with the SCM anchor for lossy coding, Test 1.4

Table 2: Bit-rate savings compared with the SCM anchor for lossless coding, Test 1.4

Table 3: Lossy BD bitrate results compared with the SCM anchor for lossy coding, Test 2.2

Table 4: Bit-rate savings compared with the SCM anchor for lossless coding, Test 2.2

Table 5: Lossy BD bitrate results compared with the SCM anchor for lossy coding, Test 3.3

Table 6: Bit-rate savings compared with the SCM anchor for lossless coding, Test 3.3

Table 7: Lossy BD bitrate results of the proposed method for MVD & BVD coding with the constraint on the max number of regular bins per CU compared with those of the proposed method without the constraint, as reported in Table 5.

Table 8: Lossless coding bitrate results of the proposed method for MVD & BVD coding with the constraint on the max number of regular bins per CU compared with those of the proposed method without the constraint, as reported in Table 6.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “SCCE1: Test 3.5 - Block vector difference coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0133, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “Non-SCCE1: Unified BVD & MVD coding using a universal entropy coding scheme,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0134, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.