The proposed modifications are based on HEVC Screen Content Coding: Draft 1 (JCTVC-R1005-v2). The inserted parts are marked in yellow. The deleted parts are marked in pink.

##### Sequence parameter set scc syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| mvd_msb_idx_coding_enabled_flag | u(1) |
| } |  |

#### Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) /* Intra BC*/ |  |
| bvd_coding( x0, y0, 2 ) |  |
| mvd_coding( x0, y0, 2 ) |  |
| else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Block vector difference syntax

| bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_bvd_greater0_flag[ 0 ] ) { |  |
| abs_bvd_minus1[ 0 ] | ae(v) |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ 1 ] ) { |  |
| abs_bvd_minus1[ 1 ] | ae(v) |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| if (mvd_msb_idx_coding_enabled_flag) { |  |
| abs_mvd_msb_idx_p1[ 0] | ae(v) |
| abs_ mvd_msb_idx_p1[1] | ae(v) |
| if(abs_mvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_mvd_msb_idx_p1 [0] > 1) |  |
| abs_mvd_refinement_bits [0] | ae(v) |
| mvd_sign_flag [0] | ae(v) |
| } |  |
| if(abs_mvd_msb_idx_p1 [1]>0 ) { |  |
| if(abs_mvd_msb_idx_p1 [1 ] > 1) |  |
| abs_mvd_refinement_bits [1] | ae(v) |
| mvd_sign_flag [ 1] | ae(v) |
| } |  |
| else { |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |

##### Sequence parameter set scc extension

intra_block_copy_enabled_flag equal to 1 specifies that intra block copying may be invoked in the decoding process for intra prediction. intra_block_copy_enabled_flag equal to 0 specifies that intra block copying is not applied. When not present, the value of intra_block_copy_enabled_flag is inferred to be equal to 0.

palette_enabled_flag equal to 1 specifies that the palette mode may be used for intra blocks. palette_enabled_flag equal to 0 specifies that the palette mode is not applied. When not present, the value of palette_enabled_flag is inferred to be equal to 0.

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive color transform is not applied to the residual. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be 0.

mvd_msb_idx_coding_enabled_flag specifies the selected syntax set for coding motion vector difference. When not present, the value of mvd_msb_idx_coding_enabled_flag is inferred to be equal to 0.

#### Prediction unit semantics

abs_bvd_greater0_flag[ compIdx ] equal to 1 specifies that the absolute value of the compIdx-th block vector component difference is greater than 0. abs_bvd_greater0_flag[ compIdx ] equal to 0 specifies that the absolute value of the compIdx-th block vector component difference is equal to 0.

abs_bvd_minus1[ compIdx ] plus 1 specifies the absolute value of the compIdx-th block vector component difference.

When abs_bvd_minus1[ compIdx ] is not present, it is inferred to be equal to −1.

bvd_sign_flag[ compIdx ] specifies the sign of the compIdx-th block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the compIdx-th block vector component difference has a positive value.

- Otherwise, the compIdx-th block vector component difference has a negative value.

When bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference BvdIntra[ compIdx ] for compIdx in the range of 0 to 1, inclusive, is derived as follows:

BvdIntra[ compIdx ] = abs_bvd_greater0_flag[ compIdx ] *	(7-XX)
	( abs_bvd_minus1[ compIdx ] + 1 ) * ( 1 − 2 * bvd_sign_flag[ compIdx ] )

[Ed. (GJS): Equation numbering needs fixing.]

mvp_l0_flag[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_l0_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_flag[ x0 ][ y0 ] has the same semantics as mvp_l0_flag, with l0 and list 0 replaced by l1 and list 1, respectively.

….

#### Motion vector difference semantics

abs_bvd_msb_idx_p1[ compIdx] minus 1 specifies the most significant bit index of the absolute value of a block vector component difference in a binary representation. abs_bvd_msb_idx_p1[ compIdx] equal to 0 indicates the absolute value of a block vector component difference is equal to 0.

abs_bvd_refinement_bits [compIdx] specifies the refinement bits of the absolute value of a block vector component difference in a binary representation. When abs_bvd_refinement_bits [compIdx] is not present, it is inferred to be equal to 0.

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] plus 2 specifies the absolute value of a motion vector component difference.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

- If mvd_msb_idx_coding_enabled_flag is equal to 1,

- If abs_mvd_msb_idx_p1[ compIdx] is equal to 0,

lMvd [ compIdx ] = 0.

- Otherwise (abs_mvd_msb_idx_p1[ compIdx] is greater than 0),

lMvd [ compIdx ] =
		(  ( 1 << ( abs_mvd_msb_idx_p1[compIdx] – 1 ) )   +  abs_mvd_refinement_bits[compIdx] ) * 	( 1 − 2 * mvd_sign_flag[ compIdx ] )

- Otherwise (mvd_msb_idx_coding_enabled_flag is equal to 0),

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
		( abs_mvd_minus2[ compIdx ] + 2 ) * 	( 1 − 2 * mvd_sign_flag[ compIdx ] ) 	(7-1)

The variable MvdLX[ x0 ][ y0 ][ compIdx ], with X being 0 or 1 1, or 2, specifies the difference between a list X vector component to be used and its prediction. The value of MvdLX[ x0 ][ y0 ][ compIdx ] shall be in the range of −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of  −215 to 215 − 1 −128 to 128, inclusive [Ed. (RJ): Please check the range]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise, if refList is equal to 1, MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1 [Ed. (RJ): Please check this part.]

| Table 9-1 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_subset_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | palette_mode_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
|  | cu_residual_act_flag | FL | cMax = 1 |
| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
|  | palette_share_flag | FL | cMax = 1 |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx = = 0 ? ( (1<<BitDepthY) − 1 ) : ( (1<<BitDepthC) − 1 ) |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | palette_escape_val | Error: Reference source not found | cIdx, qP |
|  | escape_val_present_flag | FL | cMax = 1 |
|  | palette_run_type_flag | FL | cMax = 1 |
|  | palette_index | TB | cMax = adjustedIndexMax |
|  | palette_run | Error: Reference source not found | - |
|  | palette_all_zeros_in_group | FL | cMax = 1 |
|  | palette_last_group | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| bvd_coding ( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_minus1[ ] | EG3 | - |
|  | bvd_sign_flag[ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_msb_idx_pl[ ] | R | cRiceParam = 0 |
|  | abs_mvd_refinement_bits [ ] | FL | cMax = abs_mvd_msb_idx_pl-1 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | cu_chroma_qp_offset_flag | FL | cMax = 1 |
|  | cu_chroma_qp_offset_idx | TR | cMax = chroma_qp_offset_list_len_minus1, cRiceParam = 0 |
| cross_comp_pred( ) | log2_res_scale_abs_plus1 | TR | cMax = 4, cRiceParam = 0 |
|  | res_scale_sign_flag | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | explicit_rdpcm_flag[ ][ ][ ] | FL | cMax = 1 |
|  | explicit_rdpcm_dir_flag[ ][ ][ ] | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater1_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater2_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_remaining[ ] | Error: Reference source not found | current sub-block scan index i, baseLevel |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

#### Derivation process for ctxTable, ctxIdx and bypassFlag

| Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| end_of_subset_one_bit | terminate | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | bypass | na | na | na |
| sao_eo_class_chroma | bypass | bypass | bypass | na | na | na |
| split_cu_flag[ ][ ] | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| intra_bc_flag | 0 | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| palette_mode_flag | 0 | na | na | na | Na | Na |
| palette_share_flag | 0 | na | na | na | Na | na |
| previous_palette_entry_flag | bypass | na | na | na | na | Na |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_transpose_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | Na |
| palette_escape_val | bypass | bypass | bypass | bypass | bypass | bypass |
| escape_val_present_flag | bypass | na | na | na | na | na |
| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | Na | na | Na | Na |
| palette_index | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_run | 0 | 1 | 2 | bypass | bypass | bypass |
| palette_all_zeros_in_group | bypass | na | na | na | na | na |
| palette_last_group | bypass | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| prev_intra_luma_pred_flag[ ][ ] | 0 | na | na | na | na | na |
| mpm_idx[ ][ ] | bypass | bypass | na | na | na | na |
| rem_intra_luma_pred_mode[ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] | 0 | bypass | bypass | na | na | na |
| rqt_root_cbf | 0 | na | na | na | na | na |
| cu_residual_act_flag | 0 | na | na | na | na | na |
| merge_flag[ ][ ] | 0 | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( nPbW + nPbH ) != 12 ? CtDepth[ x0 ][ y0 ] : 4 | 4 | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| split_transform_flag[ ][ ][ ] | 5 − log2TrafoSize | na | na | na | na | na |
| cbf_cb[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_cr[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_luma[ ][ ][ ] | trafoDepth = = 0 ? 1 : 0 | na | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| abs_mvd_msb_idx_pl[ ] | (subclause 9.3.4.2.x) |  |  |  |  |  |
| abs_mvd_refinement_bits [ ] | Bypass | Bypass | Bypass | bypass | bypass | bypass |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| cu_chroma_qp_offset_flag | 0 | na | na | na | na | na |
| cu_chroma_qp_offset_idx | 0 | 0 | 0 | 0 | 0 | na |
| log2_res_scale_abs_plus1[ c ] | 4 * c + 0 | 4 * c + 1 | 4 * c + 2 | 4 * c + 3 | na | na |
| res_scale_sign_flag[ c ] | c | na | na | na | na | na |
| transform_skip_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| explicit_rdpcm_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| explicit_rdpcm_dir_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| last_sig_coeff_x_prefix | 0..17 (subclause ) |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..17 (subclause ) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | 0..3 (subclause Error: Reference source not found) | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | 0..43 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater1_flag[ ] | 0..23 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater2_flag[ ] | 0..5 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_remaining[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] | bypass | na | na | na | na | na |

##### 9.3.4.2.x Derivation process of ctxInc for the syntax element abs_mvd_msb_idx_p1

Inputs to this process are the bin index binIdx and the vector component index compIdx

Output of this process is the variable ctxInc

Table 9-xx – Specification of ctxIdx[compIdx][ binIdx ]

| binIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | > 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| compIdx = 0 | 5 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |
| compIdx = 1 | 0 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |