| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0163 |
| --- | --- |

| Title: | CE6: Results of Test A.4 on palette run coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc |  |  |

_____________________________

# Abstract

This contribution reports the results of CE6 Test A.4 on the proposed method for palette run coding in JCTVC-R0136. It reportedly achieves average Luma BD-rate savings of 1.9%, 1.0%, and 0.8% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively, under the SCC CTCs.

# Proposed method

The proposal employs a new method for coding syntax palette_run by using a universal entropy coding scheme. This entropy coding method, previously employed in [3] for coding IBC block vector difference, represents a syntax element in a binary value by sending its most significant bit (MSB) index followed by its refinement bits. The bin string for a binarized syntax value is thus a concatenation of a prefix bin string and a suffix bin string. The prefix string represents the MSB index plus 1, or msb_id_plus1, of a syntax value in a unary code, or a truncated unary code if the maximal syntax value is known. msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 =  Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix bin string represents the refinement bits and is present if msb_id_plus1 > 1. The value of refinement bits, denoted by refinement_bits, is coded in a fixed-length binary code with the string length equal to msb_id_plus1 – 1. The decoded syntax value x is given by

x  =  (1 << ( msb_id_plus1-1)) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,  otherwise.

Context modeling for entropy coding the bin string of binarized msb_id_plus1 of palette_run is conditioned on the bin index and the two separate context sets are employed, corresponding to palette_run_type_flag equal to COPY_ABOVE_MODE and INDEX_MODE, respectively. Inspired by the observation that palette_run is often equal to 0 when palette_run_type_flag is equal to INDEX_MODE and the current palette index is large, context modeling for bin index 0 is further conditioned on the palette index for the INDEX_MODE run type. The context index is assigned to 0 if palette_index <= 1, 1 if 1 < palette_index <=7, or 2 otherwise. Table 1 provides the mapping table for context selection currently employed by our proposal.

Table 1 – Specification of ctxIdx[palette_run_type_flag][ binIdx ]

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| palette_run_type_flag = COPY_ABOVE_MODE | 5 | 6 | 6 | 7 | 7 | bypass |
| palette_run_type_flag = INDEX_MODE | 0,1,2 | 3 | 3 | 4 | 4 | bypass |

# Experimental results

The proposed method has been implemented using reference software HM-15.0+RExt-8.0+SCM-2.0 as the code base. The simulation is conducted according to the common test conditions specified in [2]. The simulation results are summarized in Tables 1 & 2 for lossy & lossless coding, respectively, with context selection determined by the reconstructed index value for the INDEX mode and in Tables 3 & 4 for lossy & lossless coding, respectively, with context selection determined by the coded index value. The detailed simulation results can be found in the included spread sheets.

Table 1: Lossy BD bitrate results compared with the SCM anchor for lossy coding, conditioned on the reconstructed palette index value

Table 2: Bit-rate savings compared with the SCM anchor for lossless coding, conditioned on the reconstructed palette index value

Table 3: Lossy BD bitrate results compared with the SCM anchor for lossy coding, conditioned on the coded palette index value

Table 4: Bit-rate savings compared with the SCM anchor for lossless coding, conditioned on the coded palette index value

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “Non-SCCE3:Run coding of the palette index map using a universal entropy coding scheme,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0136, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei, “SCCE1: Test 3.5 - Block vector difference coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0133, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.