| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0164 |
| --- | --- |

| Title: | CE6: Results of Test B.4 on palette index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc |  |  |

_____________________________

# Abstract

This contribution reports the results of CE6 Test B.4 on the proposed method for entropy coding palette index values in JCTVC-R0135. It reportedly achieves average Luma BD-rate savings of 0.1%, 0.0%, and 0.1% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively, under the SCC CTCs.

# Proposed method

This contribution proposes a new method for coding palette index values by using a universal entropy coding scheme. This entropy coding method, previously employed in [3] for coding the IBC block vector difference, represents a syntax element in a binary value by sending its most significant bit (MSB) index followed by its refinement bits. The bin string for a binarized syntax value is thus a concatenation of a prefix bin string and a suffix bin string. The prefix string represents the MSB index plus 1, or msb_id_plus1, of a syntax value in a unary code, or a truncated unary code if the maximal syntax value is known. msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 =  Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix bin string represents the refinement bits and is present if msb_id_plus1 > 1. The value of refinement bits, denoted by refinement_bits, is coded in a fixed-length binary code with the string length equal to msb_id_plus1 – 1. The decoded syntax value x is given by

x  =  (1 << ( msb_id_plus1-1)) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,  otherwise.

For entropy coding the palette index values, msb_id_plus1 of syntax palette_index is coded by the truncated unary code with the max palette index equal to palette_size in the current CU. When msb_id_plus1of palette_index is equal to msb_id_plus1of the max palette_index, refinement_bits of palette_index is coded by the truncated binary code, and by the fixed-length code, otherwise. msb_id_plus1 is coded in the CABAC mode and each bin index value is assigned a single context. The total 5 contexts are employed when max palette_size is equal to 32. refinement_bits is coded in the bypass mode.

# Experimental results

The proposed method has been implemented using reference software HM-15.0+RExt-8.0+SCM-2.0 as the code base. The simulation is conducted according to the common test conditions specified in [2]. The simulation results are summarized in Tables 1 & 2 for lossy & lossless coding, respectively. The detailed simulation results can be found in the included spread sheets.

Table 1: Lossy BD bitrate results compared with the SCM anchor for lossy coding

Table 2: Bit-rate savings compared with the SCM anchor for lossless coding

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “Non-SCCE3: Palette index coding using a universal entropy coding scheme,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0135, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-R1015, 18th Meeting: Sapporo, Japan, 30 June – 9 July 2014.

- Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei, “SCCE1: Test 3.5 - Block vector difference coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0133, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.