| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0171 |
| --- | --- |

| Title: | Cross-check of ‘CE2: Test 5 – Intra BC unified with Inter using intra_bc_flag’ (JCTVC-S0122) by Mediatek and Microsoft |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | V. Kolesnikov Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2973 vladimir.kolesnikov@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of CE2 test 5 on intra block copy unified with inter-prediction using intra_bc_flag by Mediatek and Microsoft. The result reported by the proponents are confirmed in this cross-check.

# Cross check procedure

The source code for JCTVC-S0122 [1] was provided by Mediatek and Microsoft. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code provided by Mediatek and Microsoft was studied and the following observation was made:

- Syntax element intra_bc_flag is retained and contexts for the BVD coding scheme are removed.

- The BVP scheme present in SCM2.0 is removed (under control of a separate macro INTRA_BC_AMVP_MVD to permit independent control of this aspect.

- Block vectors are coded in unit-pel precision (as opposed to the quarter-pel precision used for motion vectors).

- I slices are converted to P slices to provide access to intra-block copy, with the current picture being appended to LIST0.

- The inter-prediction search function is extended to support the block vector candidate list.

The normative changes listed above correspond to the description in [1].

# Coding performance measurement

Table 1 and Table 2 show the lossy and lossless simulation results respectively for JCTVC-S0122 with  AMVP and the mvd_coding() syntax for the block vectors.  Table 3 and Table 4 show the lossy and lossless simulation results respective for JCTVC-S0122 with the SCM2.0 BVP and BVD scheme. The simulation result of the provided source code matches the result reported by the proponent.

Table 1. Lossy simulation result with AMVP and MVD coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.2% | -3.7% | -3.9% |
| RGB, text & graphics with motion,720p | -1.2% | -2.2% | -2.1% |
| RGB, mixed content, 1440p | -1.6% | -2.0% | -2.2% |
| RGB, mixed content, 1080p | -1.2% | -1.9% | -2.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -2.1% | -2.1% |
| YUV, text & graphics with motion,720p | -0.5% | -1.4% | -1.6% |
| YUV, mixed content, 1440p | -0.8% | -1.5% | -1.7% |
| YUV, mixed content, 1080p | -0.7% | -2.2% | -2.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.9% | -5.8% | -5.8% |
| RGB, text & graphics with motion,720p | -1.2% | -2.6% | -2.6% |
| RGB, mixed content, 1440p | -1.3% | -2.3% | -2.5% |
| RGB, mixed content, 1080p | -1.3% | -2.5% | -2.8% |
| RGB, Animation, 720p | 0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.0% | -5.7% | -5.7% |
| YUV, text & graphics with motion,720p | -0.7% | -2.1% | -2.3% |
| YUV, mixed content, 1440p | -1.0% | -2.6% | -2.5% |
| YUV, mixed content, 1080p | -0.9% | -3.8% | -3.4% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.2% | -5.7% | -5.6% |
| RGB, text & graphics with motion,720p | -0.9% | -2.3% | -2.3% |
| RGB, mixed content, 1440p | -1.3% | -2.3% | -2.5% |
| RGB, mixed content, 1080p | -1.2% | -3.1% | -2.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.4% | -5.9% | -5.7% |
| YUV, text & graphics with motion,720p | -1.0% | -1.9% | -2.5% |
| YUV, mixed content, 1440p | -1.1% | -2.6% | -2.5% |
| YUV, mixed content, 1080p | -1.0% | -4.0% | -4.9% |
| YUV, Animation, 720p | 0.0% | -0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 96% |  |  |

Table 2. Lossless simulation result with AMVP and MVD coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.6% | 1.2% | 2.0% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 0.9% |
| RGB, mixed content, 1440p | 1.5% | 1.3% | 0.3% | 2.3% |
| RGB, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 1.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.8% | 0.7% | 0.2% | 1.3% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 1.3% | 1.2% | 0.2% | 2.1% |
| YUV, mixed content, 1080p | 1.1% | 1.1% | 1.1% | 1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.2% | 2.7% | 1.3% | 3.5% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.6% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.6% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.3% | 2.5% | 1.3% | 3.6% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.6% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.6% | 2.9% | 1.0% | 4.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.5% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.5% | 2.9% | 1.2% | 3.9% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.6% |
| YUV, mixed content, 1440p | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 3. Lossy simulation result with SCM2.0 BVP scheme and BVD coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.5% | -5.0% | -5.1% |
| RGB, text & graphics with motion,720p | -2.2% | -3.2% | -3.1% |
| RGB, mixed content, 1440p | -2.1% | -2.5% | -2.8% |
| RGB, mixed content, 1080p | -1.8% | -2.5% | -2.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.2% | -3.7% | -3.7% |
| YUV, text & graphics with motion,720p | -1.6% | -2.6% | -2.8% |
| YUV, mixed content, 1440p | -1.6% | -2.2% | -2.5% |
| YUV, mixed content, 1080p | -1.6% | -2.9% | -2.9% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.4% | -6.3% | -6.3% |
| RGB, text & graphics with motion,720p | -1.7% | -3.2% | -3.1% |
| RGB, mixed content, 1440p | -1.8% | -2.9% | -3.0% |
| RGB, mixed content, 1080p | -1.7% | -3.0% | -3.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.6% | -6.3% | -6.3% |
| YUV, text & graphics with motion,720p | -1.4% | -2.8% | -3.1% |
| YUV, mixed content, 1440p | -1.6% | -3.0% | -2.9% |
| YUV, mixed content, 1080p | -1.6% | -3.9% | -3.9% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 90% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.7% | -6.2% | -6.1% |
| RGB, text & graphics with motion,720p | -1.5% | -2.6% | -2.6% |
| RGB, mixed content, 1440p | -1.5% | -2.4% | -2.5% |
| RGB, mixed content, 1080p | -1.4% | -2.9% | -3.1% |
| RGB, Animation, 720p | -0.2% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.9% | -6.4% | -6.3% |
| YUV, text & graphics with motion,720p | -1.5% | -2.4% | -2.6% |
| YUV, mixed content, 1440p | -1.7% | -3.3% | -2.7% |
| YUV, mixed content, 1080p | -1.4% | -4.8% | -5.2% |
| YUV, Animation, 720p | 0.0% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 96% |  |  |

Table 4. Lossless simulation result with SCM2.0 BVP scheme and BVD coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.2% | 2.3% | 1.9% | 3.0% |
| RGB, text & graphics with motion,720p | 0.4% | 0.6% | 0.1% | 1.0% |
| RGB, mixed content, 1440p | 1.6% | 1.4% | 0.4% | 2.4% |
| RGB, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.5% | 1.5% | 0.9% | 1.8% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.1% | 0.7% |
| YUV, mixed content, 1440p | 1.5% | 1.3% | 0.4% | 2.3% |
| YUV, mixed content, 1080p | 1.2% | 1.2% | 1.2% | 1.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.3% | 2.6% | 1.6% | 3.9% |
| RGB, text & graphics with motion,720p | 0.2% | 0.4% | 0.1% | 0.9% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.6% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.3% | 2.4% | 1.5% | 3.3% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.2% | 0.7% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.6% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.7% | 2.8% | 1.4% | 4.3% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.5% | 2.8% | 1.5% | 4.1% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.8% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusion

The results provided by the proponents are verified in this cross-check report.

# References

- X. Xu, S. Liu, S. Lei, B. Li, J. Xu, “CE2: Test 5 – Intra BC unified with Inter using intra_bc_flag”, JCTVC-S0122, Strasbourg, FR, October 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, JCTVC-R1015, Sapporo, JP, June 2014.