| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0176 |
| --- | --- |

| Title: | CE8 Test C: Independent Uniform Prediction Intra Mode (IUP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Xingyu Zhang Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7500 cohen @merl.com eexyzhang@ust.hk |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document proposes the Independent Uniform Prediction (IUP) mode as studied in CE9 Test C. IUP targets flat or mostly flat areas in a picture by predicting a CU using a uniform color. The improvements in average BD-Rates for AI test conditions for RGB text & graphics with motion 1080p are 0.3%, 0.3%, and 0.3% for G, B, and R components respectively. For RGB text & graphics with motion 720p, the corresponding improvements in average BD-Rates are 0.4%, 0.5%, and 0.4%.

# Independent Uniform Prediction mode

As described in JCTVC-R0200 [1] and JCTVC-Q0036 [2] and repeated here for convenience, the prediction modes currently available in HEVC-RExt are based primarily on data available from nearby previously-decoded blocks. Intra-block directional prediction is based upon adjacent pixels from neighboring blocks. Intra block copy mode and motion estimation for Inter-block coding applies a limited-range search to nearby pixels in the current frame or in a picture reference buffer. Unlike typical camera-captured videos, video from a computer, or screen content video, often follows a consistent theme. This theme defines graphical elements such as background color, window colors and border shapes. Most of the existing Intra prediction schemes, due to their localized prediction process, cannot exploit the commonalities in a theme globally over the entire picture or sequence of pictures, due to the localized nature of the Intra and Inter prediction process. The recently proposed palette modes exploit the reduced number of colors that screen content pictures typically contain, however, the palette is adapted while coding the picture, and in recent implementations, is reset when starting each row of CTBs of a slice. This local adaptation is useful when the palette or set of used colors is consistent over an entire row of CTBs or picture, but when a picture comprises several similar but distant regions, the palette may not be optimal when entering each region. The purpose of the independent uniform prediction (IUP) mode is to compute a small set of global block predictors to target regions of a picture that have a limited number of commonly-occurring colors that are typical in a computing environment, such as background colors. This mode is available for coding blocks in both Inter and Intra slices.

In the slice encoding process, before CTUs in a slice are being encoded, a histogram of the pixel values (RGB or YCbCr triplets) is maintained. To limit the complexity of this process, only the center pixel from each CTU is used to compute the histogram. Before encoding the slice, these triplets form the set S1. The num_uniform_colors most frequently occurring triplets are saved as set S2. The number of colors num_uniform_colors and the color triplet values are signaled in the slice header.

In the slice decoding process, the num_uniform_colors and the color triplet values are decoded from slice header. The CU-level decoding process is shown in Figure 1. A flag independent_uniform_pred_flag is parsed from the bitstream to indicate whether the independent uniform prediction mode is used for the current CU. If independent_uniform_pred_flag is equal to 1 and num_uniform_colors >1, then uniform_color_index is parsed from the bitstream to indicate which color triplet from set S2 is used in the current CU. If independent_uniform_pred_flag is true and num_uniform_colors ==1, then uniform_color_index is inferred to be 1.

Figure 1. CU decoding process for independent uniform prediction

For predicting a CU using the independent uniform prediction mode, a uniform predictor block is formed for each color component, where all pixels in the predictor block have the same value as the corresponding element in the selected color triplet. This predictor block is then treated the same way as in other prediction methods, where the predictor is added to the reconstructed residual block to form the reconstructed block.

Because the independent uniform prediction mode targets very commonly used colors used over an entire picture, we limited num_uniform_colors in this proposal to 2.

# Experimental Results

The test conditions specified in the CE8 description [3] were used for these experiments.

## Lossy coding results

Decode time ratios may vary slightly due to differences in compute platforms.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.5% |
| YUV, text & graphics with motion,720p | -0.1% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | 0.2% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | 0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.2% |
| YUV, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.3% | 1.4% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 96% |  |  |

## Lossless coding results

Decode time ratios may vary due to differences in compute platforms.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.3% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.2% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.2% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# Modifications to working draft text

Modifications to the working draft text are provided in the accompanying file JCTVC-S0176-WDtext.docx.

# Conclusions

Independent Uniform Prediction (IUP) mode is capable of providing improvements in compression efficiency both with and without palette mode, with no increase in encoder or decoder run times. The improvements in average BD-Rates for AI test conditions for RGB text & graphics with motion 1080p are 0.3%, 0.3%, and 0.3% for G, B, and R components respectively. For RGB text & graphics with motion 720p, the corresponding improvements in average BD-Rates are 0.4%, 0.5%, and 0.4%. We propose that IUP mode be added to the screen-content coding extension.

We would like to thank participants from ITRI for their cross-check of this test.

# Patent rights declaration

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Zhang, R. Cohen, “SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0200, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.

- R. Cohen, X. Zhang, A. Vetro, K. Sugimoto, A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami, “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, 27 March – 4 April, 2014.

- R. Cohen, T.-D. Chuang, C.-C. Lin, K. Rapaka, “Description of Core Experiment 8 (CE8): Uni- and bi-colour mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1108, 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014.