| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0179 |
| --- | --- |

| Title: | On inter-component de-correlation for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yan Ye, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This contribution proposes to improve the inter-component de-correlation methods in HEVC screen content coding draft 1 in order to simplify pipeline design and improve parallelization. Firstly, it is proposed to move inverse adaptive color transform from CU/PU-level to TU-level to unify with inverse cross-component prediction, which is performed at TU-level. Secondly, it is proposed to combine inverse cross-component prediction and inverse adaptive color transform into one single decoding process.

Compared to the SCM-2.0 anchor, experimental results show that the proposed solution has the same coding performance and similar encoding and decoding time, while offering the benefit of reduced latency.

# Introduction

In HEVC screen content coding (SCC) draft 1 [1], two coding tools, namely cross-component prediction (CCP) and adaptive color transform (ACT), are employed to exploit the correlation among three color components. Both ACT and CCP operate in the residual domain. Figure 1 illustrates the decoding process where both ACT and CCP are applied.

Figure 1 Block diagram of decoding process in SCM-2.0

In the current HEVC SCC draft [1], the decoding process of the inverse CCP is a TU-level operation directly inherited from the Rext specification. The inverse ACT process in HEVC SCC draft 1 is written such that inverse ACT is performed at different levels depending on whether the CU is coded in intra mode, intra block copy mode or inter mode. Specifically, if the CU is coded with intra mode or intra block copy mode, the inverse ACT is performed at PU-level; otherwise (the CU is coded with inter mode), the inverse ACT is performed at CU-level. If a decoder implementation follows exactly the current specification, then its pipeline design and parallel processing capability could become complicated, as the inverse ACT of the samples in one CU/PU cannot be invoked until the residuals of all the TUs in that CU/PU are completely reconstructed after the inverse CCP. Figure 2 shows one example to illustrate residual decoding process when performing the inverse CCP on TU-level and the inverse ACT on CU-level. Assume the CU is partitioned into four TUs, i.e., TU0, TU1, TU2 and TU3, with the same size, and that the inverse CCP is applied to TU0 and TU2, but not applied to TU1 and TU3. Let tiinv−transquant and tiiinv−ccp denote the decoding time of de-quantization & inverse transform module and inverse CCP module for the i-th TU. As shown in Figure 2, the decoder delay before starting the inverse ACT process for the CU is equal to the longest TU decoding time of inverse quantization, transform, and CCP, i.e., maxi=0...3(tiinv−transquant+tiinv−ccp).

Figure 2 Example of residual decoding process when inverse CCP and inverse ACT are performed at TU-level and CU-level respectively

Additionally, in HEVC SCC draft 1 [1], the inverse CCP and inverse ACT modules are currently defined as two separate decoding processes. This could increase the encoding and decoding latency, as the inverse ACT can only be started after inverse CCP is finished.

# Proposed methods

Given the aforementioned problems, two improved technologies are proposed in this section to the current CCP and ACT design to facilitate the pipeline design and parallel processing and reduce the decoding latency.

## TU-level inverse ACT (editorial changes to the specification)

In this section, one editorial change is proposed to the current specification by moving the inverse ACT from CU/PU-level to TU-level for all the coding modes. It is asserted that the proposed editorial change represents a more efficient implementation with regard to the pipeline and parallel design at the decoder, given that the residuals of each TU can now be converted back to the original color space without waiting for the reconstruction of the luma and chroma residuals of all TUs in the entire CU. Moreover, the proposed editorial change could also make the ACT more consistent with the CCP design in the specification, as both of two modules are now located at TU level. Finally, as it could be seen from the experimental results in Section 3, after moving the inverse ACT to TU-level, the CBF flag of each TU could be utilized to skip unnecessary inverse ACT operations when the residuals of that TU are all zeros, which could reduce decoding complexity.

Using the same example in Figure 2, Figure 3 depicts the corresponding residual decoding process when both the inverse CCP and the inverse ACT are performed at TU-level. Compared to the decoding process in Figure 2, the residuals of each TU in Figure 3 can be immediately transformed to the original color space right after the inverse CCP.

Figure 3 Example of residual decoding process when both inverse CCP and inverse ACT are performed at TU-level

## Combined inverse CCP and inverse ACT

In the HEVC SCC draft [1], inverse CCP and inverse ACT are defined as two separate decoding processes. This could increase the decoding latency as the inverse ACT can only be invoked after the inverse CCP is done. Given that both ACT and CCP are carried out in residual domain, it is proposed to combine the inverse CCP and the inverse ACT to one single decoding process.

For lossy coding, as both the inverse CCP and the inverse ACT are linear operations, the combined decoding process can be achieved by simply multiplying the matrices of the two operations, i.e.,

|  | GBR=Minv−ACTMinv−CCPYCgCo=181101−1−11−11800βCg80βCo08YCgCo=188+βCg008−(βCg+βCo)−8−88−(βCg−βCo)−88YCgCo | (1) |
| --- | --- | --- |

where Minv−ACTand Minv−CCPare the matrices of the inverse ACT and the inverse CCP, respectively, in lossy coding; βCgand βCoare the CCP parameters of chroma components.

For lossless coding, given the lifting-based YCgCo-R transform [2] is used for the inverse ACT, the combined decoding process is obtained by concatenating two operations as follows:

|  |  | (2) |
| --- | --- | --- |

In addition, it should be mentioned that the proposed combined module of inverse CCP and ACT could replace the current inverse ACT module. This is because the inverse ACT module can be treated as a special case of the proposed combined module, where the CCP parameters are equal to 0. Moreover, it should be also emphasized that the proposed combination of the inverse CCP and the inverse ACT is based on the TU-level inverse ACT in Section 2.1, i.e., it can only be enabled after moving the inverse ACT to TU-level.

# Simulation results

The proposed methods are implemented based on SCM-2.0 and tested under the common test conditions as specified in [3]. The coding performance of SCM-2.0 is used as the anchor. The encoding and decoding time are measured by using a cluster system with homogeneous cores, and are therefore reliable.

## Coding performance for TU-level inverse ACT

Table 1 and Table 2 present BD-rate and bit-rate performances of TU-level inverse ACT for lossy coding and lossless coding, respectively, compared to the SCM-2.0 anchor. As shown in Table 1 and Table 2, because this method is only related with the level where inverse ACT is operated rather than modify the design of the core ACT transform, there are no BD-rate difference of the proposed method compared to the anchor. But, after moving the inverse ACT to TU-level, the CBF flag of each TU could be utilized to skip unnecessary inverse ACT operations when the residuals of that TU are all zeros. This could reduce decoding time. More specifically, the proposed method achieve the average decoding time reductions of 7%, 6% and 6% for AI, RA and LB configurations, respectively, for lossy coding. When it comes to lossless coding, the corresponding decoding time reductions are 5%, 6% and 3% for AI, RA and LB configurations respectively.

Table 1 BD-rate performance of TU-level inverse ACT, compared to SCM-2.0 anchor for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 94% |  |  |

Table 2 Bit-rate performance of TU-level inverse ACT, compared to SCM-2.0 anchor for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

## Coding performance of combined inverse CCP and inverse ACT

Table 3 and Table 4 present BD-rate and bit-rate performances of combined inverse CCP and inverse ACT for lossy coding and lossless coding, respectively, compared to the SCM-2.0 anchor. As can be seen in Table 3 and Table 4, there is no coding efficiency loss incurred by the combination of inverse CCP and inverse ACT, whereas the combination reduces latency. Additionally, the encoding and decoding times are also similar to those of the SCM-2.0 anchors.

Table 3 BD-rate performance of combined inverse CCP and inverse ACT, compared to SCM-2.0 anchor for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | -0.1% | 0.2% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion,720p | 0.1% | -0.4% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | 0.0% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.4% | -0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | 0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.3% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | -0.3% | 0.0% |
| YUV, Animation, 720p | 0.0% | -1.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.7% | -0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 Bit-rate performance of combined inverse CCP and inverse ACT, compared to SCM-2.0 anchor for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.2% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.2% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.2% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# References

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 1, JCTVC-R1005, August 2014, Sapporo, Japan.

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, SCCE5 Test 3.2.1: In-loop color-space transform, JCTVC-R0147, August 2014, Sapporo, Japan.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-R1015, Auguest 2014, Sapporo, Japan.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).