| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0180 |
| --- | --- |

| Title: | Adaptive color transform for different luma and chroma bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This contribution proposes to modify the adaptive color transform in HEVC screen content coding draft 1 in order to handle the internal bit-depth difference between luma and chroma components. Specifically, it is proposed to align the bit-depths of luma and chroma components before adaptive color transform by left-shifting the component with lower bit-depth to match the bit-depth of the other component; then, the adjusted component is converted to its original bit-depth by right-shift after adaptive color transform. Additionally, two encoder-only bug fixes are also provided to enable the HEVC screen content reference software (SCM-2.0) to properly work with different internal luma and chroma bit-depths. Both the bug fixes and the proposed solution are implemented and tested on the SCM-2.0 software for two settings with unequal internal luma and chroma bit-depths, namely, 10-bit luma and 8-bit chroma (Setting one), and 12-bit luma and 8-bit chroma (Setting two).

Compared to the SCM-2.0 anchor, for Setting one, the proposed bug fixes could provide average {G, B, R} BD-rate savings of {1.6%, 2.6%, 2.6%}, {0.1%, 1.3%, 1.1%} and {0.8%, 0.7%, 0.8%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {1.6%, 5.2%, 5.1%}, {0.4%, 3.9%, 3.7%} and {0.4%, 3.5%, 3.4%} for AI, RA and LB configurations in YCbCr coding. For Setting two, the corresponding coding gains of the proposed bug fixes are {5.6%, 6.5%, 6.6%}, {4.8%, 8.8%, 7.5%} and {2.5%, 11.5%, 9.4%} for the {G, B, R} BD-rate savings in RGB coding, and are {3.4%, 6.5%, 7.0%}, {2.4%, 5.1%, 5.7%} and {1.9%, 4.5%, 5.0%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

Compared to the SCM-2.0 anchor with the bug fixes, for Setting one, the proposed bit depth alignment solution could provide average {G, B, R} BD-rate savings of {15.4%, 10.1%, 10.7%}, {20.1%, 13.2%, 14.2%} and {23.6%, 18.1%, 19.2%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {0.1%, 1.3%, 0.9%}, {0.3%, 1.5%, 1.1%} and {0.3%, 1.3%, 1.0%} for AI, RA and LB configurations in YCbCr coding. For Setting two, the corresponding coding gains of the proposed bit depth alignment solution are {22.1%, 17.1%, 17.7%}, {40.0%, 33.7%, 34.6%} and {47.3%, 40.4%, 41.1%} for the {G, B, R} BD-rate savings in RGB coding, and are {0.1%, 1.3%, 0.9%}, {0.4%, 1.6%, 1.2%} and {0.3%, 1.6%, 1.2%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

# Introduction

At the 18th JCT-VC meeting, adaptive color transform (ACT) [1] was adopted into HEVC screen content coding (SCC) draft 1 [2]. The ACT coding tool provides significant coding gains by exploiting the redundancies among three color components in residual domain. However, the tests conducted under SCC CTC were limited to sequences with the same luma and chroma sample bit depth. The ACT design in [1] does not consider the case where the bit-depths of luma and chroma components are different. Therefore, the coding gains that are provided from ACT are compromised when the luma and chroma components are coded with different bit-depth.

# Proposed methods

Given the aforementioned problem, one solution is proposed in this section to improve the existing ACT design such that it can properly work when the bit-depths of luma and chroma components are different. In addition, two encoder-side bug fixes are also proposed to enhance the performance of SCM-2.0 with different luma and chroma bit-depths.

## Encoder-only bug fixes

It is noted that although the SCM-2.0 software could work when the bit-depths of luma and chroma components are different, the following issues are identified, which could negatively affect the coding performance when bit-depths for luma and chroma components are different.

- For hash-based inter search, the bit-depth that is used to derive the chroma 16-bit CRC value for the pixels in each row of one 2Nx2N CU is set equal to that of the luma component.

- For palette coding mode, in lossy coding, one color-clustering-based method is employed to select major colors, where the range of each cluster is pre-defined depending on the quantization parameter that is used. In SCM-2.0, this pre-defined value is set by assuming that the internal bit-depths of luma and chroma components are all equal to 8.

To resolve the above issues, two encoder-only bug fixes are proposed to further improve the coding performance of the SCM-2.0 software when luma and chroma components are coded with different bit-depths.

- When generating the hash table at encoder, use the bit-depth of chroma component to calculate the corresponding chroma CRC values for all the pixels in one row.

- When generating the majors colors in one CU, use 8-bit distortion to generate the palette table and the palette index map. Specifically, denoting BitDepthias the bit-depth of the i-th color component, whether one pixel is assigned to one major color is determined by

|  | i=02Pi−Ci>>BitDepthi−8≤T | (1) |
| --- | --- | --- |

where Piand Ciare the i-th component of the current pixel Pand the tested major color C; Tis the threshold.

## The proposed bit depth alignment solution

In order for ACT to properly handle the case when luma and chroma components are coded with different bit-depths, it is proposed to align luma and chroma bit-depths by scaling the component with lower bit depth to match that of the other component through left-shift before color transform; then, the scaled component will be converted to its original bit-depth via right-shift after color transform. Figure 1 illustrates the decoding process of the proposed solution where it is assumed that the luma bit-depth is bigger than the chroma bit-depth, i.e., BitDepthY>BitDepthCand the bit-depth difference is denoted asΔB=BitDepthY−BitDepthC.

Figure 1 Block diagram of the decoding process of the proposed solution

As it can be seen from Figure 1, all the changes that are introduced by the proposed solution to the decoding process are limited to the inverse color transform while all the other decoding modules are unchanged (i.e., they all  operate at the original bit-depth of the corresponding component).

# Simulation results

The proposed methods are implemented based on SCM-2.0.  Both the bug fixes and the proposed bit depth alignment solution are tested under the common test conditions as specified in [3], except that the internal bit-depths of luma and chroma, i.e. InterBitDepth and InterBitDepthC, are set to be unequal for the following two tests.

- Setting one: InterBitDepth = 10 and InterBitDepthC = 8

- Setting two: InterBitDepth = 12 and InterBItDepthC = 8

The encoding and decoding time reported in this section are inaccurate, as they are measured using a cluster system with inhomogeneous cores.

## SCM-2.0 bug fix vs. SCM-2.0

Using the performance of SCM-2.0 as anchor, Table 1 and Table 2 present BD-rate saving of the proposed bug fixes for Setting one and Setting two, respectively. For Setting one, the proposed encoder only bug fixes could provide average {G, B, R} BD-rate savings of {1.6%, 2.6%, 2.6%}, {0.1%, 1.3%, 1.1%} and {0.8%, 0.7%, 0.8%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {1.6%, 5.2%, 5.1%}, {0.4%, 3.9%, 3.7%} and {0.4%, 3.5%, 3.4%} for AI, RA and LB configurations in YCbCr coding. When it comes to Setting two, the corresponding coding gains of the proposed encoder only bug fixes are increased to {5.6%, 6.5%, 6.6%}, {4.8%, 8.8%, 7.5%} and {2.5%, 11.5%, 9.4%} for the {G, B, R} BD-rate savings in RGB coding, and are increased to {3.4%, 6.5%, 7.0%}, {2.4%, 5.1%, 5.7%} and {1.9%, 4.5%, 5.0%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

Table 1 BD-rate performance of the proposed bug fixes, compared to SCM-2.0 anchor (Setting one: InterBitDepth = 10, InterBitDepthC = 8)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.4% | -3.8% | -3.6% |
| RGB, text & graphics with motion,720p | -2.7% | -4.3% | -4.3% |
| RGB, mixed content, 1440p | -1.8% | -2.8% | -3.0% |
| RGB, mixed content, 1080p | -2.2% | -4.0% | -3.8% |
| RGB, Animation, 720p | -0.3% | -0.9% | -1.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -2.6% | -6.3% | -5.6% |
| YUV, text & graphics with motion,720p | -1.4% | -6.4% | -6.6% |
| YUV, mixed content, 1440p | -2.4% | -8.1% | -8.2% |
| YUV, mixed content, 1080p | -3.0% | -9.0% | -8.7% |
| YUV, Animation, 720p | 0.1% | -1.5% | -1.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -2.0% | -1.9% |
| RGB, text & graphics with motion,720p | 0.2% | -2.1% | -2.0% |
| RGB, mixed content, 1440p | 0.0% | -1.0% | -1.0% |
| RGB, mixed content, 1080p | -0.6% | -2.3% | -2.1% |
| RGB, Animation, 720p | 1.2% | -0.3% | -0.2% |
| RGB, camera captured, 1080p | 0.5% | 0.2% | 0.5% |
| YUV, text & graphics with motion, 1080p | -0.8% | -3.4% | -2.8% |
| YUV, text & graphics with motion,720p | 0.6% | -4.3% | -4.2% |
| YUV, mixed content, 1440p | -0.8% | -7.1% | -7.1% |
| YUV, mixed content, 1080p | -1.4% | -7.6% | -6.9% |
| YUV, Animation, 720p | 0.1% | -1.2% | -1.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.9% | -1.7% |
| RGB, text & graphics with motion,720p | -1.7% | -3.0% | -3.2% |
| RGB, mixed content, 1440p | 0.9% | -0.9% | -1.0% |
| RGB, mixed content, 1080p | 2.2% | 0.5% | -0.1% |
| RGB, Animation, 720p | 2.8% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 1.9% | 0.8% | 1.5% |
| YUV, text & graphics with motion, 1080p | -1.3% | -3.3% | -2.9% |
| YUV, text & graphics with motion,720p | -0.4% | -3.9% | -4.8% |
| YUV, mixed content, 1440p | -0.1% | -6.3% | -6.7% |
| YUV, mixed content, 1080p | -0.5% | -7.2% | -5.8% |
| YUV, Animation, 720p | 0.0% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 BD-rate performance of the proposed bug fixes, compared to SCM-2.0 anchor (Setting two: InterBitDepth = 12, InterBitDepthC = 8)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.8% | -10.1% | -9.8% |
| RGB, text & graphics with motion,720p | -9.5% | -10.3% | -10.6% |
| RGB, mixed content, 1440p | -6.6% | -7.6% | -7.9% |
| RGB, mixed content, 1080p | -8.0% | -9.1% | -9.3% |
| RGB, Animation, 720p | -0.6% | -1.6% | -1.8% |
| RGB, camera captured, 1080p | -0.1% | -0.2% | -0.3% |
| YUV, text & graphics with motion, 1080p | -7.1% | -9.9% | -9.4% |
| YUV, text & graphics with motion,720p | -5.5% | -9.7% | -12.2% |
| YUV, mixed content, 1440p | -3.5% | -8.7% | -9.1% |
| YUV, mixed content, 1080p | -4.5% | -9.5% | -9.8% |
| YUV, Animation, 720p | 0.1% | -1.4% | -1.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.9% | -12.9% | -12.5% |
| RGB, text & graphics with motion,720p | -7.9% | -9.4% | -8.7% |
| RGB, mixed content, 1440p | -3.2% | -5.0% | -5.9% |
| RGB, mixed content, 1080p | -7.1% | -6.1% | -6.7% |
| RGB, Animation, 720p | 1.4% | -11.3% | -7.8% |
| RGB, camera captured, 1080p | 0.0% | -8.1% | -3.6% |
| YUV, text & graphics with motion, 1080p | -4.0% | -6.3% | -6.0% |
| YUV, text & graphics with motion,720p | -3.9% | -6.6% | -10.0% |
| YUV, mixed content, 1440p | -3.1% | -8.1% | -8.7% |
| YUV, mixed content, 1080p | -3.5% | -8.7% | -8.6% |
| YUV, Animation, 720p | 0.1% | -1.1% | -1.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.1% | -14.2% | -13.2% |
| RGB, text & graphics with motion,720p | -8.9% | -12.6% | -11.5% |
| RGB, mixed content, 1440p | 3.9% | -1.3% | -2.7% |
| RGB, mixed content, 1080p | -3.3% | -7.0% | -8.8% |
| RGB, Animation, 720p | 1.9% | -18.1% | -13.5% |
| RGB, camera captured, 1080p | 2.3% | -15.8% | -6.7% |
| YUV, text & graphics with motion, 1080p | -3.0% | -4.9% | -4.9% |
| YUV, text & graphics with motion,720p | -3.1% | -6.4% | -9.1% |
| YUV, mixed content, 1440p | -2.5% | -7.8% | -8.2% |
| YUV, mixed content, 1080p | -2.7% | -7.5% | -7.1% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.7% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |

## The proposed bit depth alignment solution vs. SCM-2.0 with bug fixes

Using the performance of SCM-2.0 with the bug fixes in 3.1 as anchor, Table 3 and Table 4 present BD-rate saving of the proposed bug fixes for Setting one and Setting two, respectively. For Setting one, the proposed bit depth alignment solution could provide average {G, B, R} BD-rate savings of {15.4%, 10.1%, 10.7%}, {20.1%, 13.2%, 14.2%} and {23.6%, 18.1%, 19.2%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {0.1%, 1.3%, 0.9%}, {0.3%, 1.5%, 1.1%} and {0.3%, 1.3%, 1.0%} for AI, RA and LB configurations in YCbCr coding. When it comes to Setting two, the corresponding coding gains of the proposed bit depth alignment solution are {22.1%, 17.1%, 17.7%}, {40.0%, 33.7%, 34.6%} and {47.3%, 40.4%, 41.1%} for the {G, B, R} BD-rate savings in RGB coding, and are {0.1%, 1.3%, 0.9%}, {0.4%, 1.6%, 1.2%} and {0.3%, 1.6%, 1.2%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

Table 3 BD-rate performance of the proposed solution, compared to SCM-2.0 + bug fixes (Setting one: InterBitDepth = 10, InterBitDepthC = 8)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.9% | -3.4% | -3.4% |
| RGB, text & graphics with motion,720p | -11.4% | -6.3% | -8.3% |
| RGB, mixed content, 1440p | -18.3% | -11.8% | -11.2% |
| RGB, mixed content, 1080p | -14.6% | -7.8% | -8.4% |
| RGB, Animation, 720p | -23.0% | -23.1% | -20.3% |
| RGB, camera captured, 1080p | -19.5% | -8.1% | -12.8% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% |
| YUV, mixed content, 1440p | 0.1% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.3% |
| YUV, Animation, 720p | -0.1% | -6.3% | -3.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.4% | -5.2% | -5.4% |
| RGB, text & graphics with motion,720p | -14.4% | -7.2% | -10.0% |
| RGB, mixed content, 1440p | -24.5% | -15.8% | -14.6% |
| RGB, mixed content, 1080p | -17.3% | -8.5% | -8.5% |
| RGB, Animation, 720p | -27.3% | -24.6% | -22.9% |
| RGB, camera captured, 1080p | -28.6% | -18.1% | -24.2% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, text & graphics with motion,720p | -1.2% | -1.0% | -2.0% |
| YUV, mixed content, 1440p | 0.0% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | 0.1% | -0.2% | -0.3% |
| YUV, Animation, 720p | -0.6% | -6.8% | -3.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.8% | -4.9% | -4.9% |
| RGB, text & graphics with motion,720p | -14.0% | -9.0% | -11.6% |
| RGB, mixed content, 1440p | -27.6% | -19.2% | -17.8% |
| RGB, mixed content, 1080p | -22.0% | -14.7% | -16.0% |
| RGB, Animation, 720p | -34.2% | -32.5% | -31.5% |
| RGB, camera captured, 1080p | -36.1% | -28.5% | -33.2% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -1.6% | -1.4% | -2.1% |
| YUV, mixed content, 1440p | 0.0% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | 0.3% | -0.7% | -0.6% |
| YUV, Animation, 720p | -0.2% | -4.8% | -3.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 105% |  |  |

Table 4 BD-rate performance of the proposed solution, compared to SCM-2.0 + bug fixes (Setting two: InterBitDepth = 12, InterBitDepthC = 8)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -16.8% | -13.4% | -13.4% |
| RGB, text & graphics with motion,720p | -17.9% | -13.0% | -14.8% |
| RGB, mixed content, 1440p | -26.5% | -20.7% | -19.9% |
| RGB, mixed content, 1080p | -24.9% | -18.6% | -18.8% |
| RGB, Animation, 720p | -25.8% | -26.6% | -24.2% |
| RGB, camera captured, 1080p | -20.4% | -10.1% | -14.9% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% |
| YUV, mixed content, 1440p | 0.0% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.4% |
| YUV, Animation, 720p | -0.1% | -6.4% | -3.5% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -32.0% | -25.3% | -25.7% |
| RGB, text & graphics with motion,720p | -31.1% | -23.6% | -25.8% |
| RGB, mixed content, 1440p | -43.1% | -36.5% | -35.4% |
| RGB, mixed content, 1080p | -38.5% | -29.4% | -30.4% |
| RGB, Animation, 720p | -44.1% | -40.8% | -39.9% |
| RGB, camera captured, 1080p | -51.3% | -46.2% | -50.4% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.2% |
| YUV, text & graphics with motion,720p | -1.2% | -1.2% | -2.0% |
| YUV, mixed content, 1440p | -0.1% | -1.0% | -0.8% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.7% |
| YUV, Animation, 720p | -0.5% | -6.8% | -3.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -30.4% | -23.8% | -24.1% |
| RGB, text & graphics with motion,720p | -40.8% | -32.4% | -34.8% |
| RGB, mixed content, 1440p | -51.7% | -44.0% | -42.5% |
| RGB, mixed content, 1080p | -45.2% | -32.5% | -32.6% |
| RGB, Animation, 720p | -56.0% | -51.7% | -51.6% |
| RGB, camera captured, 1080p | -60.0% | -58.1% | -60.8% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -1.5% | -1.3% | -2.0% |
| YUV, mixed content, 1440p | -0.1% | -1.1% | -0.9% |
| YUV, mixed content, 1080p | 0.2% | -1.3% | -1.3% |
| YUV, Animation, 720p | -0.4% | -5.3% | -3.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 104% |  |  |

# Proposed specification changes

The proposed specification changes are provided based on the current HEVC SCC draft version [2], where all the changes are highlighted in yellow.

### 8.6.8 Residual modification process for transform blocks using adaptive colour transform

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a variable blkSize specifying the block size,

–	an (blkSize)x(blkSize) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	an modified (blkSize)x(blkSize) array rY of luma residual samples,

–	an modified (blkSize)x(blkSize) array rCb of chroma residual samples,

–	an modified (blkSize)x(blkSize) array rCr of chroma residual samples.

The (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr are modified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ]  >>  1 )
rY[ x ][ y ] = tmp + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − (  rCr[ x ][ y ]  >>  1 )
rCr[ x ][ y ] = rCb[ x ][ y ] + rCr[ x ][ y ]

- Otherwise (cu_transquant_bypass_flag is equal to 0), the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize − 1 are modified as follows:

- The variables deltaBDY and deltaBDC are derived as follows:

deltaBDY = Max(BitDepthY, BitDepthC) - BitDepthY

deltaBDC = Max(BitDepthY, BitDepthC) – BitDepthC

- Residual samples rY[ x ][ y ], rCb[ x ][ y ] and rCr[ x ][ y ] are modified as follows:

rY[ x ][ y ] = rY[ x ][ y ] << deltaBDY

rCb[ x ][ y ] = rCb[ x ][ y ] << deltaBDC

rCr[ x ][ y ] = rCr [ x ][ y ] << deltaBDC

tmp = rY[ x ][ y ] − rCb[ x ][ y ] 
rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp − rCr[ x ][ y ]
rCr[ x ][ y ] = tmp + rCr[ x ][ y ]

rY[ x ][ y ] = (rY[ x ][ y ] + 1<<(deltaBDY -1)) >> deltaBDY

rCb[ x ][ y ] = (rCb[ x ][ y ] + 1<<( deltaBDC -1)) >> deltaBDC

rCr[ x ][ y ] = (rCr [ x ][ y ] + 1<<( deltaBDC -1)) >> deltaBDC

# References

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, SCCE5 Test 3.2.1: In-loop color-space transform, JCTVC-R0147, August 2014, Sapporo, Japan.

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 1, JCTVC-R1005, August 2014, Sapporo, Japan.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-R1015, Auguest 2014, Sapporo, Japan.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).