| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0181_r1 |
| --- | --- |

| Title: | Removal of parsing dependency in palette-based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | + 1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# The specification change is added in this update.

# Abstract

This contribution proposes to modify the design of palette-based coding in order to remove the parsing dependency of various syntax elements for CUs coded with palette mode. Two modifications are introduced to achieve this goal. Firstly, it is proposed to treat the palette index used to indicate escape colors as normal palette index and them using either run mode or copy-above mode. Secondly, it is proposed to separate the parsing of escape color values from the parsing of palette indices.

Compared to the SCM-2.0 anchor, for lossy coding, the proposed methods provide the average {G/Y, B/Cb, R/Cr} BD-rate savings for AI, RA and LB of {0.4%, 0.4%, 0.4%}, {0.2%, 0.2%, 0.2%} and {0.2%, 0.2%, 0.2%}, respectively, for the sequences in the categories text & graphics with motion, 1080p in both RGB and YCbCr color formats. For lossless coding, the corresponding average bit-rate savings for AI, RA and LB are 0.8%, 0.4% and 0.4%, respectively.

# Introduction

At the 18th JCT-VC meeting, palette-based coding Error: Reference source not found was adopted into the HEVC screen content coding (SCC) draft 1 Error: Reference source not found. For a CU coded using the palette mode, in order to code the palette indices, two predictive coding modes, i.e., run mode and copy-above mode, are employed. For sample positions coded in run mode, the level of the palette index, i.e.,pLevel, at the first position in the run-length chain needs to be signaled through the syntax element palette_index. In the current palette design, palette indices are binarized using truncated binary code (TBC) and coded in bypass mode Error: Reference source not found. TBC is a variant of fixed length code (FLC).  The TBC binarization process needs the maximum level, pMax, to be specified as an input parameter.

To exploit the correlation between palette indices and improve the efficiency of palette index coding, one redundancy removal method is applied. Specifically, the following two conditions are checked before coding one palette index in run mode: 1) if the left neighbor of the current pixel is coded in run mode, then the current palette index cannot be the same as that of its left neighbor; 2) if the left neighbor of the current pixel position is coded in copy mode, then the current palette index cannot be the same as that of its above neighbor. The left neighbor in condition 1) and the above neighbor in condition 2) are referred to as the compared palette index (CPI) position. If either of the two conditions is satisfied, both pLevel and pMaxare reduced by one to decrease the length of TBC.

In the current palette design Error: Reference source not found, although escape colors are indicated by setting the palette index value equal to the palette table size, that palette index is independently coded rather than coded in either run mode or copy-above modes. In other words, one run-length chain with the same palette index is always broken whenever an escape color is encountered. Accordingly, the redundancy removal algorithm for coding palette indices can only be applied if the corresponding CPI position is not coded as escape color. DenotepIndexMax as the maximum level of the palette indices in one CU (which is set to palette table size if palette_escape_val_present_flag is equal to 1, and palette table size minus one otherwise). The input parameterpMaxto the TBC binarization is set to pIndexMax when the CPI position is an escape color, and is set to pIndexMax−1when the CPI position is a major color. This can cause parsing dependency between the syntax elements palette_index in one CU, because the current palette index cannot be parsed until the palette index of its CPI position is fully reconstructed.

In the current palette design, the syntax elements related to palette index coding are signaled in such a way that all the syntax elements specifying the level information, escape color information and run information of the current sample location are signaled before proceeding to the next sample location in the CU for which these syntax elements need to be signaled. The parsed value of the syntax element palette_index of the current position determines what syntax elements are to be parsed next. For instance, if palette_index indicates that the current position is an escape color, then the syntax element palette_escape_val is parsed next; otherwise, the syntax element palette_run is parsed next. Such sample-by-sample parsing dependency is undesirable.

# Technical description

In this section, two modifications are proposed to the current design of palette-based coding to remove the aforementioned parsing dependency for palette coded CUs.

To remove the parsing dependency between syntax elements palette_index, it is proposed to treat the palette index used to indicate escape colors (i.e., the palette table size) as a normal palette index as part of the run-length coding process in both run mode and copy-above mode. After this change, parsing dependency of palette_index can be removed. Regardless of whether the CPI position of the current pixel is coded as major color or escape color, the value of palette index can be always used to reduce the TBC length of the next palette index. Thus, the input parameterpMaxto the TBC binarization for the entire CU is always set equal topIndexMax−1. In addition, this change also removes the parsing dependency of palette_run on palette_index, as palette_run is now signaled for both major colors and escape colors. As show in Section 3, this change also improves coding efficiency by enabling run and copy-above modes to be used to code the palette index used for escape colors.

To remove the parsing dependency of palette_escape_val on palette_index, it is proposed to separate parsing of escape color values from parsing of palette indices. Specifically, instead of interleaving the signaling and parsing of palette_escape_val and palette_indx, all the syntax elements except for palette_escape_val are parsed; then, all of the syntax element palette_escape_val in the CU are parsed. This change only modifies the signaling/parsing order of the syntax elements in palette coding mode; it has no impact on coding performance.

# Simulation results

The proposed methods are implemented based on SCM-2.0 and tested under the common test conditions as specified in [3]. The coding performance of SCM-2.0 is used as the anchor.

Table 1 and Table 2 present BD-rate and bit-rate performances of the proposed methods for lossy coding and lossless coding, respectively, compared to the SCM-2.0 anchor. As shown in Table 1, for lossy coding, the proposed methods provide the average {G/Y, B/Cb, R/Cr} BD-rate savings for AI, RA and LB of {0.4%, 0.4%, 0.4%}, {0.2%, 0.2%, 0.2%} and {0.2%, 0.2%, 0.2%}, respectively, for the sequences in the categories text & graphics with motion, 1080p in both RGB and YCbCr color formats. For lossless coding results in Table 2, the corresponding average bit-rate savings for AI, RA and LB are 0.8%, 0.4% and 0.4%, respectivel

Table 1 BD-rate performance of the proposed methods, compared to SCM-2.0 anchor for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.3% | 0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.3% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | 0.0% |
| YUV, mixed content, 1080p | -0.4% | 0.1% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 Bit-rate performance of TU-level inverse ACT, compared to SCM-2.0 anchor for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 0.9% | 0.5% | 1.1% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.1% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.7% | 0.7% | 0.4% | 1.2% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.4% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.3% | 0.6% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.5% | 0.4% | 0.3% | 0.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.4% | 0.2% | 0.5% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 95% |  |  |  |

# Proposed specification change

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| palette_transpose_flag | ae(v) |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_share_flag[ x0 ][ y0 ] ) { |  |
| palette_size = previousPaletteSize |  |
| for( n = 0; n < palette_size; n++ ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][ n ] |  |
| } else { |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize; i++ ) |  |
| previous_palette_entry_flag[ i ] = 0 |  |
| palette_last_group = 0 |  |
| for( i = 0; i < previousPaletteStuffingSize && !palette_last_group && numPredPreviousPalette < max_palette_size; i++ ) { |  |
| lastPossibleGroupFlag = ( i + 4 >= previousPaletteStuffingSize ) |  |
| lastIdx = min( i + 4, previousPaletteStuffingSize ) – 1 |  |
| if( i > 3 && !lastPossibleGroupFlag ) |  |
| palette_all_zeros_in_group | ae(v) |
| Else |  |
| palette_all_zeros_in_group = 0 |  |
| if( palette_all_zeros_in_group ) |  |
| i += 4 |  |
| else { |  |
| numOnesInGroup = 0 |  |
| for( idx = i; idx <= lastIdx && numPredPreviousPalette < max_palette_size; idx++ ) { |  |
| if ( idx = = lastIdx && numOnesInGroup = = 0 ) |  |
| previous_palette_entry_flag[ idx ] = 1 |  |
| Else |  |
| previous_palette_entry_flag[ idx ] | ae(v) |
| if ( previous_palette_entry_flag[ idx ] ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ idx ] |  |
| numPredPreviousPalette++ |  |
| numOnesInGroup++ |  |
| } |  |
| } |  |
| if( !palette_all_zeros_in_group && !lastPossibleGroupFlag && numPredPreviousPalette < max_palette_size ) |  |
| palette_last_group | ae(v) |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + num_signalled_palette_entries |  |
| } |  |
| palette_escape_val_present_flag | ae(v) |
| if( palette_escape_val_present_flag ) |  |
| indexMax = palette_size |  |
| Else |  |
| indexMax = palette_size – 1 |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( scanPos > 0) { |  |
| xC_prev = x0 + travScan[ scanPos − 1 ][ 0 ] |  |
| yC_prev = y0 + travScan[ scanPos − 1 ][ 1 ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE ) |  |
| palette_mode[ xC ][ yC ] | ae(v) |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| adjustedIndexMax = indexMax |  |
| adjustedRefIndex = indexMax + 1 |  |
| } |  |
| if( scanPos > 0 && palette_mode[xC_prev][yC_prev] ! = ESCAPE) { |  |
| if( palette_mode[xC_prev][yC_prev] = = INDEX ) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = paletteMap[ xC_prev ][ yC_prev ] |  |
| } |  |
| if( scanPos > = nCbS && palette_mode[ xC_prev ][ yC_prev ] = = COPY_ABOVE && palette_mode[ xC ][ yC − 1 ] ! = ESCAPE) { |  |
| adjustedIndexMax − = 1 |  |
| adjustedRefIndex = paletteMap[ xC ][ yC − 1 ] |  |
| } |  |
| adjustedIndexMax −= 1 |  |
| } |  |
| if( palette_mode[ xC ][ yC ] ! = COPY_ABOVE ) { |  |
| if( adjustedIndexMax > 0 ) |  |
| palette_index | ae(v) |
| if( palette_index > = adjustedRefIndex ) |  |
| palette_index++ |  |
| if( palette_index = = palette_size ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| palette_mode[ xC ][ yC ] = ESCAPE |  |
| scanPos++ |  |
| } |  |
| } |  |
| if(palette_mode[xC][yC] ! = ESCAPE) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| runMode = palette_mode[ xC ][ yC ] |  |
| while ( runPos < = palette_run ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_mode[ xC ][ yC ] = = INDEX ) { |  |
| palette_mode[ xC ][ yC ] = INDEX |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } else { |  |
| palette_mode[ xC ][ yC ] = COPY_ABOVE |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ y − 1 ] |  |
| } |  |
| runPos++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ scanPos ][ 0 ] |  |
| yC = y0 + travScan[ scanPos ][ 1 ] |  |
| if( palette_index[xC][yC] = = palette_size ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| paletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| scanPos++ |  |
| } |  |
| tempPaletteEntries[ cIdx ][ i ] = palette_entries[ cIdx ][ i ] |  |
| for( i = 0; i < previousPaletteStuffingSize && current_size < max_palette_predictor_size; i++ ) |  |
| if( previous_palette_entry_flag[ i ] = = 0 ) { |  |
| for ( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| tempPaletteEntries[ cIdx ][ current_size ] = previousPaletteEntries[ cIdx ][ i ] |  |
| current_size++ |  |
| } |  |
| previousPaletteStuffingSize = current_size |  |
| previousPaletteEntries = tempPaletteEntries |  |
| } |  |

# References

- P. Onno, X. Xiu, Y.-W. Huang, R. Joshi, Suggested combined software and text for run-based palette mode, JCTVC-R0348, July 2014, Sapporo, Japan.

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 1, JCTVC-R1005, July 2014, Sapporo, Japan.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, Common Test Conditions for Screen Content Coding, JCTVC-R1015, Auguest 2014, Sapporo, Japan.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).